IEEE P802.11
Wireless LANs

| Resolutions for CID 5390, 5393 in Beamforming Comments of LB115 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408 222 1837 | hongyuan@marvell.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| John Ketchum | Qualcomm Incorporated | 9 Damonmill Square, Suite 2A Concord, MA 01742 | +1 781-276-0915 | johnk@qualcomm.com |
| Arul Palanivelu | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408-222-9806 | arulp@marvell.com |

Abstract

This submission proposed resolutions to CID 5390 in the Channel Sounding comment group, and CID 5393 in TxBF comment group. The proposed modifications are based on D3.01.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

CID 5390

| 5390 | 76.24 | 7.3.2.52.6 | The channel estimation capability definition for a device with both NDP and staggered receiving capabilities is vague. For example, if a 2-antenna device (not supporing STBC) can receive NDP with Nsts=3, but can only receive data carried in up to 2 spatial streams, setting this field equal to 2 implies that (3,0) or (3,1) can also be received, which is beyond the capability of the 2-antenna device. | Add a note saying that when staggered sounding is upported, this field should be explained by combining with Supported MCS Field and Rx STBC field. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Discussions:

For a receiving-sounding capable device, its capability on HT-LTF buffering (i.e. the capability indicated by this field), and the maximum supported Nsts used for data demodulation, implied by its supported MCS (receiver side) and STBC mode, are independently defined, but are OVERLAPPING in interpreting the data HT-LTF buffering capability when receiving non-NDP sounding packets.

In the given example, if STBC is not supported, the two-antenna device, when setting this subfield to “2”, cannot receive a staggered sounding packet with Nsts = 3 (i.e. the combinations (3,0) and (3,1)), which is restricted by its supported MCS, even though it can receive a NDP with 3 space-time streams (i.e. buffering 4 HT-LTFs).

Therefore, to minimize the confusion and eliminate the overlapping definition, it is necessary to restrict the definition of this field only for NDP and extension portion of staggered sounding packet. This is also compliant with the text in Clasues 9.16 (Link Adaptation) and 9.17 (Transmit Beamforming), where this field only influences the setting of NUM_EXTEN_SS parameter (TXVECTOR) when transmitting staggered sounding or MCS parameter (TXVECTOR) when transmitting NDP.(e.g. lines18-26 of page 159 in D3.01, lines 4-14 of page 163 in D3.01)

The “(a,b)” combinations listed for staggered sounding case in the spec are also confusing.

A side issue: when receiving NDP, Nsts shall be determined ONLY by the MCS subfield in HT-SIG (as specified in 9.19.2), so the STBC subfield shall be set to 0 (to avoid the confliction with table 20-11); also MCS here shall not be restricted by Supported MCS Set capabilities. Some clearer text in Clause 9.19.2 (Transmission of an NDP) is proposed as below.

TGn Editor: in page 76 of D3.01, the 3rd column and last row of Table 7-43k—Subfields of the Transmit Beamforming Capabilities field, make the follwing modifications:

| Channel Estimation Capability | Indicates the maximum number of space time streams (columns of the MIMO channel matrix) for which channel dimensions can be simultaneously estimated, when receiving NDP sounding PPDU or the extension portion of the long training fields in a staggered sounding PPDU. When staggered sounding is supported this limit applies independently to both the data portion and to the extension portion of the long training fields. | Set to 0 for 1 space time stream Set to 1 for 2 space time streams Set to 2 for 3 space time streams Set to 3 for 4 space time streams Note: When a sounding PPDU that is not an NDP sounding PPDU is supported, the maximum number of space time streams that is supported for demodulating the data symbols is calculated based on the Rx MCS Bitmask subfield of the Supported MCS Set field and the Rx STBC field of the HT Capabilities field, regardless of the number of space time streams indicated by the Channel Estimation Capability field. The Channel Estimation Capability field also indicates the maximum number of antennas to be used by the sender in sounding exchanges for both calibration and antenna selection, if the sounding PPDU is not a staggered sounding PPDU. If the reception of staggered sounding is not supported, the value indicated by this field is equal to the maximum number of supported space time streams. If the reception of staggered sounding is supported: Value 0: (1,0), (1,1) Value 1: (1,0), (2,0), (1,1), (1,2), (2,1), (2,2) Value 2: (1,0), (2,0), (3,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) Value 3: (1,0), (2,0), (3,0), (4,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) where the notation (a, b) indicates: a: supported channel estimation dimensions using long training symbol(s) that will be used for demodulating data symbols. b: supported channel estimation dimensions using long training symbol(s) that will not be used for demodulating data symbols |
| --- | --- | --- |

TGn Editor: on pg 505, ln 49-59, make the following text changes:

"The HT channel estimation capability indicates the maximum number

of space time streams for which channel dimensions can be

simultaneously estimated, when receiving NDP sounding PPDU or  the extension portion of the long training fields in a staggered sounding PPDU. When staggered sounding is supported this

limit applies independently to both the data portion and to the

extension portion of the long training fields, s Set to 0 fro for 1 space

time stream, set to 1 for 2 space time streams, set to 2 for 3

space time streams, set to 3 for 4 space time streams. See

7.3.2.52.6 (Transmit Beamforming Capabilities (#1498))"

TGn Editor: in Clasue 9.19.2(Transmission of an NDP),  page 177 of D3.01, make the follwing modifications:

9.19.2 Transmission of an NDP

A STA that transmits an NDP shall set the LENGTH, MCS, SOUNDING, STBC and NUM_EXTEN_SS parameters of the TXVECTOR as specified in this subclause.

—   LENGTH shall be set to 0.

- SOUNDING shall be set to SOUNDING.

- STBC shall be set to 0.

- MCS shall indicate two or more spatial streams.

The number of spatial streams corresponding to the MCS parameter of the TXVECTOR shall not exceed the

limit indicated by the Channel Estimation Capability field in the Transmit Beamforming Capabilities field

transmitted by the STA that is the intended receiver of the NDP. The MCS parameter of the TXVECTOR shall indicate the number of spatial streams sounded and shall not be limited by the Supported MCS Set field of the HT Capabilities field at either the transmitter or recipient of the NDP. A STA shall set the NUM_EXTEN_SS parameter of the TXVECTOR to zero in the PHY TXSTART.request primitive corresponding to an NDP transmission.

CID 5393

| 5393 | 163.21 | 9.17.2.4.3 | This paragraph gives the restriction for maximum number of sounded dimensions during calibration exchange, when extensive Ht_LTFs is not supported by the recipient. The same requirement should also be added for sounding packets used for link adaptation, implicit TxBF exchange, explicit TxBF exchange, and antenna selection. | Add similar paragraphs in link adaptation, implicit TxBF exchange, explicit TxBF exchange, and antenna selection. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter

Discussions:

According to the resolution of CID 5390, the Ntx requirement for sending a sounding packet in the calibration exchange, when staggered-sounding receiving is not supported by the receiver, should be separately specified in NDP and regular sounding.

Moreover, a similar requirement should be added for ASEL sounding exchanges. But link adaptation, implicit TxBF and explicit TxBF exchanges do not need a similar requirement.

TGn Editor: in page 167 of D3.01, make the following modifications on the paragraph in lines 26-32:

When transmitting a sounding PPDU during step 1 of a calibration procedure, if the Receive Staggered Capability

subfield in the Transmit Beamforming Capability information element transmitted by the intended

receiver is zero, then:

- when the sounding PPDU is not an NDP, the number of antennas used by the sender should be less than or equal to the maximum number of space time streams indicated in by the Channel Estimation Capability subfield Rx MCS Bitmask subfield of the Supported MCS Set field and Rx STBC subfield of the HT Capabilities field in the Transmit Beamforming Capability information element transmitted by the intended receiver.

- when the sounding PPDU is an NDP, the number of antennas used by the sender should be less than or equal to the number indicated in the Channel Estimation Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver.

TGn Editor: in page 175 of D3.01, after Figure 9-38, insert two paragraphs as:

Figure 9-38—Receive antenna selection

The ASEL receiver uses different antenna sets to receive these sounding PPDUs, estimates

channel state information after receiving all these sounding PPDUs, and conducts the antenna selection.

When transmitting a sounding PPDU sent in transmit and receive ASEL exchanges in Figures 9-37 and 9-38, the

NUM_EXTEN_SS parameter of the TXVECTOR shall not be set to a value greater than the limit indicated

by the Channel Estimation Capability subfield in the Transmit Beamforming Capabilities field transmitted

by the STA that is the intended receiver of the PPDU. When the sounding PPDU is an NDP, the number

of spatial streams corresponding to the MCS parameter of the TXVECTOR shall not exceed the limit

indicated by the Channel Estimation Capability subfield in the Transmit Beamforming Capabilities

field transmitted by the STA that is the intended receiver of the NDP.

When transmitting a sounding PPDU sent in transmit and receive ASEL exchanges in Figures 9-37 and 9-38, if the Receive Staggered Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver is zero, then:

- when the sounding PPDU is not an NDP, the number of antennas used by the sender should be less than or equal to the maximum number of space time streams indicated by the Rx MCS Bitmask subfield of the Supported MCS Set field and Rx STBC subfield in the HT Capabilities field transmitted by the intended receiver.

- when the sounding PPDU is an NDP, the number of antennas used by the sender should be less than or equal to the number indicated in the Channel Estimation Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver.

When both transmitter and receiver have antenna selection capabilities, the peer station shall use a fixed subset ….