IEEE P802.11
Wireless LANs

| Resolutions for CID 5390, 5393 in Beamforming Comments of LB115 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hongyuan Zhang | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408 222 1837 | hongyuan@marvell.com |
| Vinko Erceg | Broadcom | 15435 Innovation Dr. San Diego, CA 92128 | +1 858 521 5885 | verceg@broadcom.com |
| John Ketchum | Qualcomm Incorporated | 9 Damonmill Square, Suite 2A Concord, MA 01742 | +1 781-276-0915 | johnk@qualcomm.com |
| Allert van Zelst | Qualcomm Incorporated |  |  | allert@qualcomm.com |
| Bjorn Bjerke | Qualcomm Incorporated |  | +1 781-276-0912 | bbjerke@qualcomm.com |
| Arul Palanivelu | Marvell | 5488 Marvell Lane Santa Clara, CA 95054 | +1 408-222-9806 | arulp@marvell.com |
| Tomoya Yamaura | Sony Corporation |  |  | Tomoya.Yamaura@jp.sony.com |

Abstract

This submission proposes resolutions to CIDs 5390 and 5393.  The proposed modifications are based on D3.03.

CID 5390

| 5390 | 76.24 | 7.3.2.52.6 | The channel estimation capability definition for a device with both NDP and staggered receiving capabilities is vague. For example, if a 2-antenna device (not supporing STBC) can receive NDP with Nsts=3, but can only receive data carried in up to 2 spatial streams, setting this field equal to 2 implies that (3,0) or (3,1) can also be received, which is beyond the capability of the 2-antenna device. | Add a note saying that when staggered sounding is upported, this field should be explained by combining with Supported MCS Field and Rx STBC field. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter

TGn Editor: on page 77, in the middle and 3rd column and last row of Table 7-43o—Subfields of the Transmit Beamforming Capabilities field, make the following modifications:

| Channel Estimation Capability | Indicates the maximum number of space time streams (columns of the MIMO channel matrix) for which channel dimensions can be simultaneously estimated when receiving an NDP sounding PPDU or the extension portion of the HT long training fields in a staggered sounding PPDU. The maximum number of space time streams for which channel coefficients can be simultaneously estimated during the data portion of the HT long training fields is limited by 1) the Rx MCS Bitmask subfield of the Supported MCS Set field and 2) the Rx STBC subfield, both of the HT Capabilities element. When staggered sounding is supported this limit applies independently to both the data portion and to the extension portion of the long training fields. | Set to 0 for 1 space time stream Set to 1 for 2 space time streams Set to 2 for 3 space time streams Set to 3 for 4 space time streams If the reception of staggered sounding is not supported, the value indicated by this field is equal to the maximum number of supported space time streams. If the reception of staggered sounding is supported: Value 0: (1,0), (1,1) Value 1: (1,0), (2,0), (1,1), (1,2), (2,1), (2,2) Value 2: (1,0), (2,0), (3,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) Value 3: (1,0), (2,0), (3,0), (4,0), (1,1), (1,2), (1,3), (2,1), (2,2), (3,1) where the notation (a, b) indicates: a: supported channel estimation dimensions using long training symbol(s) that will be used for demodulating data symbols. b: supported channel estimation dimensions using long training symbol(s) that will not be used for demodulating data symbols |
| --- | --- | --- |

TGn Editor: on page 523, lines 10-19, make the following text changes:

"The HT channel estimation capability indicates the maximum number of space time streams for which channel dimensions can be simultaneously estimated when receiving an NDP sounding PPDU or the extension portion of the HT long training fields in a staggered sounding PPDU. When staggered sounding is supported this limit applies independently to both the data portion and to the extension portion of the long training fields. The maximum number of space time streams for which channel coefficients can be simultaneously estimated during the data portion of the HT long training fields is limited by 1) the Rx MCS Bitmask subfield of the Supported MCS Set field and 2) the Rx STBC subfield, both of the HT Capabilities element., s Set to 0 fro for 1 space time stream, set to 1 for 2 space time streams, set to 2 for 3 space time streams, set to 3 for 4 space time streams. See 7.3.2.52.6 (Transmit Beamforming Capabilities (#1498))"

TGn Editor: in clause 9.20.2 (Transmission of an NDP), page 186, make the following modifications:

9.20.2 Transmission of an NDP

A STA that transmits an NDP shall set the LENGTH, MCS, SOUNDING, STBC and NUM_EXTEN_SS parameters of the TXVECTOR as specified in this subclause.

—   LENGTH shall be set to 0.

- SOUNDING shall be set to SOUNDING.

- STBC shall be set to 0.

- MCS shall indicate two or more spatial streams.

The number of spatial streams sounded corresponding to is indicated by the MCS parameter of the TXVECTOR, and shall not exceed the limit indicated by the Channel Estimation Capability field in the Transmit Beamforming Capabilities field transmitted by the STA that is the intended receiver of the NDP.  The MCS parameter shall not be limited by the Supported MCS Set field of the HT Capabilities field at either the transmitter or recipient of the NDP. A STA shall set the NUM_EXTEN_SS parameter of the TXVECTOR to zero in the PHY TXSTART.request primitive corresponding to an NDP transmission.

CID 5393

| 5393 | 163.21 | 9.17.2.4.3 | This paragraph gives the restriction for maximum number of sounded dimensions during calibration exchange, when extensive Ht_LTFs is not supported by the recipient. The same requirement should also be added for sounding packets used for link adaptation, implicit TxBF exchange, explicit TxBF exchange, and antenna selection. | Add similar paragraphs in link adaptation, implicit TxBF exchange, explicit TxBF exchange, and antenna selection. |
| --- | --- | --- | --- | --- |

Proposed Resolution: Counter. Add a separate subclause stating restrictions for sounding PPDUs. Remove the resulting redundant text from affected subclauses.

TGn Editor: on page 166, insert the following subclause 9.16a immediately before subclause 9.17

9.16a Sounding PPDUs

A sounding PPDU is a PPDU for which the SOUNDING parameter of the corresponding RXVECTOR or TXVECTOR has the value SOUNDING, corresponding to the Not Sounding field set to zero in the HT SIG.  Sounding PPDUs are transmitted by STAs to enable the receiving STAs to estimate the channel between the transmitting STA and the receiving STA.

STAs that transmit sounding PPDUs include MCS feedback requesters (see 9.16.2 Link adaptation using the HT Control field), beamformees responding to training requests and calibration initiators/responders involved in implicit transmit beamforming (see 9.17.2.2 Unidirectional implicit transmit beamforming, 9.17.2.3 Bidirectional implicit transmit beamforming, and 9.17.2.4 Calibration), beamformers involved in explicit transmit beamforming (see 9.17.3 Explicit feedback beamforming), and ASEL transmitters involved in antenna selection (see 9.18.2 Procedure).

STAs that receive sounding PPDUs include MCS feedback responders (see 9.16.2 Link adaptation using the HT Control field), beamformers sending training requests and calibration initiators/responders involved in implicit transmit beamforming (see 9.17.2.2 Unidirectional implicit transmit beamforming, 9.17.2.3 Bidirectional implicit transmit beamforming, and 9.17.2.4 Calibration), beamformees involved in explicit transmit beamforming (see 9.17.3 Explicit feedback beamforming), and transmit ASEL responders involved in antenna selection (see 9.18.2 Procedure).

When transmitting a sounding PPDU, the transmitting STA shall follow the rules stated below to determine the maximum number of space time streams for which channel coefficients can be simultaneously estimated.

When transmitting a sounding PPDU that is not an NDP sounding PPDU and this sounding PPDU

- contains a +HTC frame with the MRQ field set to 1, or

- is sent as a response to a +HTC frame with the TRQ field set to 1, or

- is sent during a calibration sounding exchange, or

- is sent by a beamformer involved in explicit transmit beamforming, or

- is sent in transmit or receive ASEL exchanges,

the NUM_EXTEN_SS parameter in the TXVECTOR shall not be set to a value greater than the limit indicated by the Channel Estimation Capability subfield in the Transmit Beamforming Capabilities field transmitted by the STA that is the intended receiver of the sounding PPDU. The maximum number of space time streams for which channel coefficients can be simultaneously estimated during the data portion of the HT long training fields is limited by 1) the Rx MCS Bitmask subfield of the Supported MCS Set field and 2) the Rx STBC subfield, both of the HT Capabilities element.

When the sounding PPDU is an NDP transmitted under the same circumstances as above, the number of spatial streams corresponding to the MCS parameter of the TXVECTOR shall not exceed the limit indicated by the Channel Estimation Capability subfield in the Transmit Beamforming Capabilities field transmitted from the STA that is the intended receiver of the NDP.

TGn Editor: in clause 9.17.2 (Link adaptation using the HT Control field), page 167, delete the paragraph that begins on line 13 and ends on line 20.

TGn Editor: in clause 9.18.2.1 (General), page 171, delete the paragraph that begins on line 3 and ends on line 10.

TGn Editor: in clause 9.18.2.4.3 (Sounding exchange for calibration), page 175, delete the paragraph that begins on line 17 and ends on line 24. Furthermore, modify the paragraph that begins on line 26 and ends on line 31 as follows:

When transmitting a sounding PPDU during step 1 of a calibration procedure, if the Receive Staggered Capability

subfield in the Transmit Beamforming Capability information element transmitted by the intended

receiver is zero, then:

- when the sounding PPDU is not an NDP, the number of antennas used by the sender shallshould be less than or equal to the maximum number of space time streams indicated in by the Channel Estimation Capability subfield Rx MCS Bitmask subfield of the Supported MCS Set field and the Rx STBC subfield of the HT Capabilities element in the Transmit Beamforming Capability information element transmitted by the intended receiver.

- when the sounding PPDU is an NDP, the number of antennas used by the sender shall be less than or equal to the number indicated by the Channel Estimation Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver.

TGn Editor: in clause 9.18.3 (Explicit feedback beamforming), page 178, delete the paragraph that begins on line 51 and ends on line 59.

TGn Editor: in clause 9.19.2 (Procedure), page 184, insert the following paragraph on line 30:

When transmitting a sounding PPDU sent in transmit and receive ASEL exchanges in Figures 9-37 and 9-38, if the Receive Staggered Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver is zero, then:

- when the sounding PPDU is not an NDP, the number of antennas used by the sender, as indicated by the ANTENNA_SET parameter in the TXVECTOR, shall be less than or equal to the maximum number of space time streams indicated by the Rx MCS Bitmask subfield of the Supported MCS Set field and the Rx STBC subfield in the HT Capabilities element transmitted by the intended receiver.

- when the sounding PPDU is an NDP, the number of antennas used by the sender, as indicated by the ANTENNA_SET parameter in the TXVECTOR, shall be less than or equal to the number indicated by the Channel Estimation Capability subfield in the Transmit Beamforming Capability information element transmitted by the intended receiver.