IEEE P802.11
Wireless LANs

| TGn LB115 Submission for GEN Ad Hoc on Table J.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yasuhiko Inoue | NTT | 1-1 Hikari-no-oka, Yokosuka, Kanagawa 239-0847 Japan | ++81-46-859-5097 | yinoue@ansl.ntt.co.jp |
| Kengo Nagata | NTT | 1-1 Hikari-no-oka, Yokosuka, Kanagawa 239-0847 Japan | ++81-46-859-3354 | Nagata.Kengo@lab.ntt.co.jp |

Abstract

This submission proposes resolutions to CIDs 5114 and 5199 related to Japanese frequency regulation.

The changes proposed in this document are based on 802.11n D3.02.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Comments

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5114 | p.473 l.1 | J | Is it necessary to have an amendment on the 40 MHz operations for the regulatory classes in Japan as well? | Check and amend accordingly. | Accept |
| 5199 | p.474 l.36 | J | Where are the changes to Table J.3 for HT operation in Japan? Add them. | per comment | Accept |

Proposed Resolution: Accept

# Edits

Replace the first row of Table J.3 as shown:

Table J.3—Regulatory classes in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 36, 40, 44, 48 | 10 | 10 | 1 | 1, 2, 6 |

Insert new penultimate rows, change the last row and insert the note after the last row of Table J.3 as

shown:

Table J.3—Regulatory classes in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| 31 | 5 | 20 | 52, 56, 60, 64 | 10 | 10 | 1 | 1, 2, 3, 4, 5, 6 |
| 32 | 5 | 20 | 52, 56, 60, 64 | 10 | 5 | 1 | 1, 2, 4, 5, 6 |
| 33 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 10 | 50 | 1 | 1, 3, 4, 5, 6 |
| 34 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 10 | 25 | 1 | 1, 4, 5, 6 |
| 35 | 5 | 40 | 38, 46 | 5 | 5 | 1 | 1, 2, 6 |
| 36 | 5 | 40 | 54, 62 | 5 | 5 | 1 | 1, 2, 3, 4, 5, 6 |
| 37 | 5 | 40 | 54, 62 | 5 | 2.5 | 1 | 1, 2, 4, 5, 6 |
| 38 | 5 | 40 | 102, 110, 118, 126, 134 | 5 | 25 | 1 | 1, 3, 4, 5, 6 |
| 39 | 5 | 40 | 102, 110, 118, 126, 134 | 5 | 12.5 | 1 | 1, 4, 5, 6 |
| 40 | 4 | 40 | 186, 194 | 25 (1) | --- | 2 | 5, 6, 7 |
| 41 | 4 | 40 | 186, 194 | 25 (1) | --- | 2 | 5, 6, 8 |
| 42 | 4 | 40 | 186, 194 | 25 (1) | --- | 3 | 5, 6, 7 |
| 43 | 4 | 40 | 186, 194 | 25 (1) | --- | 3 | 5, 6, 8 |
| 44 | 4 | 40 | 186, 194 | 5? | --- | 1 | 5, 6, 8 |
| 45 | 2.407 | 40 | 1-9 | 5 | --- | 4 | 10,13 |
| 46 | 2.407 | 40 | 5-13 | 5 | --- | 4 | 10, 14 |
| 47-255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |
| NOTE—The Channel Spacing for Regulatory Class 33-42 is for the supported bandwidth rather than the operating bandwidth. In these regulatory domains, the AP operates in a 20/40MHz BSS, and the operating bandwidth for a non-AP STA is either 20MHz or 40MHz. NOTE(1)—Total transmit power shall be less than 250mW. |  |  |  |  |  |  |  |