IEEE P802.11
Wireless LANs

| TGp LB 110 Comment Resolution – EC IE optional issue |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Francois Simon | US DOT | ARINC, Inc. 2551 Riva Road Annapolis, MD 21401 | 410-266-4531 | fsimon@arinc.com |

Abstract

This paper addresses the comments addressing Extended Capabilities information element – “EC IE optional issue”.  It includes responses to CR#: 188, 194, 195, 196, 219, 220, 226, and 336.

|  | LB110 Comment Resolution |
| --- | --- |

- COMMENT:  [From Spreadsheet]  INSERT Original Comment Here:

| ID | Commenter | Clause | Pg | Ln | Type | Comment | Suggested Remedy | Recommended Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 194 | Myles, Andrew | 7.2.3.1 | 5 | 11 | TR | An Extended Capabilities IE is added to the Beacon However, it appears to be mandatory | Make it optional | The use of the Extended Capabilities information element is dependent of the services provided by the MAC sublayer to the layers above the MAC. See clause 2, CID# 194 of this document for explanation. |
| 195 | Caam-Winget, Nancy | 7.2.3.1 | 5 | 11 | TR | An Extended Capabilities IE is added to the Beacon but is implied to be mandatory. | Make it optional | The use of the Extended Capabilities information element is dependent of the services provided by the MAC sublayer to the layers above the MAC. See clause 2, CID# 194 of this document for explanation. |
| 196 | Emeott, Stephen | 7.2.3.1 | 5 | 12 | TR | In Table 7-8, instructions for when to include the extended capabilities IE in beacons are missing. Not every STA should need to include this IE when transmitting a beacon | Add instructions about when to include the extended capabilities IE in beacons | Agreed – A new text is being proposed for subclause 7.2.3.1. See clause 4, subclause 7.2.3.1 In this document. |
| 219 | Malinen, Jouni | 7.2.3.1 | 5 | 12 | TR | Addition of Extended Capabilities IE into the Beacon frame body (Table 7-8) seems to claim that this IE would be included in all Beacon frames, not just the ones used for WAVE on-demand beacon. This would make all existing implementation not comply with the standard which is not really acceptable. This IE should only be added when needed. | Make addition of Extended Capabilities IE conditional--something like “... is present when dot11WAVEServicesEnabled is TRUE and this is an on-demand beacon frame”. | Agreed - – A new Text is being Proposed for subclause 7.2.3.1. See clause 4, subclause 7.2.3.1 in this document. To be noted that EC IE is not exclusive to WAVE. |
| 220 | Bai , Fan | 7.2.3.1 | 5 | 12 | TR | The functionality and purpose of extended capabilities should be clearly stated and illustrated. Also, what is the difference between 'capabilities' and 'extended capabilities"? | give the explicit reference to the applicable locations of definitions of 'capabilities' and 'extended capabilities' in master document. | The Capabilities field is defined in subclause 7.3.1.4 and the Extended Capabilities Information element is Specified in Subclause 7.3.2.27 of the base standard. |
| 226 | Bai , Fan | 7.3.2 | 5 | 50 | TR | Same as the comments above | give the explicit reference to the applicable locations of definitions of 'capabilities' and 'extended capabilities' in master document | It is assume that the commenter meant 7.3.2.27. The Capabilities field is defined in subclause 7.3.1.4 and the Extended Capabilities information element is specified in subclause 7.3.2.27 of the base standard. |
| 336 | Inoue, Yasuhiko | 7.3.2.27 | 5 | 50 | TR | I would like to see entire figure of Extended Capabilities field. | Add or change the Figure 7-76a to show entire field of Extended Capabilities. | Agreed – (agree in principle). The Extended Capabilities element is shown in Figure 7-76 of the base standard. It is proposed to revise Figure 7-76a in the amendment, to align the Capabilities field with other 802.11 WGs. See clause 4, Figure 7-76a. |
| 188 | Engwer, Darwin | 7.3.2.27 | 6 | 4 | TR | there is no explanation of the "HT information exchange Support" bit field | Add an explanation of the field and the meaning of its values, or remove the field | Accepted to remove – This bit is outside the scope of 802.11p amendment. A new Fgure 7-76 is proposed. See clause 4, Figure 7-76a in this document. |
| 189 | Bai , Fan | 10.3.37.1.2 | 14 | 19 | TR | Again, the 'capability' and 'extended capabilities' information should be defined, clarified | give the explicit reference to the applicable locations of definitions of 'capabilities' and 'extended capabilities' in master document | The Capabilities field is defined in subclause 7.3.1.4 and the Extended Capabilities information element is specified in subclause 7.3.2.27 of the base standard. |

- Background, Explanation, Discussion, etc.:

These Comments are associated with comments related to “EC IE optional issue” classification

Relating to CID# 194

The Extended Capabilities information element as defined in IEEE 802.11 Std-2007 is “intended to augment the Capability Information field (CIF)”.   This information element is not exclusive to WAVE mode. TGy and TGn intend to also use this information element and may or may not require this IE to be mandatory for their applications.

For WAVE mode is concerned, the Extended Capabilities information element is MANDATORY in the proposed WAVE beacon. It is proposed to revise subclause 7.2.3.1 as specified in clause 4, subclause 7.2.3.1 of this document.

In either case, the Extended Capabilities information element should be ignored if the STA (e.g.; legacy STA) is not able to process this IE.

- Recommended Resolution of the Comments:

See the right column above for the resolutions of the individual comments.

- Recommended Changes to P802.11p D3.0:

7.2.3.1 Beacon frame format

Insert the following after the last sentence of subclause 7.2.3.1

In WAVE mode, the WAVE beacon includes zero or more WAVE information elements (WIE).  The on-demand beacon includes the Extended Capabilities information element (see 7.3.2.27).

EDIT NOTE – Table 7-8 – The Extended Capabilities information element needs to be added to the beacon frame to support the on-demand beacon.  The 802.11 technical editor will assign order numbers when merging the additional information elements into the baseline document.

Insert the following information elements into Table 7-8:

| Order | Information | Notes |
| --- | --- | --- |
| 24a | Extended Capabilities | The Extended Capabilities information element carries information of an 802.11 STA intended to augment the Capabilities Information Field (CIF). |
| 24b | WIE | The WIE contains information provided to the MAC through the MLME_SAP. Zero or more WIEs nay be included in the WAVE beacon. |

Figure 7-76a

B0		B1		B2	        B3            B4                                                                        B7

| Reserved | On-demand beacon | Reserved | WAVE indication | Reserved |
| --- | --- | --- | --- | --- |

Bits		1		1		1	       1                  1                      1                          1                      1

Figure 7-76a – Capabilities field first octet

- On-demand beacon (B1) – When the on-demand beacon bit in the Capabilities field is set to 1, the beacon transmission is initiated by the station management entity (SME) and indicates an on-demand beacon.

- WAVE indication (B3) – If MIB attribute dot11WAVEEnabled = true, then the WAVE indication bit in the first octet of the Capabilities field is set to 1, otherwise it is set to 0.

- Motion (if technical and/or significant):

Move to accept the Recommended Resolutions to these comments and the Recommended changes to P802.11p D3.0 noted above and instruct the editor to make these changes to P802.11p D3.0.

Motion by: ___Francois Simon________________Date:

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References:

IEEE P802.11p/D3.0

IEEE 802.11 Std-2007