| IPv6 TCLAS Update to 802.11v-D1.0 (LB 108) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-1-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Allan Thomson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 853 5570 | allant@cisco.com |

Abstract

This document describes proposed changes to the IEEE 802.11-2007 standard to address corrections to TCLAS specification primary pertaining to IPv6 classifiers.  This document (if approved) will resolve the ambiguous specifications noted in 11-07-2488r1, enhance the usefulness of the TCLAS element to FBMS and other services, and address the following LB108 review comments (cf. 11-07/2558r02) CID #: 835.

This document is based on IEEE P802.11v-d1.0 and IEEE 802.11-2007.

2. Normative references

Editing Instructions: Add the following references, alphabetizing as appropriate

IETF RFC 791, Internet Protocol, September 1981

IETF RFC 2460, Internet Protocol, Version 6 (IPv6), S. Deering, R. Hinden, December 1998

7. Frame Formats

7.3 Management frame body components

7.3.1 Fields that are not information elements

7.3.1.9 Status code field

Editing Instructions: Insert the following to the contents of Table 23 as shown:

Table 23—Status codes

| Status Code | Meaning |
| --- | --- |
| <ANA> | Requested TCLAS processing is not supported by AP |
| <ANA+1> | AP has insufficient TCLAS processing resources to satisfy request |

7.3.2 Information elements

7.3.2.31 TCLAS element

Editing Instructions: amend the following text as indicated:

For Classifier Type 1, frame classifier is defined for both IPv4 and IPv6, shown in Figure 7-88 and Figure 7-89, and distinguished by the Version field. The subfields in the classifier parameters are represented and transmitted in the big-endian format. The classifier parameters are the following parameters:

- In a TCP or UDP header: Source Address, Destination Address, Source Port, Destination Port, and Version, plus

- One of the following:

- In an IPv4 header: Differentiated Services Code Point (DSCP) (IETF RFC 2474-1998 [B21])and Protocol, or

- In an IPv6 header: Flow Label, Next Header and Traffic Class (IETF RFC 2460-1998).

As shown in figure 7-89a, Tthe DSCP field contains the value in the 6 LSBs, and the 2 MSBs are set to 0. The 2 MSBs of the DSCP field are ignored for frame classification.

The values for the Version field of IPv4 and IPv6 are drawn from RFC-791 and RFC-2460 respectively.

The 20-bit IPv6 Flow Label is formatted in the 3-octet Flow Label field as shown in Figure 7-89b.  The reserved bits in the Flow Lable field are set to 0 and ignored upon reception.

As shown in figure 7-89a, the Traffic Class field contains the DSCP value in the 6 LSBs, and the 2 MSBs are set to 0. The 2 MSBs of the Traffic Class field are ignored for frame classification.

Editing Instructions: Replace figure 7-89 in IEEE 802.11-2007 with figure 7-89 below:

|  | Classi-fier Type (1) | Classi-fier Mask | Ver-sion | Source IP Address | Destin-ation IP Address | Source Port | Desti-nation Port | Flow Label | Next Header | Traffic Class |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 8 | 8 | 1 | 1 | 3 | 1 | 1 |

Figure 7-89—Frame Classifier field of Classifier Type 1 for traffic over IPv6

Editing Instructions: add the following two figures as indicated:

|  | b0 b5 | b6 b7 |
| --- | --- | --- |
|  | DSCP | Reserved |
| Bits: | 6 | 2 |
| Octets: | 1 |  |

Figure 7-89a—IPv4 DSCP field and IPv6 Traffic Class field

|  | b0 b3 | b4 b7 | b0 b7 | b0 b7 |
| --- | --- | --- | --- | --- |
|  | Reserved | Flow Label |  |  |
| Bits: | 4 | 4 | 8 | 8 |
| Octets: | 1 |  | 1 | 1 |

Figure 7-89b—IPv6 Flow Label Field

10. Layer management

10.3 MLME SAP interface

10.3.24 TS management interface

10.3.24.2 MLME-ADDTS.confirm

Editing Instructions: in the Valid Range cell of the ResultCode row of the table, add the following parameters to the end of the list: REQUESTED_TCLAS_NOT_SUPPORTED, TCLAS_RESOURCES_EXHAUSTED

10.3.24.4 MLME-ADDTS.response

Editing Instructions: in the Valid Range cell of the ResultCode row of the table, add the following parameters to the end of the list: REQUESTED_TCLAS_NOT_SUPPORTED, TCLAS_RESOURCES_EXHAUSTED

11. MLME

11.4 TS operation

11.4.4 TS setup

Insert the following to the contents of Table 11-2 as shown:

Table 11-2—Encoding of ResultCode to Status Code field value

| ResultCode | Status code |
| --- | --- |
| SUCCESS | 0 |
| INVALID_PARAMETERS | 38 |
| REJECTED_WITH_SUGGESTED_CHANGES | 39 |
| REJECTED_FOR_DELAY_PERIOD | 47 |
| REQUESTED_TCLAS_NOT_SUPPORTED | <ANA> |
| TCLAS_RESOURCES_EXHAUSTED | <ANA+1> |

11.4.7 TS deletion

Editing Instructions: in Table 11-3, change the left hand column heading from “ReasonCode” to “ResultCode”