IEEE P802.11
Wireless LANs

| Proposed resolution to LB108 CID 109 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX | +441784848770 | aashley@nds.com |

Abstract

This document contains a proposed resolution to CID 109 from LB108. This also provides a resolution to the related comments CID 903, 1110, 1929 and 1930.

The proposed modifications are based up IEEE P802.11v D1.03.

In section 7.3.2.22.10a, modify the paragraph as indicated:

The Multicast Rate field specifies the maximum PHY data rate, in 0.5 Mb/s units, at which the STA has received a multicast frame with a valid FCS during the measurement period. The Multicast Rate field is encoded with the MSB set to 1 to indicate that the data rate is in the basic rate set, and set to 0 to indicate that the data rate is not in the basic rate set. The remaining 15 bit value is multiplied by 0.5 Mb/s to indicate the data rate. If no value is provided by the STA, the Multicast Rate field is set to 0.

In section 11.20.2, modify the paragraph as indicated:

An AP may send a Multicast Diagnostic Request frame to a non-AP STA that has indicated support of the multicast diagnostic capability or to a multicast group address if all non-AP STA multicast receivers support the multicast diagnostic capability. If the STA accepts the request it shall count the number of received MSDUs with a valid FCS and the specified multicast/broadcast address and the STA shall record the maximum PHY data rate used to encode these MSDUs during the requested Measurement Duration. These two values shall be returned in a Multicast Diagnostic report.

References: