IEEE P802.11
Wireless LANs

| Path Selection and Switch Mechanism |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jiyoung Huh | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +82-2-526-4151 | jyhuh@lge.com |

Abstract

This document contains a normative text proposal for the path selection and switch mechanism for Direct Link Setup. This mechanism allows the stations using direct link to decide the better one among AP-Path and Direct Link Path efficiently. This document is based on 802.11z Draft 0.1, to which several other people have contributed

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.2.2.1.6  as shown:

7.2.2.1.6 TDLS Tx Path Switch Request frame format

The TDLS Tx Path Switch Request frame contains the information shown in Table z5.

Table z5—Information for TDLS Tx Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 3 | Path | The Path (1-bit field) is set to 0 for AP path and to 1 for the direct link path in order to indicate the requested transmission path. element contains the requested transmit path. The Path element is specified in 7.3.2.z2. |
| 4 | Reason | The Reason (7-bit field) indicates the reason to send the TDLS Tx Path Switch Request frame. The Reason field is set to one of the values in Table z6. |
| 5 | RSNI3 | The RSNI 3 indicates the RSNI on frames sent to the AP from the STA requesting the Tx Path Switch. (Optional) |

The TDLS Tx Path Switch Request frame shall be sent through the AP.

Table z6—Reason for TDLS Tx Path Switch Request frame

| Reason Value | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Request for stopping the PS mode |
| 2 | The RSNI 3 is very high. In this case, the RSNI 3 field shall be included. |
| 3 | The RSNI 3 is very low. In this case, the RSNI 3 field shall be included. |
| 4-127 | Reserved |

7.2.2.1.7 TDLS Tx Path Switch Response frame format

The TDLS Path Switch Response frame contains the information shown in Table z6.

Table z67—Information for TDLS Tx Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Suspend frame. |
| 3 | Path | The Path (1-bit field) is set to 0 for AP path and to 1 for the direct link path in order to indicate the requested transmission path. element contains the requested transmit path. The Path element is specified in 7.3.2.z2. |
| 4 | Result | The Result (7-bit fields) indicates the result of the Tx Path switch request and is set to one of values in Table z8 |
| 5 | RSNI1 | The RSNI1 indicates the RSNI on frames received from the AP. The RSNI field is defined in 802.11k clause 7.3.2.41 (Optional) |
| 6 | RSNI2 | The RSNI2 indicates the RSNI on frames received from the STA requesting the Tx Path switch via direct link. The RSNI field is defined in 802.11k clause 7.3.2.41. The RSNI 1 and RSNI2 shall be included together. (Optional) |

The TDLS Path Switch Response frame shall be sent through the AP.

Table z8—Result for TDLS Tx Path Switch Response frame

| Result Value | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Accept |
| 2 | Reject because of entering power save mode |
| 3 | Reject because of RSNI1 and RSNI2. In this case, both RSNI1 and RSNI2 fields shall be included. |
| 4-127 | Reserved |

7.2.2.1.8 TDLS Rx Path Switch Request frame format

The TDLS Rx Path Switch Request frame contains the information shown in Table z8.

Table z89—Information for TDLS Rx Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 3 | Path | The Path (1-bit field) is set to 0 for AP path and to 1 for the direct link path in order to indicate the requested transmission path. element contains the requested transmit path. The Path element is specified in 7.3.2.z2. |
| 4 | Reason | The Reason (7-bit field) indicates the reason to send the TDLS Rx Path Switch Request frame. The Reason field is set to one of the values in Table z10. |
| 5 | RSNI1 | The RSNI1 indicates the RSNI on frames received from the AP. The RSNI field is defined in 802.11k clause 7.3.2.41 (Optional) |
| 6 | RSNI2 | The RSNI 2 indicates the RSNI on frames received from the STA responding to the Rx Path switch request via direct link. The RSNI field is defined in 802.11k clause 7.3.2.41. The RSNI 1 and RSNI2 shall be included together. (Optional) |

The TDLS Rx Path Switch frame shall be sent through the AP.

Table z10—Reason for TDLS Rx Path Switch Request frame

| Reason Value | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Request for entering the PS mode |
| 2 | Because of RSNI and RSNI 2. In this case, both RSNI 1 and RSNI 2 fields shall be included. |
| 3-127 | Reserved |

7.2.2.1.9 TDLS Rx Path Switch Response frame format

The TDLS Rx Path Switch Response frame contains the information shown in Table z9.

Table z911—Information for TDLS Rx Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Path Switch Request frame. |
| 3 | Path | The Path (1-bit field) is set to 0 for AP path and to 1 for the direct link path in order to indicate the requested transmission path. element contains the requested transmit path. The Path element is specified in 7.3.2.z2. |
| 4 | Result | The Result (7-bit field) indicates the result of the Rx Path switch request and is set to one of values in Table z12 |
| 5 | RSNI3 | The RSNI 3 indicates the RSNI on frames sent to the AP from the STA responding to the Rx Path Switch request. The AP provides this value for the responding STA. (Optional) |

The TDLS Rx Path Switch Response frame shall be sent through the AP.

Table z12—Result for TDLS Rx Path Switch Response frame

| Result Value | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Accept |
| 2 | Reject because of RSNI 3 |
| 3-127 | Reserved |

Change 7.3.2.21  as shown:

7.3.2.21 Measurement Request element

In P802.11k D7.0, clause 7.3.2.21, Table 29, insert a new measurement request named Link RCPIRSNI Request, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, renumber clause 7.3.2.21.11 into 7.3.2.21.12, and insert a new clause 7.3.2.21.11 as follows:

7.3.2.21.11 Link RCPIRSNI Request

The Measurement Request field corresponding to a Link RCPIRSNI Request is shown Figure z2.

|  | BSSID | STA Address |  |
| --- | --- | --- | --- |
| Octets: | 6 | 6 |  |

Figure z2—Measurement Request field for a Link RCPIRSNI Request

BSSID indicates the BSSID.

STA Address indicates the MAC address of the STA requesting the Link RCPIRSNI measurement.

Change 7.3.2.22  as shown:

7.3.2.22 Measurement Report element

In P802.11k D7.0, clause 7.3.2.22, Table 30, insert a new measurement report named  Link RCPIRSNI Report, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, insert a new clause 7.3.2.22.11 as follows:

7.3.2.22.11 Link RCPIRSNI Report

The format of the Measurement Report field corresponding to a Link RCPI Report is shown in Figure z3.

|  | BSSID | RCPI1RSNI1 | STA Address | RCPI2RSNI2 |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 1 | 6 | 1 |  |

Figure z3—Measurement Report field for a Link RCPI Report

BSSID indicates the BSSID.

RCPI1RSNI1 indicates the RCPIRSNI on frames received from the AP. The RCPIRSNI field is defined in 802.11k clause 17.3.10.6.7.3.2.41.

STA Address indicates the MAC address of the STA requesting the Link RCPIRSNI measurement.

RCPI2RSNI2 indicates the RCPIRSNI on frames received from the STA requesting the Link RCPIRSNI measurement. The RCPIRSNI field is defined in 802.11k clause 17.3.10.6.7.3.2.41.

Change 7.3.2.z2  as shown:

7.3.2.z2 Path element

The Path element identifies the path selected for a direct link. The element information format is defined in Figure z5.

|  | Element ID | Length | Path |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |

Figure z5—Path element format

The Length field shall be set to 1.

The Path field is set to 0 for the AP path and to 1 for the direct link path.

11 MLME

Change 11.z1  as shown:

11.z1 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by the fact that the signaling frames are encapsulated in Data frames, which allows them to be transmitted through any access point transparently. Therefore, a direct link can be setup using any access point. The access point does not need to be direct link aware, nor does it have to support any of the capabilities which will be used on the direct link. TDLS also includes an option to suspend receiving over the direct link, so that the station can enter a power save mode.

A STA may transmit a Link RCPIRSNI Measurement Request to an (intended) peer STA to obtain an indication of the RCPIRSNI values at the peer STA. The Link RCPIRSNI measurement request and report are sent to the peer STA directly. The RCPIRSNI information may be used to decide whether to switch over to a direct link for communication with the peer STA.

To setup a direct link, the initiator STA sends a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it responds with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it responds with a TDLS Setup Response with a status code other than 0. If there is no response within the set timeout, the initiator STA should conclude that the intended peer STA does not support TDLS and the setup procedure is terminated. The initiator then sends a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response. If security is required, the TDLS setup messages shall include the SMK handshake. When the TDLS Setup Handshake has been completed, both STAs shall accept frames received over the direct link.

After a successful response, the initiating STA further prepares the direct link for Data transmissions by starting the 4-way Handshake.

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a A STA transmitting a Data frame may send a TDLS Tx Path Switch Request frame to request indicating a switch to the direct path or to the AP path. The reason requesting a path switch shall be included and the RSNI on frames sent to the AP from the STA sending a TDLS Tx Path Switch Request may be included in this request frame. The reason shall be set to one of values in Table z6. A STA receiving this request frame sends the TDLS Tx Path Switch Response frame to the requesting STA after deciding whether the path switch request is acceptable or not based on information included in TDLS Tx Path Request frame or itself. The result of the request shall be included and RSNIs on frames received from the AP and from the STA requesting the TDLS Tx Path Switch Request via direct link may be included in this response frame. The result shall be set to one of values in Table z8. In case of switch to a direct path, Tthe first Data frame transmitted over the direct link should be transmitted over the direct link after receipt of the TDLS Tx Path Switch Response in this case. And, in case of switch to a AP path, the first Data frame should be transmitted through the AP path after receipt of the TDLS Tx Path Switch Response. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange which is part of the direct link setup or the peer key handshake for this purpose.

A STA receiving a Data frame may send a TDLS Rx Path Switch Request frame to request a switch to the direct path or to the AP path. The reason requesting a path switch shall be included and the RSNIs on frames received from the AP and the STA receiving the Rx Path request may be included in this request frame. The reason shall be set to one of values in Table z10. A STA receiving this request frame sends the TDLS Rx Path Switch Response frame to the requesting STA after deciding whether the path switch request is acceptable or not based on information included in TDLS Rx Path Request frame or itself. The result of the request shall be included and the RSNI on frames sent to the AP from the STA receiving the TDLS Rx Path Request frame may be included in this response frame. The result shall be set to one of values in Table z12.

When a STA receiving a Data frame intends to enter a power save state, it sends a TDLS Rx Path Switch Request frame to the peer STA, requesting indicating a switch to the AP path. Upon receipt of a TDLS Rx Path Switch Request indicating a switch to the AP path, the receiving A STA receiving this request frame shall cease transmissions over the direct link as soon as possible. When no further Data frames will be transmitted over the direct link, the responding STA shall send a TDLS Rx Path Switch Response, echoing the switch to the AP path. The requesting STA may enter a power save mode after receiving the TDLS Rx Path Switch Response.

A STA may request a peer STA to enable the direct Rx path by sending a TDLS Tx Path Switch Request indicating a switch to the direct path. Upon receipt of the TDLS Tx Path Switch Request indicating the direct path, the receiving A STA shall enable the direct Rx path and respond with a TDLS Tx Path Switch Response echoing the switch to the direct path. The requesting station shall not transmit frames on the direct link before receiving the TDLS Tx Path Switch Response.

A STA that is enabling its direct Rx path may inform its peer STA by sending a TDLS Rx Path Switch Request indicating a switch to the direct path. The responding STA responds by sending a TDLS Rx Path Switch Response echoing the switch to the direct Rx path. The responding STA may then transmit over the direct path, but does not have to.

To tear down a direct link, the STA sends a TDLS Teardown Request to the peer STA, after which the STA shall not transmit on the direct link any longer. Upon receipt of the TDLS Teardown Request, the peer STA shall disable the direct Rx and Tx paths and destroy the related security parameters, and then respond with a TDLS Teardown Response. Upon receipt of the TDLS Teardown Response, the STA which initiated the teardown shall disable the direct Rx path and destroy the related security parameters.