IEEE P802.11
Wireless LANs

| 802.11 TGn LB115 Submission MAC BA |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 10/12/2007 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB115 comments assigned to the author:

The changes marked in this document are based on TGn Draft version D3.01.

They relate to:

- TGn Ad-hoc: MAC

- Comment group:  MAC BA

# Comments

| CID | Page | Line | Clause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 5163 | 127.54 | 54 | 9.10.4 | The lost MSDU should be considered here. Otherwise 11 MAC may send out of order frame to the upper layer which is not allowed. | Change the sentence to: Upon arrival of a BlockAckReq frame, the recipient shall pass up the MSDU and A-MSDU starting with the starting sequence number sequentially until there is an incomplete MSDU or A-MSDU or lost MSDU. | Reject This issue is not related to the .11n. All problems found in the basic spec should be fixed in the TGmb. |
| 5164 | 127.59 | 59 | 9.10.4 | The lost MSDU should be considered here. Otherwise 11 MAC may send out of order frame to the upper layer which is not allowed. The A-MSDU shall also be included here. | Change the sentence to: If, after an MPDU is received, the buffer is full, the complete MSDU or A-MSDU with the earliest sequence number does not have lost MSDU or A-MSDU before it, the complete MSDU or A-MSDU shall be passed up the MAC protocol stack. If after an MPDU is received, the buffer is full, the complete MSDU or A-MSDU with the earliest sequence number has lost MSDU or A-MSDU before it, the received MPDU shall be discarded. | Reject This issue is not related to the .11n. All problems found in the basic spec should be fixed in the TGmb. |
| 5009 | 127.59 | 59 | 9.10.4 | "…,the complete MSDU with the earliest sequence number shall be …" A-MSDU should be added, too. | Change the cited part to "…, the complete MSDU or A-MSDU with the earliest sequence number shall be …" | Accept |
| 5621 | 134.33 | 33 | 9.10.7.9 | "the originator may solicit a BlockAck as the last activity associated with that Block Ack agreement in the current TXOP." There's nothing to stop it already doing so. | Turn whole sentence into an informative note thus: "NOTE--the originator can solicit a BlockAck as the last activity associated with that Block Ack agreement in the current TXOP to reduce the probability that data is unecessarily retransmitted due to loss of partial state." | Accept |