IEEE P802.11
Wireless LANs

| TGu MIB Modifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

This submission addresses CIDs 418, 419, 421, 422, 423, 424, 551, 595, 797, 798, 799, and 800 from 802.11 Letter Ballot#107, all relating to the specification of the Interworking MIB.

# Introduction

This submission modifies the MIB in Annex D of the 802.11u draft, to resolve the following comments:

- CID 418, which states there is no definition for dot11NonApStaBackgroundFrameCount.  This document adds the definition of the counter.

- CID 419, which states that dot11NonApStaAuthBackgroundDelay is defined by not part of the dot11InterworkingTable entry definition.  This comment is resolved by the addition of the element to the table definition.

- CID 421, which states that the dot11GeospatialLocation entry is called a table in its definition, but its name does not reflect that.  The table construction is necessary because the structure of MIB variables does not allow for floating point definitions.  This comment is rejected.

- CID 424 refers to the civic location, and the substance of the comment is very similar to CID 421.  It is rejected for the same reason.

- CID 422 and 423 refer to the definition of dot11ApGeospatialAltitudeType.  The definition of the altitude type is present in the draft as dot11ApGeospatialType.  Both of these comments are resolved by synchronizing the two names.

- CID 797, 798, 799, and 800 request that ranges be placed on values in the Geospatial MIB, and CIDs 551 and 595 request synchronization with the TGy MIB.  The ranges incorporated into this submission are from 802.11y-D7.0.

# Editing Instructions

Change the dot11InterworkingEntry as follows:

dot11InterworkingEntry ::=

SEQUENCE {

dot11NonApStaMacAddress                    MacAddress,

dot11NonApStaUserIdentity                  SnmpAdminString,

dot11NonApStaInterworkingCapability        BITS,

dot11NonApStaAssociatedSSID	      OCTET STRING,

dot11NonApStaCipherSuite                   OCTET STRING,

dot11NonApStaAuthAccessCategories          BITS,

dot11NonApStaAuthMaxVoiceRate	      Unsigned32,

dot11NonApStaAuthMaxVideoRate              Unsigned32,

dot11NonApStaAuthMaxBestEffortRate         Unsigned32,

dot11NonApStaAuthMaxBackgroundRate         Unsigned32,

dot11NonApStaAuthHCCA                      TruthValue,

dot11NonApStaAuthMaxHCCARate               Unsigned32,

dot11NonApStaAuthHCCADelay                 Unsigned32,

dot11NonApStaAuthSinkMulticast             TruthValue,

dot11NonApStaAuthBackgroundDelay           TruthValue,

dot11NonApStaMaxAuthSourceMulticast        TruthValue,

dot11NonApStaMaxAuthSourceMulticastRate    Unsigned32,

dot11NonApStaVoiceFrameCount               Counter32,

dot11NonApStaVideoFrameCount               Counter32,

dot11NonApStaBestEffortFrameCount          Counter32,

dot11NonApStaBackgroundFrameCount          Counter32,

dot11NonApStaHCCAFrameCount                Counter32,

dot11NonApStaVoiceOctetCount               Counter32,

dot11NonApStaVideoOctetCount               Counter32,

dot11NonApStaBestEffortOctetCount          Counter32,

dot11NonApStaBackgroundOctetCount          Counter32,

dot11NonApStaHCCAOctetCount                Counter32,

dot11NonApStaPowerManagementMode           INTEGER,

dot11nonApStaAuthDls                       TruthValue,

dot11nonApStaVlanId                        Unsigned32,

dot11nonApStaVlanName                      OCTET STRING,

dot11nonApStaAddtsResultCode               INTEGER

}

Insert the following entry following the dot11NonApStaAuthSinkMulticast, and renumber the following entries as indicated:

dot11NonApStaBackgroundFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented for

each MSDU successfully transmitted by the AP on the background
		access category and for each MSDU successfully

received on either user priority 1 or 2."

::= { dot11InterworkingEntry 22 }

dot11NonApStaHCCAFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For HCCA operation, this counter shall be incremented for

each MSDU successfully transmitted by the AP and for each

MSDU successfully received on either."

::= { dot11InterworkingEntry 23 }

dot11NonApStaVoiceOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented

by the length in octets in each MSDU successfully

transmitted by the AP on the

voice access category and by the length in octets in each MSDU

successfully received on either user priority 6 or 7."

::= { dot11InterworkingEntry 24 }

dot11NonApStaVideoOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented by

the length in octets in each MSDU successfully transmitted

by the AP on the video access category and by the length

in octets in each MSDU successfully received on either

user priority 4 or 5."

::= { dot11InterworkingEntry 25 }

dot11NonApStaBestEffortOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented by

the length in octets in each MSDU successfully transmitted

by the AP on the best effort access category and by the

length in octets in each MSDU successfully received on

either user priority 0 or 3."

::= { dot11InterworkingEntry 26 }

dot11NonApStaBackgroundOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For EDCA operation, this counter shall be incremented by

the length in octets in each MSDU successfully transmitted

by the AP on the background access category and by the length

in octets in each MSDU successfully received on either user

priority 1 or 2."

::= { dot11InterworkingEntry 27 }

dot11NonApStaHCCAOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"For HCCA operation, this counter shall be incremented by

the length in octets in each MSDU successfully transmitted

by the AP and by the length in octets in each MSDU

successfully received."

::= { dot11InterworkingEntry 28 }

dot11NonApStaPowerManagementMode OBJECT-TYPE

SYNTAX INTEGER { active(1), powersave(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the power management

mode of the non-AP STA."

::= { dot11InterworkingEntry 29 }

dot11nonApStaAuthDls OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

non-AP STA is permitted by the SSPN Interface to use

direct link service (DLS).  This object does not mean

the AP is capable of providing DLS service.  This service

is disabled otherwise."

DEFVAL ( true )

::= { dot11InterworkingEntry 30 }

dot11nonApStaVlanId OBJECT-TYPE

SYNTAX 		Unsigned32 (0..4095)

MAX-ACCESS 	read-only

STATUS 		current

DESCRIPTION

"This attribute indicates the VLAN ID on the DS to which

frames from the non-AP STA are bridged."

::= { dot11InterworkingEntry 31 }

dot11nonApStaVlanName OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..64))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the VLAN name corresponding to

The VLAN ID on the DS to which frames from the non-AP STA

are bridged."

::= { dot11InterworkingEntry 32 }

dot11nonApStaAddtsResultCode OBJECT-TYPE

SYNTAX INTEGER {

SUCCESS(1),

INVALID_PARAMETERS(2),

REJECTED_WITH_SUGGESTED_CHANGES(3),

REJECTED_FOR_DELAY_PERIOD(4)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the most recent result code

returned by the AP in an ADDTS Response."

::= { dot11InterworkingEntry 33 }

Change the ranges on the following objects in the dot11APGeospatialLocation table as indicated:

dot11ApGeospatialLocationLatitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the 9

bits of integer portion of Latitude."

::= { dot11ApGeospatialLocation 2 }

dot11ApGeospatialLocationLatitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the

25 bits of fraction portion of Latitude."

::= { dot11ApGeospatialLocation 3 }

dot11ApGeospatialLocationLongitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-359..359)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the 9

bits of integer portion of Longitude."

::= { dot11ApGeospatialLocation 5 }

dot11ApGeospatialLocationLongitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-16777215..16777215)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction. This field contains the 25

bits of fraction portion of Longitude."

::= { dot11ApGeospatialLocation 6 }

dot11ApGeospatialLocationAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude Type is four bits encoding the type of altitude.

Codes defined are:meters : in 2s-complement fixed-point

22-bit integer part with 8-bit fraction floors: in 2s-

complement fixed-point 22-bit integer part with 8-bit

fraction"

::= { dot11ApGeospatialLocation 7 }

dot11ApGeospatialLocationAltitudeInteger OBJECT-TYPE

SYNTAX Integer32 (-2097151..2097151)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type

field. The field is encoded as a 2s-complement fixed-point

22-bit integer Part with 8-bit fraction. This field contains

the fixed-point Part of Altitude."

::= { dot11ApGeospatialLocation 9 }

dot11ApGeospatialLocationAltitudeFraction OBJECT-TYPE

SYNTAX Integer32 (-127..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Altitude is a 30 bit value defined by the Altitude type

field. The field is encoded as a 2s-complement fixed-point

22-bit integer Part with 8-bit fraction. This field contains

the fraction part of Altitude."

::= { dot11ApGeospatialLocation 10 }

802.11u-D1.0

802.11y-D7.0