IEEE P802.11            
Wireless LANs

| Normative text changes for CID #103 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kevin Stanton | Intel Corp | 2111 NE 25th Ave, Hillsboro OR |  | Kevin.Stanton@intel.com |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR 97229 | 503-264-7799 | Emily.H.Qi@intel.com |

Abstract

This document is a normative text propos to provide comment resolutions for TGv LB108 – CID103.

The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

TGv editor: Change 10.3.50 clause as follows and renumber the following clauses:

- Location request

This set of primitives supports the exchange of location parameter information between peer SMEs. Error: Reference source not found depicts the location request and response process. The figure illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses

- Location Request and Response Protocol Exchange

- MLME-LOCATIONREQUEST.request

- Function

This primitive requests the transmission of a location request action frame to a peer entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONREQUEST.request	(
Peer MAC Address,
Dialog Token,
Location Parameters)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the location request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the SME to request that a Location Request frame be sent to a peer entity to convey location information.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Location Request frame containing the set of location elements specified. This frame is then scheduled for transmission.

- MLME-LOCATIONREQUEST.confirm

- Function

This primitive reports the result of a location request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONREQUEST.confirm	(

	
Dialog Token,
ResultCode,
TX Timestamp,
RX Timestamp

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Indicates the result of the corresponding MLME-LOCATION.request. |
| TX Timestamp | 12 octets as defined in Table v29 | 12 octets as defined in Table v29 | The TX Timestamp field contains the time at which the unicast Location Request frame was transmitted by a STA, defined to occur at the PHY-TXEND.indication of the transmitting Location Request frame. The TX Timestamp field is composed of the time fields defined in Table v29. |
| RX Timestamp | 12 octets as defined in Table v29 | 12 octets as defined in Table v29 | The RX Timestamp field contains the time at which the ACK frame in response to the Location Request frame was received by a STA, defined to occur at the PHY-RXEND.indication of the received ACK frame. The RX Timestamp field is composed of the time fields defined in Table v29. |

- When generated

This primitive is generated by the MLME when the request to transmit a Location Request frame completes.

- Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

- MLME-LOCATIONREQUEST.indication

- Function

This primitive indicates that a Location Request frame has been received requesting a location transaction.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Location Parameters)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the location request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the MLME when a valid Location Request frame is received.

- Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the location transaction.

- Location response

This set of primitives supports the exchange of location parameters between peer SMEs.

10.3.51.1 MLME-LOCATIONRESPONSE. request

10.3.51.1.1 Function

This primitive requests the transmission of location information to a peer entity, in response to a received Location Request frame.

10.3.51.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONRESPONSE. request	(
Peer MAC Address,
Dialog Token,
Management Action Pending,
Location Parameters)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Management Action Pending | Integer | 0 – 255 | The management action pending value indicates if a management action is pending for the STA. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

10.3.51.1.3

When generated

This primitive is generated by the SME to request that a Location Response frame be sent to a peer entity to convey location information.

10.3.51.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Location Response frame containing the set of location elements specified. This frame is then scheduled for transmission.

      - MLME-LOCATIONRESPONSE.confirm

        - Function

This primitive reports the result of a request to send a location response action frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONRESPONSE.confirm	(

Dialog Token,
ResultCode)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Location Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit a Location Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-LOCATIONRESPONSE.indication

        - Function

This primitive indicates that a Location Response frame has been received.

10.3.51.3.2  Semantics of the service primitive

The primitive parameters are as follows:

MLME- LOCATIONRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Management Action Pending,
Location Parameters)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location response shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Management Action Pending | Integer | 1 – 255 | The management action pending value indicates if a management action is pending for the STA. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

        - When generated

This primitive is generated by the MLME when a valid Location Response frame is received..

        - Effect of receipt

On receipt of this primitive, the SME evaluates the Location Parameters.