IEEE P802.11
Wireless LANs

Normative Text for Channel Allocation and Usage

Date: 2008-01-01

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Michelle Gong | Intel Corporation | 2200 Mission College Santa Clara, CA 95054 | 408-765-7994 | Michelle.x.Gong@intel.com |
| Brian Hart | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 |  | brianh@cisco.com |
| Jari Jokela | Nokia | Visiokatu 3, Tampere, Finland | +358504860445 | Jari.jokela@nokia.com |
| Emily Qi | Intel Corporation | 2111 NE 25th Ave, Hillsboro OR | 503-264-7799 | Emily.h.qi@intel.com |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |

Abstract

This submission contains the normative text for Channel Allocation and Usage to assist in the channel/frequency allocation and usage for non-802.11-infrastructure networking (e.g. IBSS, Mesh, Bluetooth), or other uses.

This proposal provides comment resolution for LB108 CID #102, and the text is aligned with P802.11v-D1.03.

2. Normative references

TGv editor: Add the following references, alphabetizing as appropriate

IEEE Std 802.15.1-2005 Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Wireless Personal Area Networks (WPANs)

IEEE Std 802.15.4-2006 Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specifications for Low-Rate Wireless Personal Area Networks (WPANs)

7.2.3.8 Probe Request frame format

TGv editor: Insert the following rows in Table 7-14:

| Order | Information | Notes |
| --- | --- | --- |
|  | Channel Allocation | The Channel Allocation element may be present if dot11WirelessManagementImplemented is true and dot11WNMChannelAllocationEnabled is true. |
|  | Supported Regulatory Classes | The Supported Regulatory Classes element is present if Channel Allocation element is present. |

- Probe Response frame format

TGv editor: Insert the following rows in Table 7-15:

| Order | Information | Notes |
| --- | --- | --- |
|  | Channel Allocation | The Channel Allocation element is present if the Channel Allocation element is present in the Probe Request frame, dot11WirelessManagementImplemented is true and dot11WNMChannelAllocationEnabled is true. |

- Reason Code field

Insert the following new row before the entry “Reserved” and change the entry “Reserved” in Table 7-22 as follow:

| - Reason codes |  |
| --- | --- |
| Reason code | Meaning |
| 47 | Disassociated because the non-AP STA is unable to operate non-802.11-infrastructure at the recommended channels. |
| 468-65535 | Reserved |

### 7.3.2 Information Elements

TGv editor: Insert the following rows into Table 7-26:

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Channel Allocation (see 7.3.2.81) | ANA | 4 to 256 |

- Extended Capabilities information element

TGv editor: Insert the following rows into Table 35a:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Channel Allocation | The Channel Allocation set to 1 indicates the STA supports Channel Allocation procedure as described in 11.20.15. The Channel Allocation bit set to 0 indicates that the STA does not support this capability. |

TGv editor: Insert the following after 7.3.2.80:

##### 7.3.2.81 Channel Allocation element

##### 7.3.2.81.1 Channel Allocation element definition

The Channel Allocation element defines the channel allocation and usage information for non-802.11- infrastructure networking. The format of the Channel Allocation element is shown in Figure v-x1.

|  |  |  | One or more sub-elements |
| --- | --- | --- | --- |
|  | Element ID | Length | Channel Allocation Sub-elements |
| Octets: | 1 | 1 | Variable |

Figure v-x1. Channel Allocation element format.

The Element ID field is equal to the Channel Allocation value in Table 26.

The Length field is set to the total length in octets of all Channel Allocation Sub-elements contained in the element.

The Channel Allocation Sub-elements field contains one or more Channel Allocation sub-elements described in Table v-x1. Sub-elements are ordered by increasing sub-element ID. The number of times that a sub-element is included in a Channel Allocation element is indicated in Table v-x1. The format of Channel Allocation sub-elements is described in 7.3.2.81.2 through 7.3.2.81.5.

The Channel Allocation element may be included in Probe Request frames, as described in 7.2.3.8, Probe Response frames, as described in 7.2.3.9, Channel Allocation Request frames, as described in 7.4.7a.8, and Channel Allocation Response frames, as described in 7.4.7a.9. The use of the Channel Allocation element and frames is described in 11.20.15.

Table v-x1 Channel Allocation Sub-element Types

| Channel Allocation Sub-element | Sub-element IDs | Sub-element Length field (in octets) | Number of Inclusions |
| --- | --- | --- | --- |
| Reserved | 0 |  |  |
| 802.11 Non-infrastructure | 1 | 0 to 254 | 0 or 1 |
| Bluetooth | 2 | 0 to 79 | 0 or 1 |
| Zigbee | 3 | 0 to 27 | 0 or 1 |
| CSS | 4 | 0 to 14 | 0 or 1 |
| Reserved | 5-253 |  |  |
| Unspecified Network | 254 | 0 to 255 | 0 or more |
| Reserved | 255 |  |  |

##### 7.3.2.81.2 802.11 Non-infrastructure sub-element

The 802.11 Non-infrastructure sub-element contains a sub-element ID and a list of channels that can be used for 802.11 Non-infrastructure networking. The format of the 802.11 Non-infrastructure sub-element is shown in Figure v-x2.

|  | Sub-element ID | Length (2*n) | Regulatory Class 1 | Channel Number 1 | ... | Regulatory Class n | Channel Number n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |  | 1 | 1 |

Figure v-x2 802.11 Non-infrastructure sub-element format

The Sub-element ID field is set to the 802.11 Non-infrastructure value in Table v-x1.

The Length and Regulatory Class/Channel Number pair fields are present only if the Channel Allocation element is included in the Probe Response frame and Channel Allocation Response frame.

The Length field has units of octets and is set to twice the number of Regulatory Class/Channel Number pairs.

There are n Regulatory Class/Channel Number pairs, indicated by 1 through n. Each Regulatory Class/Channel Number pair comprises a Regulatory Class and a Channel Number.

The Regulatory Class 1 through Regulatory Class n fields each contains an enumerated value from Annex J, specifying the regulatory class in which the paired Channel Number fields are valid.

The Channel Number 1 through Channel Number n fields each contains a single channel number.

Channel numbering is dependent on paired Regulatory Class according to Annex J.

##### 7.3.2.81.3 Bluetooth sub-element

The Bluetooth sub-element contains a list of frequency ranges that can be used for Bluetooth networking. The format of the Bluetooth sub-element is shown in Figure v-x3.

|  | Sub-element ID | Length (2*n) | Lower Hop Frequency 1 | Upper Hop Frequency 1 | ... | Lower Hop Frequency n | Upper Hop Frequency n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |  | 1 | 1 |

Figure v-x3 Bluetooth sub-element format

The Sub-element ID field is set to the Bluetooth value in Table v-x1.

The Length and hop frequency pair fields are present only if the Channel Allocation element is included in the Probe Response frame and Channel Allocation Response frame

The Length field has units of octets and is set to twice the number of hop frequency pairs.

There are n hop frequency pairs, indicated by 1 through n. Each hop frequency pair comprises a Lower Hop Frequency and an Upper Hop Frequency. Hop frequency values are defined in 802.15.1, clause 7.1. Each hop frequency pair indicates a range of hop frequencies from the Lower Hop Frequency to the Upper Hop Frequency inclusive.

##### 7.3.2.81.4 Zigbee sub-element

The Zigbee sub-element contains a list of channels that can be used for Zigbee networking. The format of the Zigbee sub-element is shown in Figure v-x4.

|  | Sub-element ID | Length | List of Channel Numbers |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | n |

Figure v-x4 Zigbee sub-element format

The Sub-element ID field is set to the Zigbee value in Table v-x1.

The Length and List of Channel Numbers fields are present only if the Channel Allocation element is included in the Probe Response frame and Channel Allocation Response frame.

The Length field is set to the length in octets of the List of Channel Numbers.

The List of Channel Numbers contains a variable number of octets, where each octet indicates a single channel number.

Channel number values are defined in 802.15.4, clause 6.1.2.

##### 7.3.2.81.5 CSS sub-element

The CSS sub-element contains a list of channels that can be used for CSS networking. The format of the CSS sub-element is shown in Figure v-x5.

|  | Sub-element ID | Length (n) | List of Channel Numbers |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | n |

Figure v-x5 CSS sub-element format

The Sub-element ID field is set to the CSS value in Table v-x1.

The Length and List of Channel Numbers fields are present only if the Channel Allocation element is included in the Probe Response frame and Channel Allocation Response frame.

The Length field is set to the length in octets of the List of Channel Numbers.

The List of Channel Numbers contains a variable number of octets, where each octet indicates a single channel number.

Channel number values are defined in 802.15.4, clause 6.1.2.1a.

##### 7.3.2.81.5 Unspecified sub-element

The Unspecified sub-element contains a list of frequency ranges that can be used for networking systems which are not listed in other Channel Allocation sub-elements that are included in the Channel Allocation element. The format of the Unspecified sub-element is shown in Figure v-x5.

|  | Sub-element ID | Length (4n+1) | Frequency Units | Lower Frequency 1 | Upper Frequency 1 | ... | Lower Frequency n | Upper Frequency n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 2 |  | 2 | 2 |

Figure v-x5 Unspecified sub-element format

The Sub-element ID field is set to the Unspecified value in Table v-x1.

The Length, Frequency Units, and frequency ranges fields are present only if the Channel Allocation element is included in the Probe Response frame and Channel Allocation Response frame.

The Length field is set to four times the number of frequency ranges plus one (4*n+1).

The Frequency Units field indicates the units of the Lower Frequency 1 through Lower Frequency n and Upper Frequency 1 through Upper Frequency n fields. The values of the Frequency Units field are shown in Table v-x2.

Table v-x2 – Unspecified Units field

| Value of Unspecified Units field | Units of Base Frequency field |
| --- | --- |
| 0 | Reserved |
| 1 | 1kHz |
| 2 | 10kHz |
| 3 | 100kHz |
| 4 | 1MHz |
| 5 | 10MHz |
| 6 | 100MHz |
| 7 | 1GHz |
| 8 | 10GHz |
| 9 | 100GHz |
| 10-255 | Reserved |

There are n frequency ranges, indicated by 1 through n. Each frequency range comprises a Lower Frequency field and an Upper Frequency field. The Lower Frequency and Upper Frequency fields are unsigned integers. The indicated frequency range is from the Lower Frequency to the Upper Frequency inclusive, in units indicated by the Frequency Units field.

### 7.4.7a Public Action frame details

Note 7.4.7a is defined in 802.11y D5.0.

### 7.4.7a.1 Public Action frame

Insert the following rows in table 7-57ab.

Table 7-57ab Public Action field values

| Action field value | Description |
| --- | --- |
| ANA | Channel Allocation Request |
| ANA+1 | Channel Allocation Response |
| ANA+2-255 | Reserved |

Insert the following after 7.4.7a.7( last clause number used in 802.11y D.50):

##### 7.4.7a.8 Channel Allocation Request frame format

The Channel Allocation Request frame is sent by a non-AP STA to the AP to request the specified channel allocation information.  The format of the Channel Allocation Request frame is defined in Figure v-x6.

|  | Category | Action | Dialog Token | Request Reason | Channel Allocation Element | Supported Regulatory Classes Element |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable | variable |

Figure v-x6. Channel Allocation Request frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Allocation Request frame, as specified in 7.4.7a.1.

The Dialog Token field is set to a non-zero value chosen by the non-AP STA sending the Channel Allocation Request frame to identify the request/response transaction.

The Request Reason field is set to a number that identifies request reasons as defined in Table v-x3.

The Channel Allocation Element field includes one Channel Allocation element described in 7.3.2.81 to identify the request types.

Table v-x3 Channel Allocation Request Reason Code

| Request Reason Code | Description |
| --- | --- |
| 0 | General request |
| 1 | Incapable to operate at the existing channel list that AP provided due to limitation of supported regulatory classes |
| 2 | Incapable to operate at the existing channel list that AP provided due to interference. |
| 3-253 | Reserved |
| 254 | No longer in use: no longer to operate the corresponding network type. |
| 255 | Reserved |

The Supported Regulatory Classes Element field includes a Supported Regulatory Classes element to indicate the support regulatory classes for the requested network type and consistent with the Country element in the Beacon frame of which the AP advertises. The Supported Regulatory Classes is described in 7.3.2.51. This field is present when the Request Reason field is set to 1.

Editorial Note: 7.3.2.51 is defined in TGy D5.0.

##### 7.4.7a.9 Channel Allocation Response frame format

The Channel Allocation Response frame is sent by an AP STA in response to a Channel Allocation Request frame, or autonomously. The format of the Channel Allocation Response frame is shown in Figure v-x7

The Channel Allocation Response frame is sent by an AP in response to a Channel Allocation Request frame.

|  | Category | Action | Dialog Token | Channel Allocation Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure vn. Channel Allocation Response frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Allocation Response frame, as specified in 7.4.7a.1.

The Dialog Token field is set to the value in the corresponding Channel Allocation Request frame.

The Dialog token field is set to the nonzero value received in the Channel Allocation Request frame if the Channel Allocation Response frame is being transmitted in response to a Channel Allocation Request frame. The Dialog token field is set to zero if the Channel Allocation Response frame is being transmitted, but not in response to a Channel Allocation Request frame.

The Channel Allocation Element includes one Channel Allocation element described in 7.3.2.81.

- 10. Layer Management

- 10.3 MLME ASP Interface

TGv editor: Insert the following after 10.3.57

    - Channel Allocation request Setup

The following MLME primitives support the signaling of Channel Allocation request Setup. Figure v-x101 depicts the Channel Allocation setup process. The figure illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

Figure v-x101 Channel Allocation Setup Protocol Exchange

      - MLME-CHANNELALCT.request

10.3.58.1.1 Function

This primitive requests the transmission of a Channel Allocation Request frame be sent to an AP.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELALCT.request	(
PeerSTAAddress,
Dialog Token,
Request Reason,
ChannelAllocation
SupportedRegulatoryClasses)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Channel Allocation process. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Allocation request and response transaction. |
| Request Reason | Integer | 0 – 255 | Specifies the request reason for Channel Allocation Request. |
| ChannelAllocation | As defined in Channel Allocation element | As defined in Channel Allocation element | Specifies request types for the Channel Allocation request. |
| SupportedRegulatoryClasses | As defined in Supported Regulatory Classes element | As defined in Supported Regulatory Classes element | Specifies the Supported Regulatory Classes information for the Channel Allocation Request. |

- When Generated

This primitive is generated by the SME to request that a Channel Allocation Request frame be sent to the AP with which the STA may be associated or unassociated.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Allocation Request action management frame. The STA then attempts to transmit this to the AP with which the STA may be associated or unassociated.

- MLME-CHANNELALCT.confirm

- Function

This primitive reports the result of a Channel Allocation procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELALCT.confirm(
ResultCode,
PeerSTAAddress,
Dialog Token,
ChannelAllocation)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, MALFORMED REQUEST, REQUESTED INTERVAL TOO LONG, or OVERRIDDEN DUE TO LACK OF RESOURCES | Reports the outcome of a request to send a Channel Allocation Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Channel Allocation process. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Allocation request and response transaction. |
| ChannelAllocation | As defined in Channel Allocation element | As defined in Channel Allocation element | Specifies parameters for the Channel Allocation. |

- When Generated

This primitive is generated by the MLME as a result of an MLME-CHANNELALCT.request and indicates the results of the request.

This primitive is generated when the MLME-CHANNELALCT.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a Channel Allocation Response frame from the AP.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.15.

- MLME-CHANNELALCT.indication

- Function

This primitive indicates that a Channel Allocation Request frame was received from a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELALCT.indication(
PeerSTAAddress,
Dialog Token, 
Request Reason,
ChannelAllocaiton,
SupportedRegulatoryClasses)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Channel Allocation Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Allocation request and response transaction. |
| Request Reason | Integer | 0 – 255 | Specifies the request reason for Channel Allocation Request. |
| ChannelAllocation | As defined in Channel Allocation element | As defined in Channel Allocation element | Specifies request types for the Channel Allocation request. |
| SupportedRegulatoryClasses | As defined in Supported Regulatory Classes element | As defined in Supported Regulatory Classes element | Specifies the Supported Regulatory Classes information for the Channel Allocation Request. |

- When Generated

This primitive is generated by the MLME when a valid Channel Allocation Request frame is received.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.15.

- MLME-CHANNELALCT.response

- Function

This primitive is generated in response to an MLME-CHANNELALCT.indication requesting a Channel Allocation Response frame is sent to a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELALCT.response(
PeerSTAAddress,
Dialog Token,
ChannelAllocation)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Channel Allocation Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Allocation request and response transaction. |
| Channel Allocation | As defined in Channel Allocation element | As defined in Channel Allocation element | Specifies parameters for the Channel Allocation. |

- When Generated

This primitive is generated by the SME in response to an MLME-CHANNELALCT.indication requesting a Channel Allocation Response be sent to a non-AP STA.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Allocation Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

Insert new subclause at the end of 11.20

### 11.20.15 Channel Allocation Procedures

Channel Allocation information is a set of channel lists or frequency ranges provided by an AP to non-AP STAs for non-AP STAs’ operation on a non-802.11-infrastructure network (e.g. IBSS, Mesh, Bluetooth, etc.).  A STA supporting use of Channel Allocation shall indicate such support by setting the Channel Allocation bit in the Extended Capabilities information element.

A non-AP STA that is not associated to an AP and prior to using a non-802.11-infrastructure network may send a Probe Request frame including both Supported Regulatory classes element and Channel Allocation element.

A non-AP STA supporting use of Channel Allocation may send a Channel Allocation Request frame at any time after association to the AP supporting use of Channel Allocation to request the channel allocation information for the specific networks or supported regulatory classes.  A non-AP STA may also send a Channel Allocation Request frame to the AP with the Request Reason field set to “No longer in use” to indicate that the STA no loner operates the non-802.11-infrastructure network at the channel list or frequency ranges that were provided in the previous Channel Allocation Response frame.

Upon the receipt of Channel Allocation element in the Probe Request frame or Channel Allocation Request frame, the AP shall send a Probe Response frame or Channel Allocation Response frame including the corresponding Channel Allocation sub-elements in the Channel Allocation element if the Request Reason field is not set to “254”. The AP supporting Channel Allocation shall include at least one Channel Allocation sub-element for the corresponding request type or at least one Unspecified sub-element.

The AP may send an unsolicited broadcast or unicast Channel Allocation Response frame to the STAs supporting use of Channel Allocation if the corresponding Channel Allocation information needs to be updated.

On the receipt of Channel Allocation IE in the Probe Response frame or Channel Allocation Response frame, the recipient should operate the non-802.11-infrastructure network at the corresponding channel list or frequency ranges that are provided in the Channel Allocation Response frame. If the recipient is not able to operate at the suggested channel list or frequency ranges due to interference or limitation of supported regulatory classes, the STA may send a new Channel Allocation Request frame with the Request Reason field set to one of “Incapable” reason codes defined in Table v-x3.

In the overlapped BSSs area, the non-AP STA should operate the non-802.11-infrastructure network at the corresponding channel list or frequency ranges that are provided by the associated AP if the non-AP STA is associated with an AP. If the non-AP STA is not associated with any APs, the STA should operate the non-802.11-infrastructure network at the corresponding channel list or frequency ranges that are provided by the AP of which the RSSI value of the Beacon or Probe Response is the strongest.

Insert the following rows to A.4.14

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM15 | Channel Allocation | 11.20.15; 7.3.2.81; 7.4.7a.8;7.4.7a.9 | CFv:O | Yes, No, N/A |

# Annex D

Insert the following entries at the end of the Dot11WirelessMgmtOptionsEntry sequence list:

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionPresenceEnabledTruthValue,

dot11MgmtOptionFBMSImplementedTruthValue,

dot11MgmtOptionFBMSEnabledTruthValue,

dot11MgmtOptionEventsEnabledTruthValue,

dot11MgmtOptionDiagnosticsEnabledTruthValue,

dot11MgmtOptionMultiBSSIDImplementedTruthValue,

dot11MgmtOptionMultiBSSIDEnabledTruthValue,

dot11MgmtOptionTFSImplementedTruthValue,

dot11MgmtOptionTFSEnabledTruthValue, dot11MgmtOptionSleepModeImplementedTruthValue,

dot11MgmtOptionSleepModeEnabled TruthValue,

dot11MgmtOptionTIMBroadcastImplemented	 TruthValue,

dot11MgmtOptionTIMBroadcastEnabled TruthValue,

dot11MgmtOptionProxyARPEnabled TruthValue,

dot11MgmtOptionBSSTransitionEnabled TruthValue,

dot11MgmtOptionChannelAllocationImplemented	TruthValue,

dot11MgmtOptionChannelAllocationEnabled	TruthValue}

On page 155, as part of Dot11WirelessMgmtOptionsEntry, add the followingMIB objects:

dot11MgmtOptionChannelAllocationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Channel Allocation when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionChannelAllocationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Channel Allocation is enabled. Channel Allocation is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 17 }