IEEE P802.11
Wireless LANs

Normative Text for Directed Multicast

Date: 2008-07-15

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, UK | +44 1784 848770 | aashley@nds.com |
| Raja Banerjea | Marvell Semiconductor | 5488 Marvell Lane, Santa Clara, CA 95054 | 408.222.3713 | rajab@marvell.com |
| Oren Kaidar | Intel Corporation |  |  | Oren.Kaidar@intel.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave, Sunnyvale CA | 408-754-1213 | subbu@arubanetworks.com |
| Emily Qi | Intel Corporation | 2111 NE 25th Ave, Hillsboro OR | 503-264-7799 | Emily.h.qi@intel.com |

Abstract

This submission contains the normative text for directed multicast delivery service which allows AP to transmit group addressed frames as individually addressed frames to the requesting STA.

This submission provides comment resolution for CID#269 (LB#133), and the proposed text is aligned with P802.11v-D3.0.

##### 3 Definitions

Insert the following new definitions:

3.x1 directed multicast service (DMS): A service in which an AP can transmit group addressed frames as individually addressed frames to the requesting non-AP STA.

- Abbreviations and acronyms

Insert the following new acronyms:

DMS	Directed Multicast Service

DMSID	Directed Multicast Service Identifier

- Extended Capabilities information element

TGv editor: Insert the following rows into Table 35a:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | DMS | The STA sets the DMS field to 1 When dot11MgmtOptionDMSEnabled is set to true and sets it to 0 otherwise. See 11.20.19. |

### 7.4.11 Wireless Network Management action details

TGv Editor: Insert the following rows in table v48

Table v48—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 23 | DMS Request |
| 24 | DMS Response |
| 2425-255 | Reserved |

Insert the following after 7.4.11.22

##### 7.4.11.23 DMS Request frame format

The DMS Request frame is sent by a non-AP STA to the AP to define information about a DMS request to the AP. The format of the DMS Request frame is defined in Figure v-x6.

|  |  |  |  |  | one or more DMS Descriptors |
| --- | --- | --- | --- | --- | --- |
|  | Category | Action | Dialog Token | Request Type | DMS Descriptor List |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure v-x6. DMS Request frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating DMS Request frame, as specified in 7.4.11.

The Dialog Token field is set to a non-zero value chosen by the non-AP STA sending the DMS Request frame to identify the request/response transaction.

The Request Type field is set to a number that identifies the type of DMS request. The Request Types are shown in Table v-x99.

Table v-x99 Request Type definitions

| Name | Event Type |
| --- | --- |
| Add | 0 |
| Remove | 1 |
| Change | 2 |
| Reserved | 3– 255 |

The DMS Descriptor List field contains one or more DMS Descriptors. The format of the DMS Descriptor is defined in Figure v-x9.

|  |  |  | zero or more TCLAS Elements |  |
| --- | --- | --- | --- | --- |
|  | DMSID | Length | TCLAS Elements | TCLAS Processing Element (optional) |
| Octets: | 1 | 1 | variable | 0 or 3 |

Figure v-x9. DMS Descriptor

The DMSID field is set to a non-zero value chosen by the non-AP STA identifying the DMS for the group addressed frames specified in the DMS Descriptor List field.

The Length field is set to the total length of presented TCLAS elements and presented TCLAS Processing element.

The TCLAS Elements field contains zero or more TCLAS information elements to specify group addressed frames as defined in 7.3.2.31.

The TCLAS Processing Element field is optionally present and defines how multiple TCLAS information elements that are processed as defined in 7.3.2.33.

##### 7.4.11.24 DMS Response frame format

The DMS Response frame is sent by an AP in response to a DMS Request frame, or autonomously. The format of the DMS Response frame is shown in Figure v-x7.

|  |  |  |  | one or more DMS Status |
| --- | --- | --- | --- | --- |
|  | Category | Action | Dialog Token | DMS Status List |
| Octets: | 1 | 1 | 1 | variable |

Figure v-x7. DMS Response frame format.

|  | DMSID | Status |
| --- | --- | --- |
| Octets: | 1 | 1 |

Figure v-x99. DMS Status.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating DMS Response frame, as specified in 7.4.11.

The Dialog token field is set to the nonzero value received in the DMS Request frame if the DMS Response frame is being transmitted in response to a DMS Request frame. The Dialog token field is set to zero if the DMS Response frame is being transmitted autonomously, and not in response to a DMS Request frame.

The DMS Status List field contains one or more DMS Statuses. The format of the DMS Status is defined in Figure v-x99.

The DMSID field is set to the value of the DMSID field in the DMS Descriptor received in the DMS Request frame indicating the group addressed frames.

The Status field indicates the status returned by the AP responding to the STA’s requested DMSID, as indicated in Table v34.

- 10. Layer Management

- 10.3 MLME ASP Interface

TGv editor: Insert the following after 10.3.60

    - DMS request procedure

The following MLME primitives support the signaling of DMS request procedure. The informative diagram shown in Figure v-x101 depicts the DMS request and response process and is not meant to be exhaustive of all possible protocol uses.

Figure v-x101 DMS Setup Protocol Exchange

      - MLME-DMS.request

10.3.61.1.1 Function

This primitive requests the transmission of a DMS Request frame be sent to an AP.

10.3.61.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DMS.request	(
PeerSTAAddress,
Dialog Token,
Request Type,
DMS Descriptor List)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the DMS process. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the DMS request and response transaction. |
| Request Type | Integer | 0– 255 | Specifies the Request type for the DMS Request frame. |
| DMS Descriptor List | A set of Length field, TCLAS elements and TCLAS Processing element | A set of Length field, TCLAS elements and TCLAS Processing element | Specifies group addressed frames for the requested DMS stream. |

10.3.61.1.3 When Generated

This primitive is generated by the SME to request that a DMS Request frame be sent to the AP with which the STA is associated.

10.3.61.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a DMS Request action management frame. The STA then attempts to transmit this to the AP with which the STA is associated.

10.3.61.2 MLME-DMS.confirm

10.3.61.2.1 Function

This primitive reports the result of a DMS procedure.

10.3.61.2.1 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DMS.confirm(
ResultCode,
PeerSTAAddress,
Dialog Token, 
DMS Status List)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, MALFORMED REQUEST, REQUESTED INTERVAL TOO LONG, or OVERRIDDEN DUE TO LACK OF RESOURCES | Reports the outcome of a request to send a DMS Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the DMS process. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the DMS request and response transaction. |
| DMS Status List | A set of DMS Status | A set of DMS Status | Specifies the status returned by the AP responding to the STA’s requested DMSID. |

10.3.61.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-DMS.request and indicates the results of the request.

This primitive is generated when the MLME-DMS.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a DMS Response frame from the AP.

10.3.61.2.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.19.

10.3.61.3 MLME-DMS.indication

10.3.61.3.1 Function

This primitive indicates that a DMS Request frame was received from a non-AP STA.

10.3.61.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DMS.indication(
PeerSTAAddress,
Dialog Token,
Request Type,
DMS Descriptor List )

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a DMS Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the DMS request and response transaction. |
| Request Type | Integer | 0– 255 | Specifies the Request type for the DMS Request frame. |
| DMS Descriptor List | A set of Length field, TCLAS elements and TCLAS Processing element | A set of Length field, TCLAS elements and TCLAS Processing element | Specifies group addressed frames for the requested DMS stream. |

10.3.61.3.3 When Generated

This primitive is generated by the MLME when a valid DMS Request frame is received.

10.3.61.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.19.

10.3.61.4 MLME-DMS.response

10.3.61.4.1 Function

This primitive is generated in response to an MLME-DMS.indication requesting a DMS Response frame is sent to a non-AP STA, or autonomously.

10.3.61.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-DMS.response(
PeerSTAAddress,
Dialog Token, 
DMS Status List)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a DMS Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the DMS request and response transaction. |
| DMS Status List | A set of DMS Status | A set of DMS Status | Specifies the status returned by the AP responding to the STA’s requested DMSID. |

10.3.61.4.3 When Generated

This primitive is generated by the SME in response to an MLME-DMS.indication requesting a DMS Response be sent to a non-AP STA.

10.3.61.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a DMS Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

Insert new subclause at the end of 11.20

11.20.19 DMS Procedures

The Directed Multicast Service (DMS) is a service that may be provided by an AP to its associated STAs that support the DMS service, where the AP transmits group addressed MSDUs as individually addressed A-MSDUs.

A STA that has a value of true for the MIB attribute dot11MgmtOptionDirectedMulticastEnabled is defined as a STA that supports of Directed Multicast. A STA for which the MIB attribute dot11MgmtOptionDirectedMulticastEnabled is true shall set the DMS field of the Extended Capabilities element to 1.

A non-AP STA may request use of DMS by sending a DMS Request frame that includes one or more DMS Descriptors with the Request Type field set to “Add” or “Change”.  The DMS Descriptor List field identifies group addressed frames to the AP which shall be transmitted to the requesting non-AP STA as individually addressed frames. Each requested DMS group frames is identified by a DMSID. In the TCLAS element, the Classifier Type subfield shall be set to the value 0 (Ethernet parameters), the Classifier Mask subfield shall be set to the value 2 , and the Destination Address subfield shall be set to the multicast address that the STA requests to receive as individually addressed frames.

Upon receipt of a DMS Request frame from a non-AP STA, the AP shall respond with a DMS Response frame if the Request Type field is “Add” and “Change”. A status value of “Accept” shall be set in the corresponded Status field of the DMS Status in the DMS Response frame when the AP accepts the DMS request for the requested DMSID. A status value of “Deny” shall be set in the corresponded DMS Status field of the DMS Status in the DMS Response frame when the AP denies the DMS request for the requested DMSID.

If the requested DMS is accepted by the AP, the AP shall send subsequent group addressed MSDUs that match the frame classifier specified in the DMS Descriptors as an individually addressed frame to the requesting STA in an A-MSDU frame format (see 7.4a and 9.2c). The AP shall still transmit the matching frames as group addressed frames (see 9.2.7.1 and 9.2.7.2) if at least one associated STA within the multicast group has not requested DMS for these frames.

The non-AP STA shall keep a list of group addresses for which the non-AP STA has requested DMS and that have been accepted by the AP. The requesting STA shall discard group addressed frames that match a group address from this list.

A non-AP STA may request removal of the requested DMS by sending a DMS Request frame with the Request Type set to “Remove” and the requested DMSID. The length field is set to 0. The TLCAS Elements, and TCLAS Processing Element fields shall not be included in the DMS Descriptor if the Request Type is set to “Remove”. The AP shall not send any DMS Response frame if the Request Type in the DMS Request frame is set to “Remove”. The AP shall terminate individually addressed frame delivery for the requested group addressed frames identified by the DMSID for the requesting non-AP STA upon receipt of a DMS Request frame with the Request Type field set to “Remove”.

The AP may send an unsolicited DMS Response frame at any time to cancel a granted DMS identified by the DMSID by including the DMSID and a status value of “Terminate” in the DMS Status.

Insert the following rows to A.4.14

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM19 | DMS | 7.1.3.5, 7.2.2.2, 7.4.11.23;7.4.11.24;11.20.19; | (CFv AND CF16):O | Yes, No, N/A |

# Annex D

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=
SEQUENCE {
	........

dot11DMSMAXSTAS       INTEGER}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11DMSMAXSTAS OBJECT-TYPE

SYNTAX SIGNED INTEGER (-2,147,483,648 to 2,147,483,647)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the maximum number of STAs which can be granted for Directed Multicast Service (DMS).”

DEFVAL { 0 }

::= { dot11StationConfigEntry 63}

Insert the following entries at the end of the Dot11WirelessMgmtOptionsEntry sequence list:

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionPresenceEnabledTruthValue,

dot11MgmtOptionFBMSImplementedTruthValue,

dot11MgmtOptionFBMSEnabledTruthValue,

dot11MgmtOptionEventsEnabledTruthValue,

dot11MgmtOptionDiagnosticsEnabledTruthValue,

dot11MgmtOptionMultiBSSIDImplementedTruthValue,

dot11MgmtOptionMultiBSSIDEnabledTruthValue,

dot11MgmtOptionTFSImplementedTruthValue,

dot11MgmtOptionTFSEnabledTruthValue, dot11MgmtOptionSleepModeImplementedTruthValue,

dot11MgmtOptionSleepModeEnabled TruthValue,

dot11MgmtOptionTIMBroadcastImplemented	 TruthValue,

dot11MgmtOptionTIMBroadcastEnabled TruthValue,

dot11MgmtOptionProxyARPEnabled TruthValue,

dot11MgmtOptionBSSTransitionEnabled TruthValue,

dot11MgmtOptionDMSImplemented	TruthValue,

dot11MgmtOptionDMSEnabled	TruthValue}

On page 155, as part of Dot11WirelessMgmtOptionsEntry, add the followingMIB objects:

dot11MgmtOptionDMSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting DMS when the dot11WirelessManagementImplemented is TRUE.”

DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionDMSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that DMS is enabled. DMS is disabled otherwise.”

DEFVAL { FALSE }
        ::= { dot11WirelessMgmtOptionsEntry 17 }