IEEE P802.11
Wireless LANs

| LB115 Explicit TxBF Resolutions- Part I |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2008-01-10 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Company | Address | Phone | email |  |
| Joonsuk Kim | Broadcom Corporation. |  |  | joonsuk@broadcom.com |  |

Abstract

This document proposes a resolution of Letter Ballot 115 comments in Explicit TxBF tab in Beam adhoc.

The changes marked in this document are based on TGn Draft 3.02

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# CID 5482, 5483

| 5482 | 50.36 | 7.3.1.31 |  |  | There are several references to "V matrix" or "Matrix V" in this section. V matrix is never really defined. More importantly, the use of the term implies that SVD is used to determine the matrix. This is not required in the draft, nor is it necessary that the matrix even be unitary (and they may be good reasons for it to possibly not be unitary). | Change the term from "V matrix" to "Non compressed beamforming matrix" or if a shorter term is required "NCB matrix". Explictly define the term in the definitions, and make it clear that this matrix need not be unitary |
| --- | --- | --- | --- | --- | --- | --- |
| 5483 | 52.26 | 7.3.1.32 |  |  | There are several references to "V matrix" or "Matrix V" in this section. V matrix is never really defined. More importantly, the use of the term implies that SVD is used to determine the matrix. This is not required in the draft. Unlike the non-compressed section, this matrix must be unitary for the Givens rotaiton to work properly. | Change the term from "V matrix" to "compressed beamforming matrix" or if a shorter term is required "CB matrix". Explicitly define the term in the definitions and make it clear that this matrix is unitary. |

Proposed resolution:  Counter

The beamforming matrix V is the matrix to be fed back for explicit feedback, while the steering matrix Q is the matrix the beamformer applies to enhance the reception for the beamformee. The relationship between “steering matrix Q” and “beamforming matrix V” is found in 20.3.11.2.  The matrix V is not necessarily found by SVD but by any decomposition the beamformee may choose to use.

We also agree that the definition of V matrix is not shown when it appears in 7.3.1.31 for the first time. So, we propose to change some of sentences as follows:

TGn editor: In Page 50 line 3-7, change the paragraph to “The Non-compressed Beamforming Report field is used by the Non-compressed Beamforming frame to carry explicit feedback in the form of non-compressed beamforming matrices V for use by a transmit beamformer to determine steering matrices Q, as described in 9.17.3 and 20.3.11.2 (Explicit feedback beamforming).”

In Page 52 line 7-12, change the paragraph to “The Compressed Beamforming Report field is used by the Compressed Beamforming frame (see 7.4.8.8(Compressed Beamforming frame format)) to carry explicit feedback information in the form of angles to represent compressed beamforming matrices V to a transmit beamformer for use by a transmit beamformer to determine steering matrices Q, as described in in 9.17.3 and 20.3.11.2 (Explicit feedback beamforming).”

# CID 5484

| 5484 | 50.36 | 7.3.1.31 |  |  | It is nowhere defined what the rows and columns of the V matrix represent, they are just N_c and N_r. There is a tradition in the literature to construct the matrices a specific way, but this is a standard and we should be clear and not rely on well-known but unreferenced convention. My impression is that this vagueness was a compromise to avoid arguments, but in a standard we must try for clarity sufficient to enable interoperability and not leave important details unspecified. | Clarify that the rows corrspond to active transmit chains and the columns correspond to space-time streams (unless I got that backwards). |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:  Counter

Nc and Nr are clearly defined as “number of columns” and “number of rows” of beamforming feedback matrix V, respectively in 7.3.1.31. In addition, with Table 7-25e, Nc represents the number of space-time streams. Therefore, we don’t believe we need to clarify this again in the draft. However, we’d better change some words in Table 7-25e,f,i and j

TGn editor: Replace “stream” with “space-time stream” in <field> and <meaning> within Table 7-25e (Page 50), Table 7-25f (Page 51), Table 7-25i (Page 53) and Table 7-25j (Page 54).