IEEE P802.11
Wireless LANs

| Roaming Definition Text Changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Colin Blanchard | BT Group | BT Design MLB1 PP8 Adastral Park Ipswich IP3 5RE | +44 01473 605353 | Colin.blanchard@bt.com |
| Matthew Gast | Trapeze Networks | 5753 W. Las Positas Blvd Pleasanton, CA 94588 USA | +1 925 474 2273 | msg@trapezenetworks.com |

Abstract

LB#107 comments 659, 660, 723, 1020 and 1697 have requested an improved definition of “roaming”

3.  Definitions

Make the following changes to the definition of roaming:

3.u.15 SSP Roaming: The act of a wireless terminal obtaining inteworking service from a different provider SSP than the one SSP to which it has its subscription.  For example, in the case of the SSP being a GSM operator, this is implemented using the existing Visited Network/Home Network arrangements (SSP roaming agreements)

5.9.  Generic Advertisement Service

Make the following changes to section the third dash point in this subclause:

- The non-AP STA can discover information about APs that are not part of the same administrative group as the AP with which it is associated. For example, supporting the selection of an AP belonging to a different 802.11 ANthat has an appropriate SSP roaming agreement in place

7.3.1.7.  Reason Code Field

Make the following changes to Table 7-22:

| Reason Code | Meaning |
| --- | --- |
| 47 | Disassociated because lack of SSP roaming agreement to SSPN |

References:

802.11u-D1.02