IEEE P802.11
Wireless LANs

| Normative Text for Mesh QoS Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Alexander Safonov | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |

Abstract

This document is a normative text proposal to provide an end-to-end QoS control mechanism in a mesh network. Changes are shown with respect to TGs draft D1.07.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.1.3.5 as shown:

- 7.1.3.5 QoS Control field

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 4.

Table 4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS data frames sent by non-AP | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
| QoS Data frames sent by Mesh Point | TID | EOSP | Ack Policy | A-MSDU Present | MP PS Buffer State |
| QoS Null frames sent by Mesh Point | TID | EOSP | Ack Policy | Reserved | MP PS Buffer State |

Insert the following after 7.1.3.5.7 as shown:

- 7.1.3.5.8 MP PS Buffer State subfield

The MP PS Buffer State subfield, defined in Figure zXX, is an 8-bit field that indicates the PS buffer state at the MP. The MP PS Buffer State subfield is further subdivided into three subfields: Buffer State Indicated, Highest-Priority Buffered AC, and QoS MP Buffered Load.

|  | B8 | B9 | B10 B11 | B12 B15 |
| --- | --- | --- | --- | --- |
|  | Reserved | Buffer State Indicated | Highest-Priority Buffered AC | QoS MP Buffered Load |
| Bit: | 1 | 1 | 2 | 4 |

Figure zXX—MP PS Buffer State subfield

The Buffered State Indicated subfield is 1 bit in length and is used to indicate whether the MP PS buffer state is specified. A value of 1 indicates that the MP PS buffer state is specified.

The Highest-Priority Buffered AC subfield is 2 bits in length and is used to indicate the AC of the highest priority traffic remaining that is buffered at the MP, excluding the MSDU of the present frame.

The QoS MP Buffered Load subfield is 4 bits in length and is used to indicate the total buffer size, rounded up to the nearest multiple of 4096 octets and expressed in units of 4096 octets, of all MSDUs buffered at the QoS MP (excluding the MSDU of the present QoS data frame). An QoS MP Buffered Load field value of 15 indicates that the buffer size is greater than 57 344 octets. An QoS MP Buffered Load subfield value of 0 is used solely to indicate the absence of any buffered traffic for the indicated highest priority buffered AC when the Buffer State Indicated bit is 1.

When the Buffered State Indicated subfield is set to 0, the Highest-Priority Buffered AC subfield and the QoS MP  Buffered Load subfield are reserved; and the values of these subfields are either unspecified or unknown.

Change 7.1.3.5a.2 as shown:

7.1.3.5a.2 Mesh Flags field

The Mesh Flags field, shown in Figure s5, is 8 bits in length and the flags therein are used to control mesh-specific header processing, e.g., for mesh address extension.

| B0 B1 | B2 B5 | B2 B6 B7 |
| --- | --- | --- |
| Address Extension (AE) Mode | Mesh TID | Reserved |
| Bits: 2 | Bits: 4 | 6 2 |
| Figure s5 - Mesh Flags field |  |  |

The Address Extension (AE) Mode field is used to indicate the contents of the Mesh Address Extension field. Table s2 defines valid values for the Address Extension Mode and describes the corresponding contents of the Mesh Address Extension field (see 11A.5.5 for the usage of the optional addresses contained in the Mesh Address Extension field).  If the Address Extension Mode is set to 00, the Mesh Address Extension field is not present.  For all other values, the Mesh Address Extension field follows the Mesh Sequence Number field.

The Mesh TID field identifies the end-to-end TC or end-to-end TS to which the corresponding Mesh Data, or fragment thereof, in the Frame Body field belongs. Mesh TID may be encoded in the source MP and it shall not be changed by the intermediate MPs. The intermediate MP decides the TID in the QoS Control field based on the Mesh TID. Combined with Mesh Sequence Number,

The reserved bits are set to zero.

Table s2 – Valid values for the Address Extension Mode

| Address Extension Mode value (binary) | Address Extension Mode description | Mesh Address Extension field length (octets) | Applicable frame types |
| --- | --- | --- | --- |
| 00 | No Mesh Address Extension field | 0 | Data |
| 01 | Mesh Address Extension field contains Addr4 | 6 | Management (Multi-hop Action) |
| 10 | Mesh Address Extension field contains Addr5 and Addr6 | 12 | Data |
| 11 | Mesh Address Extension field contains Addr4, Addr5, and Addr6 | 18 | Management (Multi-hop Action) |