IEEE P802.11
Wireless LANs

| Normative Text for Peer Power Save Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Alexander Safonov | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |

Abstract

This document is a normative text proposal for TDLS Peer Power Save Mode. The text is aligned with P802.11z-D0.01.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 3. Definitions

Insert the following new definitions so as to maintain alphabetical order:

3.z.2 Tunneled Direct Link Setup Peer Power Save Mode (TDLS PPSM): A power save mode which can be used by STAs supporting TLDS protocol.

- 4. Abbreviations and acronyms

Insert the following new acronyms in alphabetical order:

TDLS PPSM		Tunneled Direct Link Setup Peer Power Save Mode

Change 7.1.3.1.6 as shown:

- 7.1.3.1.6 Power Management field

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

A value of 1 indicates that the STA will be in PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

When a STA receives a frame with the Power Management field set to 1 from the peer STA through the direct link, it indicates that the peer STA will be in TDLS PPSM. For delivering buffered frames to peer STA in TDLS PPSM, the STA shall be in active mode (See 11.z1).

Change 7.1.3.5 as shown:

- 7.1.3.5 QoS Control field

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 4.

Table 4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS data frames sent by non-AP | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent over the direct link | TID | EOSP | Ack Policy | Reserved | Peer PS Buffer State |

Insert the following after 7.1.3.5.7 as shown:

- 7.1.3.5.8 Peer PS Buffer State subfield

The Peer PS Buffer State subfield, defined in Figure zXX, is an 8-bit field that indicates the PS buffer state at the peer buffering for a STA in peer power save mode. The Peer PS Buffer State subfield is further subdivided into three subfields: Buffer State Indicated, Highest-Priority Buffered AC, and STA Buffered Load.

|  | B8 | B9 | B10 B11 | B12 B15 |
| --- | --- | --- | --- | --- |
|  | Reserved | Buffer State Indicated | Highest-Priority Buffered AC | QoS STA Buffered Load |
| Bit: | 1 | 1 | 2 | 4 |

Figure zXX—STA PS Buffer State subfield

The Buffered State Indicated subfield is 1 bit in length and is used to indicate whether the STA PS buffer state is specified. A value of 1 indicates that the STA PS buffer state is specified.

The Highest-Priority Buffered AC subfield is 2 bits in length and is used to indicate the AC of the highest priority traffic remaining that is buffered at the STA, excluding the MSDU of the present frame.

The STA Buffered Load subfield is 4 bits in length and is used to indicate the total buffer size, rounded up to the nearest multiple of 4096 octets and expressed in units of 4096 octets, of all MSDUs buffered at the QoS STA (excluding the MSDU of the present QoS data frame). An STA Buffered Load field value of 15 indicates that the buffer size is greater than 57 344 octets. An STA Buffered Load subfield value of 0 is used solely to indicate the absence of any buffered traffic for the indicated highest priority buffered AC when the Buffer State Indicated bit is 1.

When the Buffered State Indicated subfield is set to 0, the Highest-Priority Buffered AC subfield and the STA  Buffered Load subfield are reserved; and the values of these subfields are either unspecified or unknown.

Change 7.2.2.1 as shown:

- 7.2.2.1 TDLS frame format

The frame body of a TDLS frame is defined in Table z1.

|  | LLC/SNAP | Remote Frame Type | Protocol Version | TDLS Packet Type | Information |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 1 | 1 | 1 | variable |

Figure z1—TDLS frame body

The LLC/SNAP field contains the LLC/SNAP header, with Ethertype 89-0d.

The Remote Frame Type field shall be set to 2 (TDLS).

The Protocol Version field shall be set to 1. Received messages with Protocol Version other than 1 shall be discarded.

The TDLS Packet Type field specifies the type of the TDLS frame, as specified in Table z1.

The Information field contains information which is specified for each TDLS Type individually.

Table z1—TDLS Packet Type values

| TDLS Type Value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown Request |
| 4 | TDLS Teardown Response |
| 5 | TDLS Tx Path Switch Request |
| 6 | TDLS Tx Path Switch Response |
| 7 | TDLS Rx Path Switch Request |
| 8 | TDLS Rx Path Switch Response |
| 9 | TDLS TIM |
| 9 – 255 | Reserved |

Change 7.2.2.1.1 as shown:

- 7.2.2.1.1 TDLS Setup Request

The TDLS Setup Request frame contains the information shown in Table z2.

Table z2—Information for TDLS Setup Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Association Request frame body | The Association Request frame body is specified in 7.2.3.4. |
| 3 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 4 | RSNIE_I | RSNIE for Initiator STA (optional) |
| 5 | SMK Message 1 FTIE | SMK Message 1 FTIE (optional) |
| 6 | AID | AID is copied from Association Response frame body specified in 7.2.3.5 |
| 7 | QoS Capability | The QoS Capability element is present when dot11Qos-OptionImplemented is true. |

The TDLS Setup Request frame shall be sent through the AP.

Change 7.2.2.1.2 as shown:

- 7.2.2.1.2 TDLS Setup Response

The TDLS Setup Response frame contains the information shown in Table z3.

Table z3—Information for TDLS Setup Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Status Code | The Status Code is defined in 7.3.1.9. |
| 3 | Association Request frame body | The Association Request frame body is specified in 7.2.3.4. Only present for Status Code 0 (Successful). |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Request. |
| 5 | RSNIE_P | RSNIE of Peer STA (optional) |
| 6 | SMK Message 2 FTIE | SMK Message 2 FTIE (optional) |
| 7 | AID | AID is copied from Association Response frame body specified in 7.2.3.5 |
| 8 | QoS Capability | The QoS Capability element is present when dot11Qos-OptionImplemented is true. |

The TDLS Setup Response frame shall be sent through the AP.

Insert the following after 7.2.2.1.9:

- 7.2.2.1.10 TDLS TIM

The TDLS TIM frame contains the information shown in Table z10.

Table z10—TDLS TIM frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1 | TIM |  |

The TIM Element field contains a TIM element starting with the Element ID. The AID 0 bit is set to 0.

Change 7.3.2.35 as shown:

- 7.3.2.35 QoS Capability element

The QoS Capability element contains a number of subfields that are used to advertise optional QoS capabilities at a QoS STA. The QoS Capability element is present in Beacon frames that do not contain the EDCA Parameter Set element and in (Re)Association Request frames and in TDLS Setup Request/Response frames. The QoS Capability element is defined in Figure 112.

|  | Element ID (46) | Length (1) | QoS Info |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 112—QoS Capability element format

The QoS Info field is 1 octet in length and is defined in 7.3.1.17.

Insert the following after 11.2.1.11:

- 11.2.1.12 Power Management with TDLS

A STA in TDLS PPSM shall operate as follows to receive an MSDU or MMPDU from the peer STA in active mode:

a) If the STA in TDLS PPSM is initiating an unscheduled SP, it wakes up and transmits a trigger frame to the peer STA in active mode. When one or more ACs are not delivery-enabled, it may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the peer STA.

b) The STA in TDLS PPSM shall remain awake until it receives a QoS data frame addressed to it, with the EOSP subfield in the QoS Control field set to 1.

c) The STA in TDLS PPSM may send additional PS-Poll frames if More Data subfield is set to 1 in unicast data or management frames that do not belong to any deliver-enabled ACs. The STA in TDLS PPSM may send additional trigger frames if the More Data subfield is set to 1 in unicast data or management frames that belong to delivery-enabled ACs.

Change 11.z1 as shown:

- 11.z1 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by the fact that the signaling frames are encapsulated in Data frames, which allows them to be transmitted through any access point transparently. Therefore, a direct link can be setup using any access point. The access point does not need to be direct link aware, nor does it have to support any of the capabilities which will be used on the direct link. TDLS also includes an option either to enter Peer Power Save Mode (PPSM) remaining on the direct link or to suspend receiving over the direct link, so that the station can enter a power save mode.

A STA may transmit a Link RCPI Measurement Request to an (intended) peer STA to obtain an indication of the RCPI values at the peer STA. The Link RCPI measurement request and report are sent to the peer STA directly. The RCPI information may be used to decide whether to switch over to a direct link for communication with the peer STA.

To setup a direct link, the initiator STA sends a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it responds with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it responds with a TDLS Setup Response with a status code other than 0. If there is no response within the set timeout, the initiator STA should conclude that the intended peer STA does not support TDLS and the setup procedure is terminated. The initiator then sends a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response. If security is required, the TDLS setup messages shall include the SMK handshake. When the TDLS Setup Handshake has been completed, both STAs shall accept frames received over the direct link.

After a successful response, the initiating STA further prepares the direct link for Data transmissions by starting the 4-way Handshake.

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a STA may send a TDLS Tx Path Switch Request indicating a switch to the direct path. The first Data frame transmitted over the direct link should be transmitted after receipt of the TDLS Tx Path Switch Response in this case. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange which is part of the direct link setup or the peer key handshake for this purpose.

When a STA intends to enter a power save state wants to suspend receiving over the direct link, it sends a TDLS Rx Path Switch Request to the peer STA, indicating a switch to the AP path. Upon receipt of a TDLS Rx Path Switch Request indicating a switch to the AP path, the receiving STA shall cease transmissions over the direct link as soon as possible. When no further Data frames will be transmitted over the direct link, the responding STA shall send a TDLS Rx Path Switch Response, echoing the switch to the AP path. The requesting STA may enter a power save mode after receiving the TDLS Rx Path Switch Response.

A STA may request a peer STA to enable the direct Rx path by sending a TDLS Tx Path Switch Request, indicating a switch to the direct path. Upon receipt of the TDLS Tx Path Switch Request indicating the direct path, the receiving STA shall enable the direct Rx path and respond with a TDLS Tx Path Switch Response echoing the switch to the direct path. The requesting station shall not transmit frames on the direct link before receiving the TDLS Tx Path Switch Response.

A STA that is enabling its direct Rx path may inform its peer STA by sending a TDLS Rx Path Switch Request indicating a switch to the direct path. The responding STA responds by sending a TDLS Rx Path Switch Response echoing the switch to the direct Rx path. The responding STA may then transmit over the direct path, but does not have to.

When a STA intends to enter power save mode, it shall either to suspend receiving over the direct link or enter TDLS PPSM with its peer remaining on the direct link with this peer. After that the STA may enter power save mode with the AP in ordinary way.

If a STA receives a frame with Power Management field set to 1 from the peer STA, it indicates that the peer STA will be in the TDLS PPSM. When the peer STA is in the TDLS PPSM, the STA should be in active mode for delivering buffered frames using the direct link.

If a STA in the TDLS PPSM receives a frame with Power Management field set to 1 from the peer STA, it indicates that the peer STA will be in the TDLS PPSM. So, after receiving a frame with Power Management field set to 1, the STA in TDLS PPSM should be in active mode because the peer STA will be in the TLDS PPSM.

The STA in TDLS PPSM may decide to wake up not only to receive Beacon frames and MSDUs buffered at the access point, but also at some arbitrary times. In these times, the STA may initiate the unscheduled SP by sending a trigger frame to its peer STA in active mode. After initiating the unscheduled SP, the STA retrieves the buffered MSDUs and MMPDUs using the power management operation in TDLS. (11.2.1.12)

In order to configure a peer STA in active mode to deliver frames during an unscheduled SP, the STA in TDLS PPSM designates one or more of its ACs to be delivery-enabled and one or more of its AC to be trigger-enabled. In order to configure a peer STA in active mode to use U-APSD in PPSM, a STA in TDLS PPSM may set individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in TDLS Setup Request and TDLS Setup Response frames. When a U-APSD Flag bit is set, it indicates that the corresponding AC is both delivery- and trigger-enabled.

The peer STA in active mode may assemble the partial virtual bitmap containing the buffer status for STAs in the PPSM and send it out in the TIM field of TDLS TIM broadcast frame. If STA in TDLS PPSM receives TDLS TIM frame, it must initiate the unscheduled SP for retrieving the buffered MSDUs and MMPDUs from the peer STA. The transmission interval of TDLS TIM frame is out of scope and it depends on the implementation issues such as the buffer capacity of STA.

To tear down a direct link, the STA sends a TDLS Teardown Request to the peer STA, after which the STA shall not transmit on the direct link any longer. Upon receipt of the TDLS Teardown Request, the peer STA shall disable the direct Rx and Tx paths and destroy the related security parameters, and then respond with a TDLS Teardown Response. Upon receipt of the TDLS Teardown Response, the STA which initiated the teardown shall disable the direct Rx path and destroy the related security parameters.

Insert the following new rows into A.4.z1:

### A.4.z1 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.2.2.1, 11.z1 | CFz:M | Yes, No. N/A |
| TDLS1.1 | TDLS Setup | 7.2.2.1.1, 7.2.2.1.2 | CFz:M | Yes, No. N/A |
| TDLS1.2 | TDLS Teardown | 7.2.2.1.3, 7.2.2.1.4 | CFz:M | Yes, No. N/A |
| TDLS1.3 | TDLS Tx Path Switch | 7.2.2.1.5, 7.2.2.1.6 | CFz:M | Yes, No. N/A |
| TDLS1.4 | TDLS Rx Path Switch | 7.2.2.1.7, 7.2.2.1.8 | CFz:M | Yes, No. N/A |
| TDLS1.5 | TDLS Peer Key Handshake | 8.5.9 | CFz:M | Yes, No. N/A |
| TDLS1.6 | Link RCPI Request/Report | 7.3.2.21.11, 7.3.2.22.11 | CFz:M | Yes, No. N/A |
| TDLS1.7 | TDLS Peer Power Save Mode | 7.2.2.1.10 | CFz:M | Yes, No. N/A |