IEEE P802.11
Wireless LANs

| Normative Text for Peer Power Save Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Alexander Safonov | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |
| Menzo Wentink |  | Oudegracht 3, Utrecht, the Netherlands | +31651836231 | menzow@hotmail.com |
| Henry Ptasinski | Broadcom Corporation | 190 Mathilda Place, Sunnyvale CA | +1-408-543-3316 | henryp@broadcom.com |

Abstract

This document is a normative text proposal for TDLS Peer Power Save Mode. The text is aligned with P802.11z-D0.01.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 3. Definitions

Insert the following new definitions so as to maintain alphabetical order:

3.z.2 Peer Power Save Mode (Peer PSM): A power save mode which can be used by STAs which support TLDS. There are two modes for Peer PSM operation, AP mode and client mode.

- 4. Abbreviations and acronyms

Insert the following new acronyms in alphabetical order:

Peer PSM		Peer Power Save Mode

Change 7.1.3.1.6 as shown:

- 7.1.3.1.6 Power Management field

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

A value of 1 indicates that the STA will be in (Peer) PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

Change 7.1.3.5 as shown:

- 7.1.3.5 QoS Control field

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 4.

Table 4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS data frames sent by non-AP to an AP | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent over the direct link | TID | EOSP | Ack Policy | Reserved | Reserved |

Change 7.2.2.1 as shown:

- 7.2.2.1 TDLS frame format

The frame body of a TDLS frame is defined in Table z1.

|  | LLC/SNAP | Remote Frame Type | Protocol Version | TDLS Packet Type | Information |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 1 | 1 | 1 | variable |

Figure z1—TDLS frame body

The LLC/SNAP field contains the LLC/SNAP header, with Ethertype 89-0d.

The Remote Frame Type field shall be set to 2 (TDLS).

The Protocol Version field shall be set to 1. Received messages with Protocol Version other than 1 shall be discarded.

The TDLS Packet Type field specifies the type of the TDLS frame, as specified in Table z1.

The Information field contains information which is specified for each TDLS Type individually.

Table z1—TDLS Packet Type values

| TDLS Type Value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown Request |
| 4 | TDLS Teardown Response |
| 5 | TDLS Tx Path Switch Request |
| 6 | TDLS Tx Path Switch Response |
| 7 | TDLS Rx Path Switch Request |
| 8 | TDLS Rx Path Switch Response |
| 9 | Peer Traffic Indication |
| 10 – 255 | Reserved |

Change 7.3.2.27 as shown:

- 7.3.2.27 Extended Capabilities information element

Add a row to Table 35a as follows and renumber the reserved values accordingly:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Peer PSM AP Mode | The Peer PSM AP mode capability bit set to 1 indicates that the STA supports Peer PSMP AP mode as described in 11.2.1.12. The Peer PSM AP mode capability bit set to 0 indicates that the STA does not support this capability. |

Insert a new section after 7.2.2.1.9:

7.2.2.1.10 Peer Traffic Indication

The Peer Traffic Indication frame contains the information shown in Table z10.

Table z10—Peer Traffic Indication

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | AC0 backlogged | 1 octet field which indicates the number of frames of AC0 |
| 3 | AC1 backlogged | 1 octet field which indicates the number of frames of AC1 |
| 4 | AC2 backlogged | 1 octet field which indicates the number of frames of AC2 |
| 5 | AC3 backlogged | 1 octet field which indicates the number of frames of AC3 |
| 6 | Peer PSM Indication Window | 2 octets field which indicates the minimum interval between successive Peer Traffic Indication frames, expressed in TUs. (Optional) |

ThePeer Traffic Indication frame indicates the power save buffer state at the peer STA buffering data for a STA in Peer PSM.

Each of the ACs backlogged field is 1 octet in length and set to the number of frames of the corresponding AC (AC0, AC1, AC2, AC3) that are buffered at the STA. An AC backlogged field value of 255 indicates that the number of buffered frames is greater than 255.

Insert the following after 11.2.1.11:

- 11.2.1.12 Power Management with TDLS

When a peer of a STA supports the Peer PSM AP mode, a STA supporting the Peer PSM client mode can enter the Peer PSM by sending a frame with the power management bit set. Such a STA is called a STA in Peer PSM client mode.

When a peer of a STA is in the Peer PSM client mode, a STA shall buffer MSDUs for the peer locally without sending them to the access point and deliver the buffered MSDUs over the direct link. Such a STA is called a STA in Peer PSM AP mode.

The behavior of a STA in Peer PSM AP mode with respect to traffic buffered for a peer STA in client mode is the same as a regular U-APSD AP, with the following exception. When an AC becomes backlogged and no Service Period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in AP mode sends a unicast Peer Traffic Indication frame to the peer STA in client mode, indicating the backlogged AC(s).

The behavior of a STA in Peer PSM client mode with respect to traffic buffered at the peer STA in AP mode is the same as a regular U-APSD STA, with the following exception. The indication that traffic is buffered at the peer STA in AP mode is obtained from received Peer Traffic Indication frames from the peer STA in AP mode. To reduce the number of Peer Traffic Indication frames transmitted for a continuous uni-directional traffic stream without return traffic, a new Service Period may be started within Peer PSM Indication Window after the occurrence of a Service Interval during which a Data MPDU was received.

The Peer PSM Indication Window value is optionally present in the Peer Traffic Indication frame. The default Peer PSM Indication Window is 200 TUs.

When a peer of a STA is in the Peer PSM client mode, the STA may choose either to be in the Peer PSM AP mode or to switch to the Peer PSM client mode by sending to its peer STA a frame with the power management bit set during a SP initiated by the peer STA. (The SP may start after the frame is indicated by Peer Traffic Indication frame).

A STA in Peer PSM client mode transmitting a Peer Traffic Indication frame shall switch to the Peer PSM AP mode.

Change 11.z1 as shown:

- 11.z1 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by the fact that the signaling frames are encapsulated in Data frames, which allows them to be transmitted through any access point transparently. Therefore, a direct link can be setup using any access point. The access point does not need to be direct link aware, nor does it have to support any of the capabilities which will be used on the direct link. TDLS also includes an option either to enter Peer Power Save Mode (Peer PSM) remaining on the direct link or to suspend receiving over the direct link, so that the station can enter a power save mode.

A STA may transmit a Link RCPI Measurement Request to an (intended) peer STA to obtain an indication of the RCPI values at the peer STA. The Link RCPI measurement request and report are sent to the peer STA directly. The RCPI information may be used to decide whether to switch over to a direct link for communication with the peer STA.

To setup a direct link, the initiator STA sends a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it responds with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it responds with a TDLS Setup Response with a status code other than 0. If there is no response within the set timeout, the initiator STA should conclude that the intended peer STA does not support TDLS and the setup procedure is terminated. The initiator then sends a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response. If security is required, the TDLS setup messages shall include the SMK handshake. When the TDLS Setup Handshake has been completed, both STAs shall accept frames received over the direct link.

After a successful response, the initiating STA further prepares the direct link for Data transmissions by starting the 4-way Handshake.

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a STA may send a TDLS Tx Path Switch Request indicating a switch to the direct path. The first Data frame transmitted over the direct link should be transmitted after receipt of the TDLS Tx Path Switch Response in this case. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange which is part of the direct link setup or the peer key handshake for this purpose.

When a STA intends to enter a power save state wants to suspend receiving over the direct link, it sends a TDLS Rx Path Switch Request to the peer STA, indicating a switch to the AP path. Upon receipt of a TDLS Rx Path Switch Request indicating a switch to the AP path, the receiving STA shall cease transmissions over the direct link as soon as possible. When no further Data frames will be transmitted over the direct link, the responding STA shall send a TDLS Rx Path Switch Response, echoing the switch to the AP path. The requesting STA may enter a power save mode after receiving the TDLS Rx Path Switch Response.

A STA may request a peer STA to enable the direct Rx path by sending a TDLS Tx Path Switch Request, indicating a switch to the direct path. Upon receipt of the TDLS Tx Path Switch Request indicating the direct path, the receiving STA shall enable the direct Rx path and respond with a TDLS Tx Path Switch Response echoing the switch to the direct path. The requesting station shall not transmit frames on the direct link before receiving the TDLS Tx Path Switch Response.

A STA that is enabling its direct Rx path may inform its peer STA by sending a TDLS Rx Path Switch Request indicating a switch to the direct path. The responding STA responds by sending a TDLS Rx Path Switch Response echoing the switch to the direct Rx path. The responding STA may then transmit over the direct path, but does not have to.

When a STA intends to enter a power save mode, it may suspend the direct link by shutting down the direct Rx path, or enter the Peer PSM by sending a frame with the power management bit set. See 11.2.1.12.

To tear down a direct link, the STA sends a TDLS Teardown Request to the peer STA, after which the STA shall not transmit on the direct link any longer. Upon receipt of the TDLS Teardown Request, the peer STA shall disable the direct Rx and Tx paths and destroy the related security parameters, and then respond with a TDLS Teardown Response. Upon receipt of the TDLS Teardown Response, the STA which initiated the teardown shall disable the direct Rx path and destroy the related security parameters.

Insert the following new rows into A.4.z1:

### A.4.z1 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.2.2.1, 11.z1 | CFz:M | Yes, No. N/A |
| TDLS1.1 | TDLS Setup | 7.2.2.1.1, 7.2.2.1.2 | CFz:M | Yes, No. N/A |
| TDLS1.2 | TDLS Teardown | 7.2.2.1.3, 7.2.2.1.4 | CFz:M | Yes, No. N/A |
| TDLS1.3 | TDLS Tx Path Switch | 7.2.2.1.5, 7.2.2.1.6 | CFz:M | Yes, No. N/A |
| TDLS1.4 | TDLS Rx Path Switch | 7.2.2.1.7, 7.2.2.1.8 | CFz:M | Yes, No. N/A |
| TDLS1.5 | TDLS Peer Key Handshake | 8.5.9 | CFz:M | Yes, No. N/A |
| TDLS1.6 | Link RCPI Request/Report | 7.3.2.21.11, 7.3.2.22.11 | CFz:M | Yes, No. N/A |
| TDLS1.7 | Peer PSM (AP mode) | 7.2.2.1.10 | CFz:O | Yes, No. N/A |
| TDLS1.8 | Peer PSM (client mode) | 7.2.2.1.10 | CFz:O | Yes, No. N/A |

Annex D

Insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry::=

SEQUENCE {

dot11PeerPSMAPModeImplemented TruthValue,

dot11PeerPSMClientModeImplemented TruthValue,

dot11PeerPSMIndicationWindow INTEGER}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11PeerPSMAPModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM AP mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMClientModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM client mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMIndicationWindow OBJECT-TYPE

SYNTAX INTEGER (1...65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the minimum interval in TUs

between successive Peer Traffic Indication frames.

This value is transmitted in the Peer Traffic Indication frame.

The default value of this attribute is 200 TUs.”

::= { dot11StationConfigEntry <ANA>}