IEEE P802.11
Wireless LANs

| Normative text for Adding EBR to ADDTS Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-1-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

This submission provides text to add the Expedited Bandwidth Request element to TS Management MLME primitive.  It addresses the following CIDs: 1926.

This submission is based on IEEE Draft 802.11u-d1.02 and IEEE 802.11-2007.

4 Abbreviations

Editing instructions: add the following abbreviation to the list, alphabetitizing as appropriates:

EBR	Expedited Bandwidth Request

10 Layer Management

10.3 MLME SAP interface

10.3.24 TS Management Interface

10.3.24.1 MLME-ADDTS.request

10.3.24.1.2 Semantics of the service primitive

Editor: change the attribute list and and insert text at the appropriate row in associated table:

MLME-ADDTS.request	(

DialogToken,

TSPEC,

TCLAS,

TCLASProcessing,

EBR,

ADDTSFailureTimeout,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EBR (optional) | As defined in frame format | As defined in frame format | Specifies the precedence level of the TS request. |

10.3.24.2 MLME-ADDTS.confirm

10.3.24.2.2 Semantics of the service primitive

Editor: change the attribute list and and insert text at the appropriate row in associated table:

MLME-ADDTS.confirm	(

ResultCode,

DialogToken,

TSDelay,

TSPEC,

Schedule,

TCLAS,

TCLASProcessing,

EBR

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EBR (optional) | As defined in frame format | As defined in frame format | Specifies the precedence level of the TS request. |

10.3.24.3 MLME-ADDTS.indication

10.3.24.3.2 Semantics of the service primitive

Editor: change the attribute list and and insert text at the appropriate row in associated table:

MLME-ADDTS.indication (

DialogToken,

Non-APSTAAddress

TSPEC,

TCLAS,

TCLASProcessing,

EBR

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EBR (optional) | As defined in frame format | As defined in frame format | Specifies the precedence level of the TS request. |

10.3.24.4 MLME-ADDTS.response

10.3.24.4.2 Semantics of the service primitive

Editor: change the attribute list and and insert text at the appropriate row in associated table:

MLME-ADDTS.response (

ResultCode,

DialogToken,

Non-APSTAAddress,

TSDelay,

TSPEC,

Schedule,

TCLAS,

TCLASProcessing,

EBR

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| EBR (optional) | As defined in frame format | As defined in frame format | Specifies the precedence level of the TS request. |

11 MLME

11.4 TS management

11.4.1 Introduction

Editing instructions: amend the following text from IEEE 802.11-2007 as indicated:

A TSPEC describes the traffic characteristics and the QoS requirements of a TS. The main purpose of the TSPEC is to reserve resources within the HC and modify the HC’s scheduling behavior. It also allows other parameters to be specified that are associated with the TS, such as a traffic classifier and acknowledgment policy.

A TS may have one or more TCLAS (within the discretion of the STA that sets up the stream) associated with it. The AP uses the parameters in the TCLAS elements to filter the MSDUs belonging to this TS so that they can be delivered with the QoS parameters that have been set up for the TS.

A TS may have zero or one Expedited Bandwidth Request (EBR) element associated with it.  An AP uses the parameters in the EBR to understand the precedence level requested by a non-AP STA (see Annex T.1.3).  For example, the precedence level may be used to convey to the AP that the requested TS is for the purposes of placing an emergency call.

TSPEC, and the optional TCLAS elements and the optional EBR element are transported on the air by the ADDTS, in the corresponding QoS Action frame and across the MLME SAP by the MLME-ADDTS primitives.