IEEE P802.11
Wireless LANs

| Key Hash and MIC Extensions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 408-457- | dharkins@arubanetworks.com |
| Jouni Malinen | Devicescape Software, Inc. | 900 Cherry Ave San Bruno, CA 94066 | +1 650-829-2600 | jkm@devicescape.com |
| Henry Ptasinski | Broadcom Corporation |  |  | henryp@broadcom.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Jesse Walker | Intel Corporation | 2111 NE 25th Avenue JF2-306 Hillsboro, OR USA 97124 | +1 503-712-1849 | Jesse.walker@intel.com |

Abstract

This document contains TGw normative text changes to support use of SHA256 based key derivation in the 4-Way Handshake.

.

Background: IEEE Std 802.11-2007 uses the SHA1 algorithm for key derivation. The changes described here enable SHA2-256 to be used in key derivation. Given the success of recent attacks against the collision resistance of the MD hashing family, it is not known whether HMAC-MD5 and HMAC-SHA1 will resist attack against the PRF function. Since the express purpose of P802.11w is to extend 802.11i (now IEEE Std 802.11-2007) protections to management frames, this uncertainty will extend to 802.11w unless action is taken. We introduce a new PRF based on SHA-256 to remove this uncertainty in 802.11w. By design, the PRF automatically extends to remove the uncertainty from the 802.11i mechanisms as well.

Change Table 7-34 in 7.3.2.25.2 as follows:

Table 7-34 – Authentication and Key Management Suite Selectors

| OUI |  | Suite Type |  | Meaning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Authentication Type |  | Key Management Type |  |
| 00-0F-AC |  | 0 |  | Reserved |  | Reserved |
| 00-0F-AC |  | 1 |  | Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2 – RSNA default |  | RSNA Key Management as defined in 8.5 or using PMKSA caching as defined in 8.4.6.2 – RSNA default |
| 00-0F-AC |  | 2 |  | PSK |  | RSNA Key Management as defined in 8.5, using PSK |
| 00-0F-AC |  | 3 |  | Fast BSS Transition Authentication negotiated over IEEE 802.1X |  | Fast BSS Transition Key management as defined in 8.5.1.5 |
| 00-0F-AC |  | 4 |  | Fast BSS Transition Authentication using PSK |  | Fast BSS Transition Key management as defined in 8.5.1.5 |
| 00-0F-AC |  | 5 |  | Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2, with SHA256 Key Derivation |  | RSNA Key Management as defined in 8.5 or using PMKSA caching as defined in 8.4.6.2, with SHA256 Key Derivation |
| 00-0F-AC |  | 6 |  | PSK, with SHA256 Key Derivation |  | RSNA Key Management as defined in 8.5, using PSK, with SHA256 Key Derivation |
| Other |  | 7-255 |  | Reserved |  | Reserved |

Insert the following text at the end of 8.5.1.1:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6, HMAC-SHA-256 is used in the PRF. A is a unique label for each different purpose of the PRF; Y is a single octet containing 0; X is a single octet containing the parameter; and || denotes concatenation:

H-SHA-256(K, A, B, X) HMAC-SHA-256(K, A || Y || B || X)

PRF(K, A, B, Len)

for i 0 to (Len+255)/256 do

R R || H-SHA-256(K, A, B, i)

return L(R, 0, Len)

PRF-128(K, A, B) = PRF(K, A, B, 128)

PRF-192(K, A, B) = PRF(K, A, B, 192)

PRF-256(K, A, B) = PRF(K, A, B, 256)

PRF-384(K, A, B) = PRF(K, A, B, 384)

PRF-512(K, A, B) = PRF(K, A, B, 512)

Insert the following text at the end of 8.5.1.2:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6, HMAC-SHA-256 is used to calculate the PMKID, and the PMK identifier is defined as

PMKID = Truncate-128(HMAC-SHA-256(PMK, "PMK Name" || AA || SPA).

Insert the following text at the end of 8.5.1.4:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6, HMAC-SHA-256 is used to calculate the SMKID, and an SMK identifier is defined as

SMKID = Truncate-128(HMAC-SHA-256(SMK, "SMK Name" || PNonce || MAC_P || INonce || MAC_I).

Change 8.5.2 Key Information field list item 1, iii as shown below:

iii) The value 3 shall be used for all EAPOL-Key frames to and from a STA when the negotiated AKM is 00-0F-AC:3,  00-0F-AC:4, 00-0F-AC:5, or 00-0F-AC:6. This value indicates the following:

—AES-128-CMAC is the EAPOL-Key MIC. AES-128-CMAC is defined by FIPS SP800-38B. The output of the AES-128-CMAC shall be 128 bits.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

References: