IEEE P802.11
Wireless LANs

| Key Hash and MIC Extensions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 408-457- | dharkins@arubanetworks.com |
| Jouni Malinen | Devicescape Software, Inc. | 900 Cherry Ave San Bruno, CA 94066 | +1 650-829-2600 | jkm@devicescape.com |
| Henry Ptasinski | Broadcom Corporation |  |  | henryp@broadcom.com |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Jesse Walker | Intel Corporation | 2111 NE 25th Avenue JF2-306 Hillsboro, OR USA 97124 | +1 503-712-1849 | Jesse.walker@intel.com |

Abstract

This document contains TGw normative text changes to support use of SHA256 based key derivation in the 4-Way Handshake.

.

Background: IEEE Std 802.11-2007 uses the SHA1 algorithm for key derivation. The changes described here enable SHA2-256 to be used in key derivation. Given the success of recent attacks against the collision resistance of the MD hashing family, it is not known whether HMAC-MD5 and HMAC-SHA1 will resist attack against the PRF function. Since the express purpose of P802.11w is to extend 802.11i (now IEEE Std 802.11-2007) protections to management frames, this uncertainty will extend to 802.11w unless action is taken. We introduce a new PRF based on SHA-256 to remove this uncertainty in 802.11w. By design, the PRF automatically extends to remove the uncertainty from the 802.11i mechanisms as well.

Change Table 7-34 in 7.3.2.25.2 as follows:

Table 7-34 – Authentication and Key Management Suite Selectors

| OUI |  | Suite Type |  | Meaning |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Authentication Type |  | Key Management Type |  |
| 00-0F-AC |  | 0 |  | Reserved |  | Reserved |
| 00-0F-AC |  | 1 |  | Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2 – RSNA default |  | RSNA Key Management as defined in 8.5 or using PMKSA caching as defined in 8.4.6.2 – RSNA default |
| 00-0F-AC |  | 2 |  | PSK |  | RSNA Key Management as defined in 8.5, using PSK |
| 00-0F-AC |  | 3 |  | Fast BSS Transition Authentication negotiated over IEEE 802.1X |  | Fast BSS Transition Key management as defined in 8.5.1.5 |
| 00-0F-AC |  | 4 |  | Fast BSS Transition Authentication using PSK |  | Fast BSS Transition Key management as defined in 8.5.1.5 |
| 00-0F-AC |  | 5 |  | Authentication negotiated over IEEE 802.1X or using PMKSA caching as defined in 8.4.6.2, with SHA256 Key Derivation |  | RSNA Key Management as defined in 8.5 or using PMKSA caching as defined in 8.4.6.2, with SHA256 Key Derivation |
| 00-0F-AC |  | 6 |  | PSK, with SHA256 Key Derivation |  | RSNA Key Management as defined in 8.5, using PSK, with SHA256 Key Derivation |
| Other |  | 7-255 |  | Reserved |  | Reserved |

In section 8.4.1.2.1 in the second bulleted item under the heading “A STA Roaming within an ESS…”change the second-to-last sentence to:

If none of the PMKIDs of the cached PMKSAs matches any of the supplied PMKIDs, or if the AKM of the cached PMKSA differs from that offered in the (Re)Association Request, or the PMK in the cached PMKSA is no longer valid, then the Authenticator shall perform another IEEE 802.1X authentication.

In the second paragraph of 8.4.6.2 replace the final two sentences with:

Upon receipt of a (Re)Association Request with one or more PMKIDs an AP checks whether its Authenticator has retained a PMK for the PMKIDs, whether the AKM in the cached PMKSA matches the AKM in the (Re)Association Request, and whether the PMK is still valid. If so, it shall asserts possession of that PMK by beginning the 4-Way Handshake after association has completed, otherwise it shall begins a full IEEE 802.1X authentication after association has completed.

Insert the following text at the end of 8.5.1.1:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6 the KDF used will be the KDF specified in section 8.5.1.5.2.

Insert the following text at the end of 8.5.1.2:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6, HMAC-SHA-256 is used to calculate the PMKID, and the PMK identifier is defined as

PMKID = Truncate-128(HMAC-SHA-256(PMK, "PMK Name" || AA || SPA).

Insert the following text at the end of 8.5.1.4:

When the negotiated AKM is 00-0F-AC: 5 or 00-0F-AC:6, HMAC-SHA-256 is used to calculate the SMKID, and an SMK identifier is defined as

SMKID = Truncate-128(HMAC-SHA-256(SMK, "SMK Name" || PNonce || MAC_P || INonce || MAC_I).

Change 8.5.2 Key Information field list item 1, iii as shown below:

iii) The value 3 shall be used for all EAPOL-Key frames to and from a STA when the negotiated AKM is 00-0F-AC:3,  00-0F-AC:4, 00-0F-AC:5, or 00-0F-AC:6. This value indicates the following:

—AES-128-CMAC is the EAPOL-Key MIC. AES-128-CMAC is defined by FIPS SP800-38B. The output of the AES-128-CMAC shall be 128 bits.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

Insert into PICS after item PC34.1.3.3:

Entries for AKM for 00-0F-AC: 5 and 00-0F-AC:6 both optional.

References: