IEEE P802.11
Wireless LANs

| MDA MIB variables |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Dee Denteneer | Philips | HTC 37 5656 AE Eindhoven | +31 40 27 49743 | Dee.denteneer@philips.com |
| Guido Hiertz | Philips | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |

Abstract

This document introduces the MIB variables for MDA and  potentially resolves CIDs 740, 741, 742, 1887, 1888, 1889, 2223, 2224, 2225, 3105, 3106, 3109, 3111, 3117, 3120, 4644, 4645, 4646

## Introduce the following text at the end of  Annex D, following the variable dot11shortMulticastFrameLengthLimit

dot11MAFlimit OBJECT-TYPE

SYNTAX INTEGER (0..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum MDA access fraction allowed at the MP. This number is always a multiple of (1/16)."

::= { dot11MeshPointConfigEntry 19}

dot11MDAdvertPeriodMax  OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the mimimum interval with which an MDA active MP advertises its MDAOP Advertisements information element.”

::= { dot11MeshPointConfigEntry 20}

dot11MDAOPtimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

" This object specifies the timeout value for an MDAOP teardown. It is expressed in TU units."

::= { dot11MeshPointConfigEntry 21}