IEEE P802.11
Wireless LANs

| Liaison Change Request from 3GPP SA1 to IEEE 802.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joerg Swetina | 3GPP SA1 |  |  |  |

3GPP TSG-SA WG1 Meeting #38 	S1-071913

- Ljubljana, Slovenia, 29th October - 2nd November 2007 (Email Approval)

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.278 | CR | 0015 |  rev | 3 |  Current version: | 8.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Enhancements to Access network discovery and steering of access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Siemens Networks, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S1 |  |  |  |  |  |  |  |  |  |
| Work item code:  | AIPN-SAE |  |  |  |  |  |  | Date:  |  | 15/11/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Since EPS allows support of non-3GPP access networks the requirements on the discovery of these non-3GPP access networks need to be enhanced. Also the PLMN should be able to request a UE to use non-3GPP RATs. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Requirements on access network discovery are clarified. A new section on steering of access has been added. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Insufficient requirements concerning non-3GPP access networks. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 7.1.5, new section 7.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | X |  | Other core specifications  |  |  | 23.402 |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  | Clarification of requirements had been requested by SA2 (LS S1-071227 = S2-073112) |  |  |  |  |  |  |  |  |

### 7.1.5 	Access network discovery

To avoid unnecessary background scan by the UE and to facilitate service continuity by the UE it shall be possible for the VPLMN and the HPLMN to provide the UE with access network information pertaining to locally supported non-3GPP access technologies. The mechanism for provisioning of this information shall allow for frequent occurrences of service continuity within and across 3GPP access systems and non 3GPP access systems. The access network information may also include operator preferences based on locally available 3GPP and non-3GPP access technologies. The information may be restricted to the access technologies, based e.g. on the UE’s current location and circumstances, or access networks the UE can use.

When discovering non-3GPP accesses a UE shall be able to receive information from a non-3GPP access network concerning to which PLMN, or PLMNs, the non-3GPP access network provides access.

Note: The capability to provide such information by a non-3GPP access network is out of scope of 3GPP.

When a UE receives service via a non-3GPP access it shall be possible for the PLMN that provides the non-3GPP access to indicate local availability of 3GPP access to the UE, subject to capabilities of the non-3GPP access network.

### 7.1.x	Steering of access

If a UE has access to the Evolved Packet Core via E-UTRA then the operator of the PLMN that provides the access may want to request the UE to use - any or a specific - non-3GPP RAT. Similarly, if a UE has access to the Evolved Packet Core via a non-3GPP RAT then the operator of the PLMN that provides the access may want to request the UE to use E-UTRA. The reason for such steering may be load balancing (for camped- and traffic load balancing), operator policy, private networks/home cells, service based mobility control etc.

Note: The PLMN operator may provide access to the Evolved Packet Core either through his own access network (E-UTRA or non-3GPP access) or in collaboration with an access network operator that operates a non-3GPP access network.

If a UE has access to the Evolved Packet Core via E-UTRA then:

- the PLMN that provides the access shall be able to request this UE to use any non-3GPP RAT rather than the current RAT for access to this PLMN.

- the PLMN that provides the access shall be able to request this UE to use a specific non-3GPP RAT for access to this PLMN.

Note: A specific non-3GPP RAT may e.g. be identified by RAT type and the access network name (as advertized by the access network), or a list of access network names.

If a UE has access to the Evolved Packet Core via a non-3GPP RAT then:

- the PLMN that provides the access shall be able to request this UE to use E-UTRA rather than the current RAT for access to this PLMN.

Upon request from the PLMN to use any or a specific non-3GPP RAT or to use E-UTRA rather than the current RAT the UE may comply to this request, depending on local availability of the new RAT and on HPLMN operator policies and user preferences.