IEEE P802.11
Wireless LANs

| LB108 Co-located Interference comment resolution for CIDs 591, 1277, 1745, 1746 and 1748 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jari Jokela | Nokia | Visiokatu 3, 33720 Tampere, Finland | +358504860445 | Jari.jokela@nokia.com |

Abstract

This document contains normative text proposal to address LB108 CIDs 591, 1277, 1745, 1746 and 1748.

This document is based on IEEE P802.11v-d1.03.

- Co-located Interference Reporting

Co-located interference may cause degradation of 802.11 STA performance either periodically or continuously. The Co-located Interference reporting allows a requesting STA to receive information concerning the co-located interference being experienced by another STA on the operating channel. Such interference may be due to an interaction between radios where a reporting STA is co-located with another radio device. Co-located Interference information can be used by the requesting STA to manage communication to the reporting STA such that the effect of the interference is limited.

A STA that supports Co-located Interference reporting shall set the Co-located Interference Reporting capability bit in the Extended Capabilities element to 1.

A requesting STA may request a Co-located Interference reporting from another STA by sending a Co-located Interference Request containing a unique Dialog Token. The STA, if it accepts the request, shall return a Co-located Interference Response frame that includes the Dialog Token that matches the one in the Co-located Interference Request frame.

Editing Instructions: Modify the following text as follows:

Alternatively, a requesting STA may request that automatic Co-located Interference reporting is enabled at other STAs that have indicated support for the interference diagnostics capability. To enable automatic Co-located Interference reporting, the STA shall send a Co-located Interference Request frame with Automatic Response Enabled bit set to 1. A change event could occur when the co-located interference is detected, when the level of co-located interference significantly changes, when the periodicity of co-located interference changes, or when the co-located interference is no longer present. The requesting STA can disable automatic reporting by sending a Co-located Interference Request frame with the Automatic Response Enabled field set to 0.

A STA accepting a request for automatic Co-located Interference reporting shall send a Co-located Interference Response frame to the requesting STA if it detects that a co-located radio or another co-located interferer is causing performance degradation to its WLAN receiver. The Dialog Token field is set to the nonzero value received in the Co-located Interference Request frame which was used to enable automatic responses. If co-located interference is present the reporting STA shall send Co-located Interference Response frames with interval indicated by the Report Period field or if the level of the interference caused by co-located interferer is significantly changing or if the time characteristics of the interference is significantly changing. The criteria a reporting STA uses for determining significant changes are internal to the reporting STA and outside the scope of this standard. The reporting STA shall not generate Interference Responses more frequently than indicated by the Report Timeout field in Interference Request if the level of the interference or the time characteristics of the interference do not change significantly. New Co-located Interference Request supersedes any previously received Co-located Interference Request sent by the same STA as the new Co-located Interference Request.

The reporting STA shall use the Interference Index field in Co-located Interference Response frame to indicate different types of interference. For example if the reporting STA is detecting co-located interference originated from two interference sources the reporting STA shall report both type of interferences using separate Response Info fields having separate Interference Index field. Both Response Info fields can be sent in the same Co-located Interference Response frame and both can have the same Report Period.

Automatic Co-located Interference reporting shall be terminated on receipt of a Co-located Interference Request frame with the Automatic Response bit set to 0.

A reporting STA may report any co-located interference that it is able to determine and that is causing degradation to its performance. The criteria a reporting STA uses for determining performance degradation are internal to the reporting STA and outside the scope of this standard. The methods used by a reporting STA to determine the periodicity, level of interference, accuracy of the reported interference level, interference center frequency and interference bandwidth are outside the scope of this standard.