IEEE P802.11
Wireless LANs

| Reference Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Stephen McCann | Nokia Siemens Networks | Roke Manor Research Ltd, Old Salisbury Lane, Romsey, Hampshire, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |

Abstract

This submission resolves CIDs:

WISPr	598, 842, 1107

NENA	840, 841, 1106

from 802.11 LB#107.

# Editing Instructions

Change the following subclause:

- Abbreviations and acronyms

Insert the following new acronym in alphabetical order:

3G	Third Generation

3GPP	3G Partnership Project

3GPP2	3G Partnership Project 2

AN	Access Network

DN	Destination Network

DS	Distribution System

DSS	Distribution System Services

EAS	Emergency Alert System

EASN	Emergency Alert System Notification

ESO	Emergency Service Only

ESS	Extended Service Set

GAS 	Generic Advertisement Service

MICS	Media Independent Command Services

MIES 	Media Independent Event Services

MIH 	Media Independent Handover

MIIS	Media Independent Information Service

MIHF 	Media Independent Handover Function

NAI 	Network Access Identifier

PoS	Point of Service

PSAP	Public Safety Answering Point

SSIDC IE	SSID container information element.

Change the following table:

Table 7-43t—Network Type Codes

| Network Type Code Bits (b0-b3) | Meaning | Description |
| --- | --- | --- |
| 0x0 | Private network | Network requires user accounts for access |
| 0x1 | Private network with guest access | Network requires user accounts for access, but guest accounts area available |
| 0x2 | Chargeable | Network is accessible to anyone, however, access to the network requires payment. Further information on types of charges may be available through other methods (802.21, http or https re-direct, etc.). |
| 0x3 | Free public network | Network is accessible to anyone and no charges apply for network use. |
| 0x4 | Emergency Network | The network supports emergency services. Further information is available from a GAS native query (see 7.3.3) |
| 0x5-0xE | Reserved | Not used. |
| 0xF | Reserved | Reserved for wildcard network type |

Change the following subclause:

      - Native Info Network Authentication Type Information element

The Native Info Network Authentication Type information element provides a list of authentication types that are used on the indicated SSID. The format of the Native Info Network Authentication Type element is shown in Figure 7-95bj..

|  | Info ID (3) | Length | Status Code | Network Authentication Type Unit #1 | Network Authentication Type Unit #2 | … | Network Authentication Type Unit #N |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 2 | variable | variable | … | Variable |

Figure 7.95bj—Native Info Network Authentication Type element format

The Length field is the length of the Status Code plus the Network Authentication Type element and is determined by the number and size of the Network Authentication Type Units, as described in this subclause.

The Status Code is a 2-octet field with values from Table 7-23.

Each Network Authentication Type Unit has the structure shown in Figure 7-95bk.

|  | Network Authentication Type Indicator Value | Network Authentication Type Unit Length | Network Authentication Type Indicator data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure 7-95bk—Network Authentication Type Unit

The Network Authentication Type indicator value has one of the values shown in Table 7-43x.

The Network Authentication Type Unit Length field is set to the number of octets in the Network Authentication

Type Unit.

Each indicator value defines additional information that may be communicated.

Table 7-43z—Network Authentication Type indicator value

| Value | Meaning |
| --- | --- |
| 0 | Acceptance of legal terms and conditions |
| 1 | On-line enrolment supported |
| 2 | http or https re-direct |
| 3-255 | Reserved |

If the Network Authentication Type indicator value is 0, the network requires the user to accept terms and conditions. No Network Authentication Type data is present.

If the Network Authentication Type indicator value is 1, the network supports on-line enrollment. Higher-layer software on the non-AP STA may indicate to the user that accounts may be created. No Network Authentication Type Data is present.

If the Network Authentication Type indicator value is 2, the network supports http or https re-direct.  Higher layer software on the non-AP STA will exchange credentials with the network.  The Network Authentication Type Data may be used to indicate the version of the protocol chosen.

Change the following subclause:

### T.1.1	Background on Emergency Call Support Over 802.11 Access Network

When IEEE 802.11 WLANs are used to carry VoIP calls, current solutions provide no special handling for emergency services. There are no mechanisms for a user to determine whether an access point and the network infrastructure can provide any support for emergency services (e.g. E911 services). In addition, to use a public access point a user must go through the standard authentication process (e.g. EAP-based or http/https re-direct) before being able to use the access point for emergency calls which limits the ability to make emergency calls.

Change the following subclause:

Bibliography

P.1 General

Insert the following refences to the end of the bibliograpy list. ANA to assign the next reference number; the number indicated below is tentative.

[B33] International Code Council, Inc., “International Building Code 2006”, November 2006, ISBN-13: 978-1-58001-251-5.

[B34] IETF RFC-4282, “The Network Access Identifier”, B. Aboba, M. Beadles, J. Arkko, and P. Eronen, December 2005 (status: informational).

[B35] IETF RFC-2903 “Generic AAA architecture”, C. de Laat, G. Gross, L. Gommans, J. Vollbrechtm and D. Spence, August 2000 (status informational)

[B36] IETF RFC-350 “IEEE 802.1X Remote Authentication Dial In User Service (RADIUS) Usage Guidelines”, P. Congdon, B. Aboba, A. Smith, G. Zorn, and J. Roese, Sept 2003.

[B37] NENA Functional and Interface Standards for Next Generation 9-1-1 (i3) NENA 08-002V120071218, Issue 0.1 DRAFT,  < http://www.nena.org/media/files/08-002V120071218.pdf>, May 15, 2007.

[B38] Wi-Fi Alliance, “Best Current Practices for Wireless Internet Service Provider (WISP) Roaming”, version 1.0. B. Anton, B. Bullock, and J. Short, February 2003.

References:

TGu - working draft body  802.11u D1.03.pdf

802.11-2007