IEEE P802.11
Wireless LANs

| LB115-CID5154-AMSDU |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

Addressing comments from TGn LB115 from the MAC adhoc subgroup comment group A-MSDU – see document 11-07-2706.

| CID | Commenter name | Location | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 5154 | Chu, Liwen | 101.04 | 9.1 | Base-line 802.11 standard section 9.1.3.1 should be included in 802.11n draft with A-MSDU being inserted after each MSDU. | Please change the text accordingly. | Counter – editor shall insert an appropriate header into TGn draft D3.02 to note that the amendment is modifying the baseline subclause “9.1.3.1 HCF contention-based channel access (EDCA)” and shall show changes to the baseline being the replacement of the sentence “A value of 0 for this duration means that the EDCA TXOP is limited to a single MSDU or MMPDU at any rate in the operational set of the BSS.” found in subclause 9.1.3.1. with “A value of 0 for this duration means that the EDCA TXOP is limited as per the rule for TXOP Limit value 0 found in 9.9.1.2 EDCA TXOPs“ Commenter to note that the first instance of MSDU in this subclause is not limiting, since it simply states that MSDUs may be transmitted – because A-MSDUs and A-MPDUs contain MSDUs, they both are allowed by the statement, which is not specific to the style of frame that delivers the MSDUs. |
| 5603 | Stephens, Adrian | 103.42 | 9.2.5.5 | I question whether we should call out A-MSDUs in 9.2.5.5. This is a DCF section. Surely an HT STA transmitting any Data MPDU is using an HCF mechanism. | Remove the edits to 9.2.5.5. | Accept |
| 5879 | Adachi, Tomoko | 119.20 | 9.7c | "A STA that has a value of true for the MIB attribute dot11HighthroughputOptionImplemented shall not transmit an A-MSDU to a STA that has the value of false for its MIB attribute dot11HighthroughputOptionImplemented." How in the world can the transmitting STA know that the setting of a MIB variable in the receiving STA is? I really doubt that the transmitting STA has access to the receiving STA's MIB variables. | Change the condition in this sentence to something that the transmitting STA can directly determine (which may be a more visible manifestation of the setting of the MIB variable) | Counter – accept in principile - see resolution to CID 5617 which asked for a similar change. |

References: