IEEE P802.11
Wireless LANs

| TGp LB 110 Editorial Change to Clause 7.3.2.27 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-02-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Francois Simon | ARINC, Inc. | ARINC, Inc. 2551 Riva Road Annapolis, MD 21401 | 410-266-4531 | fsimon@arinc.com |

Abstract

This submittal shows the change to Figure 7-76a, Capabilities field first octet, to convert this to a table (7-35a, Capabilities field).  This is in response to a request by TGv and concurrence by the Task Group editors.   See P802.11y D7.1 for the initial submittal of this change. This does not change the present content of the clause but allows for a simpler update of the field.  (TGv is in the process of requesting a significant number of bits which will expand this to several octets.)

7.3.2.27 Extended Capabilities information element

EDITORIAL NOTE - The changes made to this IEEE 802.11 Std 2007 subclause are consistent with changes made by TGn and TGy.

Change the first paragraph of subclause 7.3.2.27 as follows:

The Extended Capabilities information element carries information about the capabilities of an IEEE 802.11 STA intended to and augments the Capability Information field (CIF) when these bits are fully allocated. The format of this information element is shown in Figure 7-76.

Change the last paragraph of subclause 7.3.2.27 as follows:

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the information element. There are no capabilities defined for this field in this revision of the standard.   The length of the Capabilities field is a variable n.  The Capabilities is shown in Table 7-35a

Table 7-35a – Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | On-demand beacon | When the On-demand beacon bit is set to 1, the beacon transmission is initiated by the station management entity (SME) and indicates an On-demand beacon. |
| 2 | Reserved |  |
| 3 | WAVE indication | If MIB attribute dot11WAVEEnabled is true, then the WAVE indication bit is set to 1, otherwise it is set to 0. |
| 4 - n | Reserved |  |

| Date | CID# | STATUS | IEEE DOC. | COMMENTS |
| --- | --- | --- | --- | --- |
| 02/28/08 | N/A | IP | P802.11y/D8.0 | Alignment with TGn and TGy |

References:

P 802.11y D7.1

P802.11p D3.03