IEEE P802.11
Wireless LANs

| Frame Reorder Avoiding in Tunneled Direct Link Setup (TDLS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-24-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics | 1060 East Brokaw Rd. San Jose, CA 95131 | +1-408-467-8436 | liwen.chu@st.com |
| George Vlantis | STMicroelectronics | 1060 East Brokaw Rd. San Jose, CA 95131 | +1-408-451-8109 | George.vlantis@st.com |
| Vincenzo Scarpa | STMicroelectronics | via per Arnesano 16, c/o Distretto Tecnologico, 73100 Lecce - Italy | +39 083-235-3124 | mailto:vincenzo.scarpa@st.com |

Abstract

This document contains a normative text proposal to avoid frame reorder in Tunneled Direct Link Setup (TDLS). This document is based on TGz Draft 0.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

11 MLME

11.z1 Tunneled Direct Link Setup

Change paragraph 5 of clause 11.z1 to following text:

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a STA may send a TDLS Tx Path Switch Request indicating a switch to the direct path. The first Data frame transmitted over the direct link should be transmitted after receipt of the TDLS Tx Path Switch Response in this case, possibly after additional delay to account for potential EDCA related reordering inside the AP. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange which is part of the direct link setup or the peer key handshake for this purpose.