IEEE P802.11
Wireless LANs

| Text for Preventing Detrimental Greenfield Effects on DFS Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Jan Kruys | Cisco Systems | Haarlerbergpark, Haarlerbergweg 13-19, Amsterdam, Netherlands 1101 CH | +31-20-357-2447 | jkruys@cisco.com |
| Luke Qian | Cisco Systems | 4125 Highlander Parkway Richfield , OH 44286 | 330-523-2051 | lchia@cisco.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 | 408-331-6835 | audeh@tropos.com |
| Jorjeta Jetcheva | Firetide, Inc. | 16795 Lark Ave, Suite 200 Los Gatos, CA 9503 | 408-355-7215 | jjetcheva@firetide.com |
| Stephen Rayment | BelAir Networks | 603 March Road Kanata ON Canada K2K 2M5 | 613-254-7070 x112 | srayment@belairnetworks.com |
| Peter Loc | Marvell Semiconductors | 5488 Marvell Lane Santa Clara, CA 95054 | 408-222-9148 | ploc@marvell.com |
| Tomoko Adachi | Toshiba | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki 212-8582, JAPAN | +81-44-549-2433 | tomo.adachi@toshiba.co.jp |

Abstract

This document proposes resolutions for comments from the coex ad hoc: 5123, 5795 and 5363.  They point out that the detrimental effects from Greenfield transmissions on DFS Channels must be prevented.

This submission is accompanied with a presentation, 08/0351, which provides more background to this issue and also explain the prevention protocol proposed here.

Draft text change instructions based on D3.03.

TGn Editor: Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please insert the following new MIB attribute in the appropriate location of Annex D of TGn draft D3.03, with the appropriate value of dot11HTStationConfigEntry or dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11HTStationConfigEntry or dot11OperationEntry list, as appropriate.

dot11GreenfieldDisabledTransitionDelayInterval OBJECT-TYPE

SYNTAX INTEGER (30…2880)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the interval in minutes to re-enable HT-greenfield reception after it has been disabled due to existence of non-HT OBSS on a DFS channel.”

DEFVAL { 30 }

:::= { dot11OperationEntry 42 }

TGn Editor: Please insert the following new subclause following subclause 11.9.8.4 Channel management at the AP and in an IBSS, on page 227 of D3.03.

11.9.8.5 HT-greenfield transmissions in DFS bands

The requirements described in this subclause apply only when an HT STA is operating in a regulatory class for which its behavior limits set listed in Annex J includes or, due to current regulations, should include the value 4; that is, the regulatory class is subject to DFS.

In this subclause the following terms are used:

– GF STA: an HT STA with the HT-greenfield subfield of the HT Capabilities element equal to 1.

– GF AP: a GF STA that is an AP or the DFS Owner of an IBSS where there is one or more GF STAs.

– GF non-AP STA: a GF STA in an infrastructure BSS that is not an AP or a GF STA in an IBSS that is not the DFS Owner.

– Ex-GF AP: an AP that was a GF AP and maintains a GF Countdown Timer that is non-zero.

A Non-HT OBSS scan operation is a passive or active scan of the primary channel, and secondary channel if it is within a 20/40 MHz BSS, which a STA currently uses or intends to use.  During a Non-HT OBSS scan operation, the channel scan duration is a minimum of dot11OBSSScanPassiveTotalPerChannel TU when scanning passively and a minimum of dot11OBSSScanActiveTotalPerChannel TU when scanning actively.

Before a GF STA starts a BSS, the GF STA shall perform a Non-HT OBSS Scan to search for any existing non-HT OBSSs.

Before an HT AP changes OBSS Non-HT STAs Present field of the HT Information element from 1 to 0, that is to become a GF AP, the HT AP shall perform a Non-HT OBSS Scan to search for any existing non-HT BSSs.

An HT AP while it is a GF AP or an Ex-GF AP shall maintain a GF Countdown Timer that stops counting when it reaches zero.

When a GF AP detects there is one or more non-HT OBSS, the GF AP shall load the GF Countdown Timer with the value dot11GreenfieldDisabledTransitionDelayInterval and start the countdown. The AP is now an Ex-GF AP. The Ex-GF AP shall set the OBSS Non-HT STAs Present field of the HT Information element to 1.

Whenever a detection of a non-HT OBSS occurs, the Ex-GF AP shall reload the countdown timer with the value dot11GreenfieldDisabledTransitionDelayInterval.  When the Ex-GF AP’s countdown timer reaches zero, the AP is no longer an Ex-GF AP and it may set the OBSS Non-HT STAs Present field of the HT Information element to 0.

Note—Detection of a non-HT OBSS can be achieved by the reception of a Beacon or Probe Response that does not contain an HT Capabilities element or HT Information element.

An HT AP shall not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF while it is an Ex-GF AP.  An HT AP may transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if it is a GF AP.

A GF non-AP STA shall not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF after the GF non-AP STA decodes a frame received from its associated AP containing an HT Information element with the OBSS Non-HT STAs Present field set to 1.

Note—This requirement applies to Direct Link Setup as well.

A GF non-AP STA may transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF after GF non-AP STA decodes a frame received from its associated AP containing an HT Information element with the OBSS Non-HT STAs Present field set to 0.

TGn Editor: Please change row 9 of the table “Table 7-43q—HT Information element” starting on line 7 on page 82 of D3.03 as shown.

| Field | Definition | Encoding | Reserved in IBSS ? |
| --- | --- | --- | --- |
| OBSS Non- HT STAs Present | Indicates if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. If on channel subject to DFS, this field indicates if there exists any non-HT OBSSs and if HT-greenfield transmissions are allowed. Present in Beacon and Probe response frames transmitted by an AP. Otherwise reserved. See 9.13.3.3 (Use of OBSS Non-HT STAs Present) and 11.9.8.5 (HT-greenfield transmissions in DFS bands). | If not on channel subject to DFS: Set to 1 if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. Examples of when this bit may be set to 1 include, but are not limited to, when — one or more non-HT STAs are associated — a non-HT BSS is overlapping (a non- HT BSS may be detected by the reception of a Beacon where the supported rates only contain Clause 15, 17, 18 or 19 rates) — a management frame (excluding a Probe Request) is received where the supported rate set includes only Clause 15, 17, 18 and 19 rates. Set to 0 otherwise. If on channel subject to DFS: Set to 1 if there exists one or more non-HT OBSSs and that HT-greenfield transmissions are disallowed. Set to 0 otherwise. | YN |