IEEE P802.11
Wireless LANs

| Text for Preventing Detrimental Greenfield Effects on DFS Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Jan Kruys | Cisco Systems | Haarlerbergpark, Haarlerbergweg 13-19, Amsterdam, Netherlands 1101 CH | +31-20-357-2447 | jkruys@cisco.com |
| Luke Qian | Cisco Systems | 4125 Highlander Parkway Richfield , OH 44286 | 330-523-2051 | lchia@cisco.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 | 408-331-6835 | audeh@tropos.com |
| Jorjeta Jetcheva | Firetide, Inc. | 16795 Lark Ave, Suite 200 Los Gatos, CA 9503 | 408-355-7215 | jjetcheva@firetide.com |
| Stephen Rayment | BelAir Networks | 603 March Road Kanata ON Canada K2K 2M5 | 613-254-7070 x112 | srayment@belairnetworks.com |
| Rohit Nabar | Marvell Semiconductors | 5488 Marvell Lane Santa Clara, CA 95054 | 408-222-0865 | rnabar@marvell.com |
| Tomoko Adachi | Toshiba | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki 212-8582, JAPAN | +81-44-549-2433 | tomo.adachi@toshiba.co.jp |

Abstract

This document proposes resolutions for comments from the coex ad hoc: TBD.  They point out that the detrimental effects from Greenfield transmissions on DFS Channels must be prevented.

This submission is accompanied with a presentation, 08/0666r0, which provides more background to this issue and also explain the prevention protocol proposed here.

Draft text change instructions based on D4.01 plus speculative edits if submission 08/0598r3 is accepted.

Discussion – not to be incorporated into the draft:

USA FCC references for 50-100 us DFS requirements may be checked at http://www.atcb.com/publicdocs/FCC-06-96A1.pdf and Japanese ARIB references may be obtained from http://www.arib.or.jp/english/html/overview/st_ej.html.

802.11 Annex J for the USA and Japan is outdated in that it omits latest DFS requirements. This draft makes appropriate updates where required.

TGn Editor: Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please insert the following new MIB attribute in the appropriate location of Annex D, with the appropriate value of dot11HTStationConfigEntry or dot11OperationEntry provided by the editor, and with the appropriate addition of the new MIB attribute to the dot11HTStationConfigEntry or dot11OperationEntry list, as appropriate.

dot11GreenfieldDisabledTransitionDelayInterval OBJECT-TYPE

SYNTAX INTEGER (30…2880)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the interval in minutes to re-enable HT-greenfield reception after it has been disabled due to existence of non-HT OBSS on a DFS channel.”

DEFVAL { 30 }

:::= { dot11OperationEntry 42 }

TGn Editor: Please insert the following new subclause following subclause 11.9.7.2 Selecting and advertising a new channel in an IBSS

11.9.7.3 HT-greenfield transmissions in regulatory classes with a Behavior limits set of 16

The requirements described in this subclause apply only when an HT STA is operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16; that is, the regulatory class is subject to DFS with 50-100 microsecond radar pulses.

In this subclause the following terms are used:

– GF STA: an HT STA with the HT-greenfield subfield of the HT Capabilities element equal to 1.

– GF AP: a GF STA that is an AP.

– GF non-AP STA: a GF STA that is not an AP.

A Non-HT OBSS scan operation is a passive or active scan of the primary channel, and secondary channel if it is within a 20/40 MHz BSS, which a STA currently uses or intends to use.  During a Non-HT OBSS scan operation, the channel scan duration is a minimum of dot11OBSSScanPassiveTotalPerChannel TU when scanning passively and a minimum of dot11OBSSScanActiveTotalPerChannel TU when scanning actively.

Before a GF STA starts a BSS, the GF STA shall perform a Non-HT OBSS Scan to search for any existing non-HT OBSSs.

Before a GF AP changes OBSS Non-HT STAs Present field of the HT Information element from 1 to 0, the HT AP shall perform a Non-HT OBSS Scan without detecting any non-HT OBSSs.

A GF AP shall maintain a GF Countdown Timer that stops counting when it reaches zero.

When a GF AP detects there is one or more non-HT OBSS, the GF AP shall load the GF Countdown Timer with the value dot11GreenfieldDisabledTransitionDelayInterval and start the countdown.

Whenever a detection of a non-HT OBSS occurs, the GF AP shall reload the countdown timer with the value dot11GreenfieldDisabledTransitionDelayInterval.

Note—Detection of a non-HT OBSS can be achieved by the reception of a Beacon or Probe Response that does not contain an HT Capabilities element or HT Information element.

The GP AP may transmit an HT Information element containing the OBSS Non-HT STAs Present field set to 0 if and only if its GF Countdown timer contains the value zero.

A GF AP should not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if its GF Countdown timer contains a non-zero value and the TXTIME of the PPDU frame is between 50 and 100 microseconds inclusive.

A GF non-AP STA should not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if the most recently received frame received from its AP containing an HT Information element has the OBSS Non-HT STAs Present field set to 1 and the TXTIME of the PPDU frame is between 50 and 100 microseconds inclusive..

Note—This requirement applies to Direct Link Setup as well.

TGn Editor: Please change row 9 of the table “Table 7-43q—HT Information element” in Section 7.3.2.58 as shown.

| Field | Definition | Encoding | Reserved in IBSS ? |
| --- | --- | --- | --- |
| OBSS Non- HT STAs Present | Indicates if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. If the BSS is operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16, this field indicates if there exist any non-HT OBSSs and whether HT-greenfield transmissions are allowed. Present in Beacon and Probe response frames transmitted by an AP. Otherwise reserved. See 9.13.3.4 (Use of OBSS Non-HT STAs Present) and 11.9.7.3 (HT-greenfield transmissions in regulatory classes with a Behavior limits set of 16). | If not operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16: Set to 1 if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. Examples of when this bit may be set to 1 include, but are not limited to, when — one or more non-HT STAs are associated — a non-HT BSS is overlapping (a non- HT BSS may be detected by the reception of a Beacon where the supported rates only contain Clause 15, 17, 18 or 19 rates) — a management frame (excluding a Probe Request) is received where the supported rate set includes only Clause 15, 17, 18 and 19 rates. Set to 0 otherwise. If operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16: Set to 1 if there exists one or more non-HT OBSSs. Indicates that HT-greenfield transmissions of duration between 50 and 100 microseconds are disallowed in the BSS. Set to 0 otherwise. | Y |

TGn Editor: Please change the last row of table I-3 of D4.00 into the two rows as identified below.

Table I.3—Behavior limits sets

| Behavior limits set |  | USA | Europe | Japan |
| --- | --- | --- | --- | --- |
| 16 | DFS with 50-100 microsecond pulses | FCC 47CFR2.947 and FCC MO&O 06-96, Appendix section 6.2, Table 6, Radar Type 5 | Reserved | ARIB STD-T71 (Edition 5.0) Section 3.1.7(2)(ii)(3)A)(b)2) |
| 1617-255 |  | Reserved | Reserved | Reserved |

TGn Editor: Please insert at least the following rows of table J.1 of IEEE 802.11:2007 into the 11n draft and change the rows as identified below.

Table J.1—Regulatory classes in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1, 4, 16 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1, 4, 16 |

TGn Editor: Please change the following rows of table J.1 of D4.01 as identified below.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 5 | 40 | 100, 108, 116, 124, 132 | 200 | - | 1 | 1,4,13,16 |
| 29 | 5 | 40 | 104, 112, 120, 128, 136 | 200 | - | 1 | 1,4,14,16 |

TGn Editor: Please change the following rows of table J.3 of D4.01 as identified below.

Table J.3—Regulatory classes in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (dBm) | Transmit Power Limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 50 | 1 | 1, 3, 4, 5, 6, 16 |
| 35 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 25 | 1 | 1, 4, 5, 6, 16 |
| 39 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 13, 16 |
| 40 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 13, 16 |
| 44 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 14, 16 |
| 45 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 14, 16 |