IEEE P802.11
Wireless LANs

| Text for Preventing Detrimental Greenfield Effects on DFS Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Jan Kruys | Cisco Systems | Haarlerbergpark, Haarlerbergweg 13-19, Amsterdam, Netherlands 1101 CH | +31-20-357-2447 | jkruys@cisco.com |
| Luke Qian | Cisco Systems | 4125 Highlander Parkway Richfield , OH 44286 | 330-523-2051 | lchia@cisco.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 | 408-331-6835 | audeh@tropos.com |
| Jorjeta Jetcheva | Firetide, Inc. | 16795 Lark Ave, Suite 200 Los Gatos, CA 9503 | 408-355-7215 | jjetcheva@firetide.com |
| Stephen Rayment | BelAir Networks | 603 March Road Kanata ON Canada K2K 2M5 | 613-254-7070 x112 | srayment@belairnetworks.com |
| Rohit Nabar | Marvell Semiconductors | 5488 Marvell Lane Santa Clara, CA 95054 | 408-222-0865 | rnabar@marvell.com |
| Tomoko Adachi | Toshiba | 1, Komukai Toshiba-cho, Saiwai-ku, Kawasaki 212-8582, JAPAN | +81-44-549-2433 | tomo.adachi@toshiba.co.jp |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |

Abstract

This document proposes resolutions for LB 129 comments from the coex ad hoc: CIDs 7002, 7004, 7005 and 8103.

Draft text change instructions based on D5.02.

Discussion – not to be incorporated into the draft:

USA FCC references for 50-100 us DFS requirements may be checked at http://www.atcb.com/publicdocs/FCC-06-96A1.pdf and Japanese ARIB references may be obtained from http://www.arib.or.jp/english/html/overview/st_ej.html.

TGn Editor: Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft D5.02.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please insert the following new subclause following subclause 11.9.7.2 Selecting and advertising a new channel in an IBSS

11.9.7.3 HT-greenfield transmissions in regulatory classes with a Behavior limits set of 16

The requirements described in this subclause apply only when an HT STA is operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16; that is, the regulatory class is subject to DFS with 50-100 microsecond radar pulses.

A Non-HT OBSS scan operation is a passive or active scan of the primary channel, and secondary channel if it is within a 20/40 MHz BSS, which a STA currently uses or intends to use.  During a Non-HT OBSS scan operation, the channel scan duration is a minimum of dot11OBSSScanPassiveTotalPerChannel TU when scanning passively and a minimum of dot11OBSSScanActiveTotalPerChannel TU when scanning actively.

Before a HT STA starts a BSS with the OBSS Non-HT STAs Present field of the HT Operation element set to 0, the HT STA shall perform a Non-HT OBSS Scan in order to search for any existing non-HT OBSSs.

When an HT STA detects there is one or more non-HT OBSS, if the HT STA starts a BSS on that channel then the HT STA shall set the OBSS Non-HT STAs Present field of the HT Operation element to 1; otherwise, the HT STA may set the OBSS Non-HT STAs Present field of the HT Operation element to 0.

Note—Detection of a non-HT OBSS can be achieved by the reception of a Beacon or Probe Response that does not contain an HT Capabilities element or HT Operation element.

An HT AP shall not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if the OBSS Non-HT STAs Present field of the HT Operation element is set to 1 in the most recently transmitted management frame that contained that element.

An HT non-AP STA shall not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if the most recently received frame received from its AP containing an HT Operation element has the OBSS Non-HT STAs Present field set to 1.

Note—This requirement applies to Direct Link Setup as well.

When moving the BSS to a new channel and before completing a non-HT OBSS scan of the new channel, an HT AP shall set the OBSS Non-HT STAs Present field of the HT Operation element to 1; after having completed one non-HT OBSS scan of the new channel, if the HT AP has detected that there are zero non-HT OBSSs then the HT AP may set the OBSS Non-HT STAs Present field of the HT Operation element to 0.

TGn Editor: Please change row 9 of the table “Table 7-43q—HT Operation element” in Section 7.3.2.58 as shown.

| Field | Definition | Encoding | Reserved in IBSS ? |
| --- | --- | --- | --- |
| OBSS Non- HT STAs Present | Indicates if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. If the BSS is operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16, this field indicates if there exist any non-HT OBSSs and whether HT-greenfield transmissions are allowed. Present in Beacon and Probe response frames transmitted by an AP. Otherwise reserved. See 9.13.3.4 (Use of OBSS Non-HT STAs Present) and 11.9.7.3 (HT-greenfield transmissions in regulatory classes with a Behavior limits set of 16). | If not operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16: Set to 1 if the use of protection for non-HT STAs by overlapping BSSs is determined to be desirable. See NOTE 2. Set to 0 otherwise. If operating in a regulatory class for which the Behavior limits set listed in Annex J includes the value 16: Set to 1 if there exists one or more non-HT OBSSs. Indicates that HT-greenfield transmissions are disallowed in the BSS. Set to 0 otherwise. | Y |

TGn Editor: Please change the last row of table I-3 of IEEE 802.11:2007 into the two rows as identified below.

Table I.3—Behavior limits sets

| Behavior limits set |  | USA | Europe | Japan |
| --- | --- | --- | --- | --- |
| 16 | DFS with 50-100 microsecond pulses | FCC 47CFR2.947 and FCC MO&O 06-96, Appendix section 6.2, Table 6, Radar Type 5 | Reserved | ARIB STD-T71 (Edition 5.0) Section 3.1.7(2)(i)(3)A)(b)2) |
| 1617-255 |  | Reserved | Reserved | Reserved |

TGn Editor: Please change the following rows of table J.1 of D5.02 as identified below.

Table J.1—Regulatory classes in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1, 4, 16 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1, 4, 16 |

TGn Editor: Please change the following rows of table J.1 of D5.02 as identified below.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 5 | 40 | 100, 108, 116, 124, 132 | 200 | - | 1 | 1,4,13,16 |
| 29 | 5 | 40 | 104, 112, 120, 128, 136 | 200 | - | 1 | 1,4,14,16 |

TGn Editor: Please change the following rows of table J.3 of D5.02 as identified below.

Table J.3—Regulatory classes in Japan

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (dBm) | Transmit Power Limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 50 | 1 | 1, 3, 4, 5, 6, 16 |
| 35 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 25 | 1 | 1, 4, 5, 6, 16 |
| 39 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 13, 16 |
| 40 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 13, 16 |
| 44 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 14, 16 |
| 45 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 14, 16 |