IEEE P802.11
Wireless LANs

| Text for Disallowing Greenfield on DFS Channels |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 | 408-331-6835 | audeh@tropos.com |
| Jorjeta Jetcheva | Firetide, Inc. | 16795 Lark Ave, Suite 200 Los Gatos, CA 9503 | 408-355-7215 | jjetcheva@firetide.com |
| Stephen Rayment | BelAir Networks | 603 March Road Kanata ON Canada K2K 2M5 | 613-254-7070 x112 | srayment@belairnetworks.com |
| Peter Loc | Marvell Semiconductors | 5488 Marvell Lane Santa Clara, CA 95054 | 408-222-9148 | ploc@marvell.com |

Abstract

During the discussion of submission 08/351r1 on this topic, members of the task group expressed concerns that the proposed compromise wasn’t strict enough to provide complete protection of legacy devices from unreasonable effects by Greenfield transmissions on the DFS channels.

As such, we propose text here for this complete protection mechanism, namely to disallow Greenfield operation on the DFS channels at all.

This document, if accepted, proposes resolutions for comments from the coex ad hoc: 5123, 5795 and 5363.

Draft text change instructions based on D3.03.

TGn Editor: Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please insert the following new subclause following subclause 11.9.8.4 Channel management at the AP and in an IBSS, on page 227 of D3.03.

11.9.8.5 HT-greenfield transmissions in DFS bands

An HT STA shall not transmit a PPDU with the FORMAT parameter of the TXVECTOR set to HT_GF if the HT STA is operating in a regulatory class for which its behavior limits set listed in Annex J includes or, due to current regulations, should include the value 4; that is, the regulatory class is subject to DFS.

Note — This requirement applies to Direct Link Setup as well.