IEEE P802.11
Wireless LANs

| TGp Frame Usage discussion |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Justin McNew | TechnoCom | 2030 Corte Del Nogal Suite 200 Carlsbad, CA 92011 | 760 438 5115 x 175 | jmcnew@technocom-wireless.com |

Abstract

This document includes resolutions for remaining unresolved comments assigned to McNew from letter ballot 110.

QoS related comments:

Problem: 802.11p implies the use of QoS, however, it does not indicate restrictions on data frame usage.  Also, the current draft would permit the use of many management and control frames not useful for WAVE.

Recommendations:

1. Recommend to change Table 7-58 as follows:

| Frame subtype | WAVE BSS | IBSS |  | Infrastructure BSS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Non-QoS | QoS | non-QoS |  |  |  | QoS |  |  |  |
|  |  | CP | CP | CP |  | CFP |  | CP |  | CFP |  |
|  |  | STA | STA | STA | AP | STA | AP | STA | AP | STA | AP |
| (Re)Association Request | --- | --- | --- | T | R | --- | --- | T | R | --- | --- |
| (Re)Association Response | --- | --- | --- | R | T | R | T | R | T | R | T |
| Probe Request | --- | T,R | T,R | T | R | --- | --- | T | R | --- | --- |
| Probe Response | --- | Tbe,R | Tbe,R | R | T | R | T | R | T | R | T |
| Beacon | T,R | Tb,R | Tb,R | R | T | R | T | R | T | R | T |
| ATIM | --- | T,R | T,R | --- | --- | --- | --- | --- | --- | --- | --- |
| Disassociation | --- | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| Authentication | --- | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| Deauthentication | --- | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| ADDTS Request | --- | --- | --- | --- | --- | --- | --- | T | R | T | R |
| ADDTS Response | --- | --- | --- | --- | --- | --- | --- | R | T | R | T |
| DELTS | --- | --- | --- | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| Schedule | --- | --- | --- | --- | --- | --- | --- | R | T | R | T |
| DLS Action | --- | --- | --- | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| Block Ack Action | --- | --- | T,R | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| Block Ack Req/BlockAck | --- | --- | T,R | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| PS-Poll | --- | --- | --- | T | R | --- | --- | T | R | T | R |
| RTS | T,R | T,R | T,R | T,R | T,R | --- | --- | T,R | T,R | T,R | T,R |
| CTS | T,R | T,R | T,R | T,R | T,R | --- | --- | T,R | T,R | T,R | T,R |
| ACK | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| CF-End | --- | (R) | (R) | (R) | (R) | R | T | (R) | (R) | R | T |
| CF-End+CF-Ack | --- | (R) | (R) | (R) | (R) | R | T | (R) | (R) | R | T |
| Null | --- | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| Data | --- | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R | T,R |
| (Data+)CF-Poll(+CF-Ack) | --- | --- | --- | --- | --- | R | T | --- | --- | --- | --- |
| (Data+)CF-Ack | ---- | ---- | ---- | ---- | ---- | T,R | T,R | --- | --- | T,R, Rda,Rq | T,R, Rda,Rq |
| QoS Null | --- | --- | T,R | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| QoS Data | T,R | --- | T,R | --- | --- | --- | --- | T,R | T,R | T,R | T,R |
| QoS (Data+)CF-Poll | --- | --- | --- | --- | --- | --- | --- | R | T | R | T |
| QoS (Data+)CF-Poll+Ack | --- | --- | --- | --- | --- | --- | --- | Rq,Rda | Tda,Tq, | Rq,Rda |  |
| QoS (Data+)CF-Ack | --- | --- | --- | --- | --- | --- | --- | T,Rq, Rda | Tda, Tq,R | T,Rq, Rda | Tda, Tq,R |

2. Recommend to add the following text to the end of the last sentence in paragraph in 7.1.3.5.1: TID always identifies the TC for WAVE; i.e. traffic streams are not used WAVE mode.

3. Recommend to change the last sentence in the first row of table 7-6 to:

For QoS Null (no data) frames and QoS data frames sent while in WAVE mode, this is the only permissible value for the Ack Policy subfield.

4. Recommend to change the second sentence of the first paragraph of 7.1.3.5.5 to: “…frames sent by STAs associated in a BSS, or frames sent within a WAVE BSS, with bit 4 of the QoS Control field set to 1.

Motion: Accept recommendations 1 – 4 and instruct editor to make the necessary changes to the 802.11p draft.

Move:

Second:

Yes:	No:	Abstain:

Comment 184:

Existing text in clause 7.2.2:

The BSSID of the Data frame is determined as follows:

a) If the STA is an AP or is associated with an AP, the BSSID is the address currently in use by the STA contained in the AP.

b) If the STA is a member of an IBSS, the BSSID is the BSSID of the IBSS.

Motion: Add the following text under item b in 7.2.2 (above) and instruct the editor to make the necessary changes to the 802.11p draft:

c) If transmitting a data frame in WAVE mode when not commincating within a WAVE BSS, the BSSID is the wildcard BSSID.

Move:

Second:

Yes:	No:	Abstain:

Comment 217:

7.3.2.2, first paragraph, first sentence should read:

The Supported Rates element specifies up to eight rates in the Operational-Rate-Set parameter, as described in the MLME-JOIN.request, MLME-START.request and MLME-ONDEMANDBEACON.request primitives

7.3.2.2, third paragraph, first sentence should read:

The Supported Rate information in Beacon and Probe Response management frames is delivered to the management entity in a STA via the BSSBasicRateSet parameter in the MLME-SCAN.confirm and MLME-ONDEMANDBEACON.indication primitives.

Motion: Adopt the changes as noted above to resolve comment 217 and instruct the editor to make the necessary changes to the 802.11p draft.

Move:

Second:

Yes:	No:	Abstain:

General recommendations for draft 3.04

Editorial:

- 5.2.3.3 line 9 – need to put ‘to and from’ in front of the word ‘the’.

- 7.3.2.29, page 6 line 52 – delete ‘prioritized access operation’

- Figure 7-95a1 – ‘content’ should be plural

- 10.3.9.1.4 page 9 line 2.  Change “the locally” to “a locally”

- 10.3.42.1.2 – ‘Country’ is missing from parameters of MLME-ONDEMANDBEACON.request.  Also, replace WIE in primitive with WIE contents.

- 10.3.42.3.2 – ‘Country’ is missing from parameters of MLME-ONDEMANDBEACON.indication.  Also, replace WIE in primitive with WIE contents.

No motion required.

Technical (put on server Tuesday at 2:55 pm):

- 5.2.2a line 39 – 40 change last sentence to Distribution system services and security services are deferred to the station management entity or higher layers.

- 5.2.3.3 lines 8 – 9.  Recommend to move this sentence to be last sentence of fourth paragraph (above table 7-7)

- 7.3.2.29, page 7 line 2 – add ‘if the EDCA parameter set is present in the WAVE beacon; otherwise the default parameter set shall be used.

- 7.1.3.3.3 line 49 – delete this line.  Replace with ‘the value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS.7.1.3.3.3 line 49 – delete this line.  Replace with ‘the value of the BSSID in a WAVE BSS shall be the MAC address of the STA initiating the WAVE BSS.

- 11.3 – Change iii) to read as follows: “iii) Data frames between STAs that belong to a WAVE BSS sent with the BSSID of the WAVE BSS wherein either the “To DS” or “From DS” FC bits may be set to true."

Motion: Adopt Technical recommendations above and instruct the editor to make the necessary changes to the 802.11p draft.

Move:

Second:

Yes:	No:	Abstain