IEEE P802.11
Wireless LANs

| Channel Rejection Issues in workgroup draft |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| John Kenney | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 574-272-1403 | john@jbkenney.com |
| Vinuth Rai | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 650-251-0530 | vinuth@us.toyota-itc.com |

Abstract

We propose minor amendments to Clause 17 and Annex D to clarify the relationship between the MIB attribute dot11ACRType and the channel rejection specifications in Tables 17-13 and 17-13a.

Background

The TGp working group draft “Draft P802.11p_D3.03.pdf” discusses channel rejection in Clause 17 and Annex D.  This submission proposes two clarifications and one editorial fix.

Issue #1: the definition of the MIB attribute dot11ACRType in Annex D does not explain what meaning is attached to each of the specified values.

This MIB attribute has two defined values, 1 and 2.  Our previous letter ballot text, D3.0, defined the MIB attribute with the following text, which explained what each of those values meant:

“The Adjacent/Alternate Channel Rejection performance: Category 1 or Category 2.”

We still have two levels of adjacent/alternate channel rejection, but we no longer refer to them as Category 1 and Catetgory 2.  Instead, we have the “baseline” adjacent/alternate channel rejection levels specified in 802.11-2007 Table 17-13, and we have a set of “enhanced performance” adjacent/alternate channel rejection levels specified in Table 17-13a of our working group draft.  The text in Annex D was changed so that it no longer uses the obsolete terms.  The new text, however, is unclear:

“The Adjacent/Alternate Channel Rejection enhanced receiver performance.”

We propose to modify the Annex D text defining dot11ACRType to:

“The Adjacent/Alternate Adjacent Channel Rejection performance: 
when this attribute = 1 the levels in Table 17-13 apply;
when this attribute = 2 the levels in Table 17-13a apply.”

Issue #2:  the text in Clause 17 refers to “enhanced performance” without clarifying how that is indicated.

There are two sentences in Clause 17 that invoke the channel rejection levels in Table 17-13a.  One appears in Clause 17.3.10.2, “Adjacent Channel Rejection.” The other appears in Clause 17.3.10.3, “Alternate Adjacent Channel Rejection.”   The two sentences are identical:

“In WAVE mode, if enhanced performance is desired, the corresponding rejection shall be no less than that specified in Table 17-13a.”

The phrase “if enhanced performance is desired” is ambiguous for two reasons.  First, enhanced performance may always fairly be considered as desired.  Second, there is no attempt to indicate how this phrase relates to the MIB attribute dot11ACRType.  We propose to replace the sentence above in Clause 17.3.10.2 with:

“For a STA in WAVE mode, if enhanced performance is indicated 
(i.e. dot11ACRType = 2), the adjacent channel rejection levels in Table 17-13a apply.”

We further propose that the same sentence in Clause 17.3.10.3 be replaced with:

“For a STA in WAVE mode, if enhanced performance is indicated 
(i.e. dot11ACRType = 2), the alternate adjacent channel rejection levels in Table 17-13a apply.”

Editorial Issue:  Baseline table 17-13 uses a column heading “alternate adjacent channel rejection”.  The table for enhanced performance we’re adding, Table 17-13a, uses a column heading “nonadjacent channel rejection.”  We should instead use the same phrase from Table 17-13.

We propose changing “nonadjacent” to “alternate adjacent” in table 17-13a.

- Motion (if technical and/or significant):

Move to accept the proposed text from above and instruct the TGp editor to incorporate the proposed changes in text noted above

Motion by: John Kenney

Date:

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |