IEEE P802.11
Wireless LANs

| LB123 TCLAS Comment Resolutions Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-Mar 18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

- Abstract

This document contains normative text changes to resolve comment IDs 63, 340, 341, 342, 343, 344, 345, 346, 348, 351, 352 and 353 from   LB123.

The submission is referenced to draft D2.0 and augments to all editor instructions as well as the actual base draft modifications.

All new text relative to the draft D2.0 is identified in blue color and underlined and all deleted text is identified by strikethrough.

- TCLAS element

Change the first paragraph as follows:

The TCLAS element specifies an information element that contains a set of parameters necessary to identify incoming MSDUs (from a higher layer in all STAs or from the DS in an AP) with a particular TS to which they belong. The TCLAS element is also used when the traffic is not connected to a TS, for example by the FBMS and TFS services. If required, the TCLAS element is provided in ADDTS Request and ADDTS Response frames only for the downlink or bidirectional links. TCLAS element need not be provided for the uplink or direct link transmissions. The structure of this element is shown in Figure 7-85.

Insert a new row into Table 7-42 and change the Reserved value as follows:

| - Frame classifier type |  |
| --- | --- |
| Classifier type | Classifier parameters |
| 3 | Filter Offset parameters |
| 34 – 255 | Reserved |

Change the seventh and eighth paragraphs as follows:

For Classifier Type 1, frame classifier is defined for both IPv4 and IPv6, shown in Figure 7-88 and Figure 7-89 respectively and distinguished by the Version subfield. The classifier parameters represent corresponding values in a received IPv4 or IPv6 packet. The subfields in the classifier parameters are represented and transmitted in the big-endian format. The classifier parameters are the following parameters:

- IP Version,

- Source and Destination Address,

- Source and Destination Port (when Protocol/Next Header is TCP or UDP)

- DSCP value in Type of Service (IPv4) or Traffic Class (IPv6)

- Protocol (IPv4) or Next Header (IPv6)

- Flow Label (IPv6 only)

Note: Frame classification when extension headers are used is supported only if the TCLAS does not classify on ports (Classifier Mask has the Source and Destination Port bits set to 0) .

Change Figure 7-88 as follows:

|  | Classifier Type (1) | Classifier Mask | Version (4) | Source IP Address | Destination IP Address |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 4 |
|  | Source Port | Destination Port | DSCP | Protocol | Reserved |
| Octets: | 2 | 2 | 1 | 1 | 1 |
| Figure 7-88 Frame Classifier field of Classifier Type 1 for traffic over IPv4 |  |  |  |  |  |

Change Figure 7-89 as follows:

|  | Classifier Type (1) | Classifier Mask | Version (6) | Source IP Address | Destination IP Address |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 16 | 16 |
|  | Source Port | Destination Port | DSCP | Next Header | Flow Label |
| Octets: | 2 | 2 | 3 | 1 | 1 |
| - Frame Classifier field of Classifier Type 1 for traffic over IPv6 |  |  |  |  |  |

Insert the following paragraphs andthe figure after Figure 7-89 as indicated below:

The value in the Version subfield is set to the value specified in IETF RFC 791-1981[Bxx] or IETF RFC 2460-1998 [Byy].

IPv6 Flow Label Field..

The DSCP subfield contains the value in the 6 least significant bits. The 2 most significant bits  are reserved.

The Flow Label subfield contains the value in the 20 least significant bits. The 4 most significant bits are reserved.

Note: For example, the flow label 0x12345 is represented as the octet sequence 0x01, 0x23, 0x45.

..

A valid TCLAS element meets the following conditions, and is in error otherwise:

- If  Version subfield bit of the Classifier Mask is set to 0, then all the bits corresponding to classifier parameters other than the Source Port, Destination Port, DSCP and Protocol/Next Header are set to 0 in the Classifier Mask. The classifier applies to both IPv4 and IPv6 packets.

- If one or both of the Source and Destination subfield bits in the Classifier Mask are set to 1 then the IPv4 Protocol subfield bit or the IPv6 Next Header subfield bit in the Classifier Mask is set to 1 and the value in the Protocol or Next Header subfield is either set to TCP or UDP (IETF RFC 3232-2002 [Bxx]).

Insert the following text at the end of 7.3.2.31:

For Classifier Type 3, the classifier parameters are defined by a filter offset field and a filter value field. The Frame Classifier field of Classifier Type 3 for Filter Offset parameters is defined in Frame Classifier field of Classifier Type 3. It has a variable length.

|  | Classifier Type (3) | Filter Offset | Filter Value | Filter Mask |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable | Variable |
| - Frame Classifier field of Classifier Type 3 |  |  |  |  |

The value of the Filter Offset field is the number of octets following the MAC header (see 7.1) at which the Filter Value is compared, after any necessary decryption, reassembly or disaggregation. A value of zero for the Filter Offset indicates that the Filter Value field is to be compared to the first octet of the payload after decryption following the MAC header.

The Filter Value field is an octet string that is compared to the frame content, beginning at the octet indicated by the Filter Offset.

The Filter Mask field is an octet string that is used to indicate which bits in the Filter Value field are compared. The length of the Filter Mask field is equal to the length of the Filter Value field. A bit in the Filter Value field is only compared if the matching bit in the Filter Mask field is set to 1.

- TCLAS Processing element

Change the first sentence of 7.3.2.33 as follows:

The TCLAS Processing element is present in the ADDTS Request, and ADDTS Response and FBMS Request frames if there are multiple TCLASs associated with the request.

- TS deletion