IEEE P802.11
Wireless LANs

| Odd Mesh Header |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Javier Cardona | cozybit Inc. | 165 Jessie St., San Francisco, CA 94105 | +1-415-974-6770 | javier@cozybit.com |
| Johannes Berg | Sip Solutions | Adlerweg 10, 32108 Bad Salzuflen, Germany | +49-179-7049278 | johannes@sipsolutions.net |
| Guido Hiertz | Philips | ComNets, Chair of Communication Networks, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-8025829 | hiertz@ieee.org |

Abstract

Normative text changes to resolve the mesh header alignment problems described in IEEE 802.11-08/0341r3

Change Section  7.1.3.5b.4 as follows:

7.1.3.5b.4 Mesh Sequence Number field

The Mesh Sequence Number field is an unsigned integer 24 32 bits in length and used to detect duplicate reception of messages in a Mesh network. See 11B.5.5.3 for details on how the Mesh Sequence Number is used to discard duplicate frames.

Note: it is believed that a 24 32 bit sequence number is sufficient as the rollover would occur after a period of 28 minutes 5 days assuming a source transmission rate of 10^4 packets per second.

Change Section 11B.5.5.2.1 as follows:

The Source MP shall set the Mesh Sequence Number field in the Mesh Header to a value from a single modulo-65536 4294967296 counter that is incrementing by 1 for each new frame.

Change Figure s4 as follows:

| Octets: 1 | 1 | 3 4 | 0, 6, 12 or 18 |
| --- | --- | --- | --- |
|  |  | Mesh Sequence Number |  |