IEEE P802.11
Wireless LANs

| Speculative TGz Draft 0.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-3-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document contains a speculative TGz Draft 0.3, which includes the normative text proposals which were approved on Tuesday, March 18, the peer power save proposal from doc. 11-08-0071-04-000z, and off-channel DLS from doc. 11-07-2762-05-000z (the latter two are speculative, pending approval of the task group). This document is based on TGz Draft 0.2.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

IEEE  P802.11z™/D0.3

Draft STANDARD for Information Technology- Telecommunications and information exchange between systems- 
Local and metropolitan area networks-
Specific requirements-

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

Amendment 7: Direct Link Setup

Prepared by the 802.11 Working Group of the IEEE 802 Committee

Copyright © 2007 by the IEEE.

3 Park Avenue

New York, NY 10016-5997, USA

All rights reserved.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

IEEE Standards Activities Department

Standards Licensing and Contracts

445 Hoes Lane, P.O. Box 1331

Piscataway, NJ 08855-1331, USA

Abstract: This amendment provides Direct Link Setup enhancements to the IEEE 802.11 MAC, and PHY, extending direct link setup to be independent of the access point, and adding power save capabilities.

Keywords: Direct Link, Power Saving

Introduction

(This introduction is not part of IEEE P802.11z/D0.1, Draft Standard for Information Technology – Telecommunications and information exchange between systems - Local and metropolitan area networks – Specific requirements - Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications. Amendment 7: Direct Link Setup)

This amendment specifies enhancements to the following draft standard and draft amendments, in order to support direct link setup:

— IEEE P802.11-2007

— IEEE P802.11k (RevCom Mar08)

— IEEE P802.11r (RevCom Jun08)

— IEEE P802.11y (RevCom Jun08),

— IEEE P802.11w (RevCom Mar09)

— IEEE P802.11n (RevCom Jun09),

— IEEE P802.11u (RevCom Sep09),

The IEEE 802.11z PAR includes the following project purpose:

The purpose of this document is to provide amendments to the IEEE 802.11 PHY/MAC layers related to direct link setup.

Notice to users

Errata

Errata, if any, for this and all other standards can be accessed at the following URL: http:// standards.ieee.org/reading/ieee/updates/errata/index.html. Users are encouraged to check this URL for errata periodically.

Interpretations

Current interpretations can be accessed at the following URL: http://standards.ieee.org/reading/ieee/interp /index.html.

Patents

Attention is called to the possibility that implementation of this standard may require use of subject matter covered by patent rights. By publication of this standard, no position is taken with respect to the existence or validity of any patent rights in connection therewith. The IEEE shall not be responsible for identifying patents or patent applications for which a license may be required to implement an IEEE standard or for conducting inquiries into the legal validity or scope of those patents that are brought to its attention. A patent holder or patent applicant has filed a statement of assurance that it will grant licenses under these rights without compensation or under reasonable rates and nondiscriminatory, reasonable terms and conditions to applicants desiring to obtain such licenses. The IEEE makes no representation as to the reasonableness of rates, terms, and conditions of the license agreements offered by patent holders or patent applicants. Further information may be obtained from the IEEE Standards Department.

Participants

At the time this draft amendment to standard was completed, the IEEE 802.11 Working Group had the following membership:

Stuart J. Kerry, Chair

Al Petrick and Harry Worstell, Vice-chair

Stephen McCann, Secretary

EDITORIAL NOTE—a three column list of voting members of 802.11 on the day the draft was sent for sponsor ballot will be inserted

The following were officers of Task Group z:

Menzo Wentink, Chair

Daniel Borges, Secretary

, Technical Editor

Major contributions were received from the following individuals:

EDITORIAL NOTE—a three-column list of individuals who made a major contribution will be inserted.

The following members of the balloting committee voted on this Standard. Balloters may have voted for approval, disapproval, or abstention.

EDITORIAL NOTE—a three-column list of responding sponsor ballot members will be inserted by IEEE staff

Editorial Notes

EDITORIAL NOTE—Two forms of editorial markup are used: Notes and Comments. Editorial Notes and Editorial Comments are not part of the amendment and will be removed before it is published, together with any other contents in this subclause. This paragraph is an example of how an Editorial Note is marked. Editorial Comments are marked (Ed:), and contain references to submissions or comment resolutions to track the origin of changes.

EDITORIAL NOTE—Headings with empty content or Headings preceding editing instructions that modify the contents of the referenced subclause are there to provide context to the reader of this document, they have no other significance.

EDITORIAL NOTE—Except when referring to tables and figures that exist in the baseline, figure and table numbers are preceded by “z” and are assigned sequentially. This will be changed prior to sponsor ballot.

EDITORIAL NOTE—The default IEEE-SA style for tables is to “float”. This means that they be repositioned later, usually at the head of the next page, to avoid splitting the table and reduce the amount of blank space. The table can appear to move out of the subclause it is referenced first from, and can even split a paragraph. This is the intended IEEE-SA behavior, please do not report it as a defect in the draft.

EDITORIAL NOTE—Line numbering is only approximate. This is a limitation of the FrameMaker tool. Whitespace between paragraphs is part of the IEEE-SA style, as defined in their templates. The combination of these two facts leads to the appearance of blank lines in the draft between every paragraph. Please do not report this as an editorial defect as it is the unavoidable behavior.

EDITORIAL NOTE—New subclauses are generally introduced by an editorial instruction “insert the following new subclause”. New subclause headings are generally introduced by an editorial instruction “insert the following new subclause heading”. Each new heading or subclause has its own editorial instruction. The instruction intentionally does not include where to insert the subclause because that is determined uniquely by the subclause number.

Contents

EDITORIAL NOTE—Insert table of contents.

List of figures

EDITORIAL NOTE—Insert list of figures.

List of tables

EDITORIAL NOTE—Insert list of tables.

IEEE  P802.11z™/D0.3

Draft STANDARD for Information Technology- Telecommunications and information exchange between systems- 
Local and metropolitan area networks-
Specific requirements-

Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) specifications

Amendment 7: Direct Link Setup

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

3 Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.z<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert the following new definitions into Clause 3, so as to maintain alphabetic ordering of definitions and renumbering as appropriate:

3.z1 Tunneled Direct Link Setup: Direct Link Setup protocol which uses a specific Ethertype encapsulation to tunnel setup frames through any AP. Power save is supported as well.

3.z2 Peer Power Save Mode (Peer PSM): A power save mode which can be used by STAs which support TDLS. There are two modes for Peer PSM operation, AP mode and client mode.

4 Abbreviations and acronyms

Insert the following new abbreviations and acronyms into Clause 4 so as to maintain alphabetic ordering:

Peer PSM		Peer Power Save Mode

TDLS			Tunneled Direct Link Setup

7 Frame formats

7.1 MAC frame formats

Change 7.1.3.1.6 as shown:

7.1.3.1.6 Power Management field

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

A value of 1 indicates that the STA will be in (Peer) PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

Change 7.1.3.5 as shown:

- 7.1.3.5 QoS Control field

The QoS Control field is a 16-bit field that identifies the TC or TS to which the frame belongs and various other QoS-related information about the frame that varies by frame type and subtype. The QoS Control field is present in all data frames in which the QoS subfield of the Subtype field is set to 1 (see 7.1.3.1.2). Each QoS Control field comprises five subfields, as defined for the particular sender (HC or non-AP STA) and frame type and subtype. The usage of these subfields and the various possible layouts of the QoS Control field are described 7.1.3.5.1 through 7.1.3.5.7 and illustrated in Table 4.

Table 4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS data frames sent by non-AP to an AP | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent over the direct link | TID | EOSP | Ack Policy | Reserved | Reserved |

7.2 Format of individual frame types

7.2.1 Control frames

7.2.2 Data frames

Insert the following new clauses at the end of 7.2.2:

7.2.2.1	TDLS frame format

The frame body of a TDLS frame is defined in Table z1.

|  | LLC/SNAP | Remote Frame Type | TDLS Packet Type | Information |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 1 | 1 | variable |  |

Figure z1—TDLS frame body

The LLC/SNAP field contains the LLC/SNAP header, with Ethertype 89-0d.

The Remote Frame Type field shall be set to 2 (TDLS).

The TDLS Packet Type field specifies the type of the TDLS frame, as specified in Table z1.

The Information field contains information which is specified for each TDLS Type individually.

Table z1—TDLS Packet Type values

| TDLS Type Value | Meaning |
| --- | --- |
| 0 | TDLS Setup Request |
| 1 | TDLS Setup Response |
| 2 | TDLS Setup Confirm |
| 3 | TDLS Teardown Request |
| 4 | TDLS Teardown Response |
| 5 | TDLS DL Path Switch Request |
| 6 | TDLS DL Path Switch Response |
| 7 | TDLS AP Path Switch Request |
| 8 | TDLS AP Path Switch Response |
| 9 | Peer Traffic Indication |
| 10 | TDLS Channel Switch Request |
| 11 | TDLS Channel Switch Response |
| 12 – 255 | reserved |

7.2.2.1.1 TDLS Setup Request frame format

The TDLS Setup Request frame contains the information shown in Table z2.

Table z2—Information for TDLS Setup Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Association Request frame body | The Association Request frame body is specified in 7.2.3.4. |
| 3 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 4 | RSNIE_I | RSNIE for Initiator STA (optional) |
| 5 | SMK Message 1 FTIE | SMK Message 1 FTIE (optional) |
| 6 | DH_I | Public value for Initiator STA (optional) |
| 7 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in TGy 7.3.2.51 (optional) |

The TDLS Setup Request frame shall be sent through the AP.

7.2.2.1.2 TDLS Setup Response frame format

The TDLS Setup Response frame contains the information shown in Table z3.

Table z3—Information for TDLS Setup Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Status Code | The Status Code is defined in 7.3.1.9. |
| 3 | Association Request frame body | The Association Request frame body is specified in 7.2.3.4. Only present for Status Code 0 (Successful). |
| 4 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Request. |
| 5 | RSNIE_P | RSNIE of Peer STA (optional) |
| 6 | SMK Message 2 FTIE | SMK Message 2 FTIE (optional) |
| 7 | DH_P | Public value for Peer STA (optional) |
| 8 | Supported Regulatory Classes | The Supported Regulatory Classes element is defined in TGy 7.3.2.51 (optional) |

The TDLS Setup Response frame shall be sent through the AP.

7.2.2.1.3 TDLS Setup Confirm frame format

The TDLS Setup Response frame contains the information shown in Table z4.

Table z4—Information for TDLS Setup Confirm frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Response. |
| 3 | SMK Message 3 FTIE | SMK Message 3 FTIE (optional) |

The TDLS Setup Confirm frame shall be sent through the AP.

7.2.2.1.4 TDLS Teardown Request frame format

The TDLS Teardown frame contains the information shown in Table z5.

Table z5—Information for TDLS Teardown Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 3 | Dialog Token | The Dialog Token contains a unique value for this conversation. |

The TDLS Teardown Request frame shall be sent through the AP.

7.2.2.1.5 TDLS Teardown Response frame format

The TDLS Teardown frame contains the information shown in Table z6.

Table z6—Information for TDLS Teardown Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Teardown Request. |

The TDLS Teardown Response frame shall be sent through the AP.

7.2.2.1.6 TDLS DL Path Switch Request frame format

The TDLS DL Path Switch Request frame contains the information shown in Table z7.

Table z7—Information for TDLS DL Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 3 | Reason | The Reason field (1-octet) indicates the reason to send the path switch request frame. The reason field is set to one of the values in Table z8. |

The TDLS DL Path Switch Request frame shall be sent through the AP.

Table z8—Reason for the Path Switch Request frame

| Reason Value | Description |
| --- | --- |
| 0 | Unspecified |
| 1 | Change in power save mode |
| 2 | Change in link state |
| 3-255 | Reserved |

7.2.2.1.7 TDLS DL Path Switch Response frame format

The TDLS DL Path Switch Response frame contains the information shown in Table z9.

Table z9—Information for TDLS DL Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Suspend frame. |
| 3 | Result | The Result field (1-octet) indicates the result of the Path Switch request and is set to one of values in Table z10. |

The TDLS DL Path Switch Response frame shall be sent through the AP.

Table z10—Result for the DL Path Switch Response frame

| Result Value | Description |
| --- | --- |
| 0 | Accept |
| 1 | Reject because of entering power save mode |
| 2 | Reject because of the link status |
| 3 | Reject because of unspecified reason |
| 4-255 | Reserved |

7.2.2.1.8 TDLS AP Path Switch Request frame format

The TDLS AP Path Switch Request frame contains the information shown in Table z11.

Table z11—Information for AP TDLS Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token contains a unique value for this conversation. |
| 3 | Reason | The Reason field (1-octet) indicates the reason to send the path switch request frame. The reason field is set to one of the values in Table z8. |

The TDLS AP Path Switch frame shall be sent through the AP.

7.2.2.1.9 TDLS AP Path Switch Response frame format

The TDLS AP Path Switch Response frame contains the information shown in Table z12.

Table z12—Information for TDLS AP Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Path Switch Request frame. |

The TDLS AP Path Switch Response frame shall be sent through the AP.

Insert a new section after 7.2.2.1.9:

7.2.2.1.10 Peer Traffic Indication

The Peer Traffic Indication frame contains the information shown in Table z13.

Table z13—Peer Traffic Indication

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 2 | AC0 backlogged | 1 octet field which is zero if the AC0 is empty and is non-zero otherwise |
| 3 | AC1 backlogged | 1 octet field which is zero if the AC1 is empty and is non-zero otherwise |
| 4 | AC2 backlogged | 1 octet field which is zero if the AC2 is empty and is non-zero otherwise |
| 5 | AC3 backlogged | 1 octet field which is zero if the AC3 is empty and is non-zero otherwise |
| 6 | Peer PSM Indication Window | 1 octet field which indicates the minimum interval between successive Peer Traffic Indication frames sent to the same peer, expressed in Beacon Intervals. (Optional) |

The Peer Traffic Indication frame indicates the power save buffer state at the STA buffering data for a peer STA in Peer PSM client mode.

Each of the ACs backlogged field is 1 octet in length. It is set to zero value if the corresponding AC (AC0, AC1, AC2, AC3) is empty and to a non-zero value otherwise.

Add two new clauses 7.2.2.1.11 and 7.2.2.1.12 as follows:

### 7.2.2.1.11 TDLS Channel Switch Request frame format

The format of the TDLS Channel Switch Request frame contains the information shown in Table z14.

Table z14—Information for TDLS Channel Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Target Channel | 1-octet field which specifies the channel number of the target channel. |
| 2 | Regulatory Class | 1-octet field which specifies the regulatory class for the target channel. |

The TDLS Channel Switch Request frame shall be sent over the direct path. Switching back to the base channel shall always be accepted. The regulatory class for the base channel shall correspond to the regulatory class of the BSS to which the stations are associated.

### 7.2.2.1.12 TDLS Channel Switch Response frame format

The TDLS Channel Switch Request frame contains the information shown in Table z15.

Table z15—Information for TDLS Channel Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Result Code | 1-octet field which contains an integer which is set to 0 to indicate that the switch request is accepted and set to 1 if the switch request is rejected. |

The TDLS Channel Switch Response frame shall be sent over the direct path.

7.3 Management frame body components

7.3.1 Fields that are not information elements

7.3.2 Information elements

In table 26, add the following New Information elements, and renumber the reserved values accordingly:

Table 26—Element IDs

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Link Identifier (see 7.3.2.z1) | z1 | 20 |
| DH_I (see 7.3.2.z2) | z2 | 192 |
| DH_P (see 7.3.2.z3) | z3 | 192 |

7.3.2.21 Measurement Request element

In P802.11k D7.0, clause 7.3.2.21, Table 29, insert a new measurement request named Link RCPI Request, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, renumber clause 7.3.2.21.11 into 7.3.2.21.12, and insert a new clause 7.3.2.21.11 as follows:

7.3.2.21.11 Link RCPI Request

The Measurement Request field corresponding to a Link RCPI Request is shown Figure z2.

|  | BSSID | STA Address |  |
| --- | --- | --- | --- |
| Octets: | 6 | 6 |  |

Figure z2—Measurement Request field for a Link RCPI Request

BSSID indicates the BSSID.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

7.3.2.22 Measurement Report element

In P802.11k D7.0, clause 7.3.2.22, Table 30, insert a new measurement report named  Link RCPI Report, with Measurement Type 9, in the Radio Measurement category, and renumber the reserved Measurement Types accordingly.

In P802.11k D7.0, insert a new clause 7.3.2.22.11 as follows:

7.3.2.22.11 Link RCPI Report

The format of the Measurement Report field corresponding to a Link RCPI Report is shown in Figure z3.

|  | BSSID | RCPI 1 | STA Address | RCPI 2 |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 1 | 6 | 1 |  |

Figure z3—Measurement Report field for a Link RCPI Report

BSSID indicates the BSSID.

RCPI 1 indicates the RCPI on frames received from the AP. The RCPI field is defined in 802.11k clause 17.3.10.6.

STA Address indicates the MAC address of the STA requesting the Link RCPI measurement.

RCPI 2 indicates the RCPI on frames received from the STA requesting the Link RCPI measurement. The RCPI field is defined in 802.11k clause 17.3.10.6.

7.3.2.25 RSN information element

7.3.2.25.2 AKM suites

Insert the following new entry in Table 34 and update the reserved values accordingly:

Table 34—AKM suite selectors

| OUI | Suite type | Authentication type | Key management type |
| --- | --- | --- | --- |
| 00-0F-AC | <ANA> | N/A | SMK Handshake |

Change 7.3.2.27 as shown:

- 7.3.2.27 Extended Capabilities information element

Add a row to Table 35a as follows and renumber the reserved values accordingly:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Peer PSM AP Mode | The Peer PSM AP mode capability bit set to 1 indicates that the STA supports Peer PSM AP mode as described in 11.2.1.12. The Peer PSM AP mode capability bit set to 0 indicates that the STA does not support this capability. |
| <ANA> | TDLS Channel Switching | The TDLS Channel Switching capability bit set to 1 indicates the STA supports Tunneled Direct Link Setup (TDLS) with Channel Switching as described in 11.z1. The TDLS Channel Switching bit set to 0 indicates that the STA does not support this capability. |

7.3.2.46 Fast BSS transition information element

Add the following new entries to table 7-43d of 802.11r-D7.0:

Table 7-43d—Sub-element IDs

| Value | Contents of data field | Length (in octets) |
| --- | --- | --- |
| 4 | MAC_I | 6 |
| 5 | MAC_P | 6 |
| 6 | BSSID | 6 |
| 8-255 | Reserved |  |

MAC_I contains the MAC address of the Initiator STA.

MAC_P contains the MAC address of the Peer STA.

BSSID contains the BSSID.

Lifetime contains the lifetime of the SMKSA in seconds.

Insert the following new clauses after the last subclause of 7.3.2:

7.3.2.z1 Link Identifier element

The Link Identifier element contains information which identifies the direct link. The element information format is defined in Figure z4.

|  | Element ID | Length | BSSID | Source Address | Destination Address | Regulatory Class | Channel Number |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 6 | 6 | 1 | 1 |  |

Figure z4— Link Identifier element format

The Length field shall be set to 20.

The BSSID field shall be set to the BSSID of the STAs current association.

The Source Address field shall be set to the transmitting STAs MAC address.

The Destination Address field shall be set to the MAC address of the destination STA.

The Regulatory Class field shall be set to the Regulatory Class of the STAs current association.

The Channel Number field shall be set to the channel number of the STAs current association.

Figure z5—Path element format

The Length field shall be set to 1.

The Path field is set to 0 for the AP path and to 1 for the direct link path.

7.3.2.z2 DH_I element

The DH_I element contains the public value of the Initiator STA of a TDLS direct link. The element information format is defined in Figure z6.

|  | Element ID | Length | DH_I |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 192 |  |

Figure z6—DH_I element format

The Length field shall be set to 192.

The DH_I field is set to the public value of the Initiator STA.

7.3.2.z3 DH_P element

The DH_P element contains the public value of the Peer STA of a TDLS direct link. The element information format is defined in Figure z7.

|  | Element ID | Length | DH_P |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 192 |  |

Figure z7—DH_P element format

The Length field shall be set to 192.

The DH_P field is set to the public value of the Peer STA.

8 Security

Add a new clause 8.5.9 after clause 8.5.8 as follows:

8.5.9 TDLS Peer Key Handshake

The TDLS Peer Key Handshake occurs after any other direct link setup procedures and is used to create an STKSA providing data confidentiality between the two STAs. The AP must establish an RSNA with each STA prior to PeerKey setup. The initiator STA starts the PeerKey Handshake, providing the key to use for securing the connection.

The TDLS Peer Key EAPOL-Key exchange provides a mechanism for obtaining the keys to be used for direct STA-to-STA communication. The initiator STA shall start a timer when it sends the first EAPOL-Key message and the peer STA shall do the same on receipt of the first EAPOL-Key message. On expiration of this timer, the STA shall transition to the STKINIT state.

A STA should use the TDLS Peer Key Handshake prior to transferring any direct STA-to-STA data frames. The STKSA should be deleted when the STA to STA connection is terminated.

The following assumptions apply:

- FTIE denotes an Fast BSS Transition information element as specified in Clause 7.3.2.46 of 802.11r-D7.0.

- STA_I is the initiator STA

- STA_P is the peer STA

- AP is the access point with which both the STA_I and the STA_P are associated

- MAC_I is the MAC address of STA_I

- MAC_P is the MAC address of STA_I

- INonce is the nonce generated by STA_I

- PNonce is the nonce generated by STA_P

- DH_I is the public value of STA_I

- DH_P is the public value of STA_P

The TDLS Peer Key Handshake has two components:

- SMK Handshake: This handshake is initiated by the initiator STA and, as a result of this handshake, the SMKSA is installed in both the STAs. This message exchange goes through the AP and is protected using the PTK.

- 4-Way STK Handshake: Using the installed SMKSA, the initiator STA initiates the 4-Way Handshake (as per 8.5.3.4) and, as a result of this, the STKSA gets installed in both the STAs. The STKSA is used for securing data exchange between the initiator STA and the peer STA. This message exchange goes directly between the peer STAs.

8.5.9.1 SMK Handshake

The Initiator STA initiates the SMK Handshake by sending first message to the Peer STA through the AP path. This is done to establish a SMKSA between Imitator and Peer STA associated with the same AP. Unlike the 4-Way Handshake and Group Key Handshake, the SMK Handshake is initiated by the initiator STA.

For SMK Handshake, the modulus p shall be 1536 bits (as per RFC 3526) in length and the generating element g shall be 2.

The prime is: 2^1536 - 2^1472 - 1 + 2^64 * { [2^1406 pi] + 741804 }. Its hexadecimal value is:

FFFFFFFF FFFFFFFF C90FDAA2 2168C234 C4C6628B 80DC1CD1

29024E08 8A67CC74 020BBEA6 3B139B22 514A0879 8E3404DD

EF9519B3 CD3A431B 302B0A6D F25F1437 4FE1356D 6D51C245

E485B576 625E7EC6 F44C42E9 A637ED6B 0BFF5CB6 F406B7ED

EE386BFB 5A899FA5 AE9F2411 7C4B1FE6 49286651 ECE45B3D

C2007CB8 A163BF05 98DA4836 1C55D39A 69163FA8 FD24CF5F

83655D23 DCA3AD96 1C62F356 208552BB 9ED52907 7096966D

670C354E 4ABC9804 F1746C08 CA237327 FFFFFFFF FFFFFFFF

The information flow of the SMK handshake is as follows:

SMK Message 1:	Initiator STA → Peer STA

RSNIE_I, FTIE(0, 0, 0, INonce, MAC_I, BSSID, Lifetime, DH_I)

SMK Message 2:	Peer STA → Initiator STA

RSNIE_P, FTIE(7, MIC, PNonce, INonce, MAC_I, MAC_P, BSSID, Lifetime, DH_P)

SMK Message 3:	Initiator STA → Peer STA

RSNIE_I, FTIE(4, MIC, PNonce, INonce, MAC_I, MAC_P, BSSID, Lifetime, DH_I)

8.5.9.1.1 SMK Handshake Message 1

The Initiator STA performs following steps before generating Message 1:

- Create RSNIE by filling the element id (fixed hex 30), length, version (1), and pairwise cipher suite list field. Since group cipher suit field is before pairwise cipher suite list field (so STA needs to fill it), STA shall fill this field with any value and on the other side STA processing this field shall ignore it.

- Generate 256 bit random number which is sent in Key Nonce field.

- Decides on lifetime of SMKSA, which is sent as part of Lifetime.

- Initiator STA selects a random integer A < p and computes public DH_I = gA mod p.

The initiator STA sends Message 1 to the peer STA P through AP. After sending the message 1, the Initiator STA starts a timer (different from the Lifetime-Timer) and waits for response message from the Peer STA.

On receipt of Message 1, the peer STA performs following actions:

- Verify the initiator MAC address against existing direct link. If no direct link exists, it silently discards the message.

- If all checks succeed,

  - Generate 256 bit random number which is sent in Key Nonce field.

  - Check the suggested lifetime value proposed by initiator STA. If needed select a smaller lifetime value or use the same lifetime value to create the Lifetime to be used in message 2.

  - Peer STA selects a random integer B < p and computes public DH_P = gB mod p.

  - Peer STA computes the SMK as

SMK-Key-Data = KDF-384(SHA-256(DH_IB mod p), "SMK Key Derivation", BSSID || MAC_I || MAC_P || INonce || PNonce)

SMK-KCK = L(SMK-Key-Data, 0, 128)

SMK = L(SMK-Key-Data, 128, 256)

where

—   KDF-384 is the KDF function as defined in 8.5.1.5.2 used to generate a key of length 384 bits.

—   L(-) is defined in 8.5.1.

—   "SMK Key Derivation" is 0x534d4b204b65792044657269766174696f6e.

—   BSSID is the basic service set identification as defined in 7.1.3.3.3.

  - Peer STA selects one of the ciphersuites from the ciphersuite list in the initiator RSNIE and creates peer RSNIE.

- Using all the information, peer STA creates Message 2 and sends it to initiator STA through AP.

8.5.9.1.2 SMK Handshake Message 2

The peer STA sends Message 2 to the initiator STA through the AP path. After sending Message 2, the Peer STA starts a timer (different from the Lifetime-Timer) and waits for response message from the Initiating STA.

On receipt of Message 2, the Initiator STA performs the following actions:

- Verify the Peer MAC address against existing direct link. If no direct link exists, it silently discards the message.

- Verify the Initiator MAC address and INonce from the FTIE and if it does not match, the Initiator STA silently discards the message.

- If all checks succeed,

  - The Initiator STA computes the SMK as

SMK-Key-Data = KDF-384(SHA-256(DH_PA mod p), "SMK Key Derivation", BSSID || MAC_I || MAC_P || INonce || PNonce)

SMK-KCK = L(SMK-Key-Data, 0, 128)

SMK = L(SMK-Key-Data, 128, 256)

- Verify the MIC on Message 2.  If the MIC verification fails, the Initiator STA silently discards the message. The MIC shall be calculated using the SMK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits. The MIC shall be calculated on the concatenation, in the following order, of:

- MAC_I

- MAC_P

- BSSID

- RSNIE_P

- PNonce

- INonce

- Lifetime

- DH_P

- If the MIC verification succeeds,  the Initiator STA creates Message 3 and sends it to the Peer STA through the AP path.

8.5.9.1.3 SMK Handshake Message 3

The Initiator STA sends Message 3 to the Peer STA through the AP path. After sending Message 3, the SMK handshake is complete from initiator STA side.

On reception of Message 3, the peer STA performs following actions:

- Verify the Initiator MAC address against existing direct link. If no direct link exists, silently discard the message.

- Verify the Peer MAC address and PNonce from the FTIE and if either does not match, silently discard the message.

- Verify the MIC on Message 3.  If the MIC verification fails, the Peer STA silently discards the message. . The MIC shall be calculated using the SMK-KCK and the AES-128-CMAC algorithm. The output of the AES-128-CMAC shall be 128 bits. The MIC shall be calculated on the concatenation, in the following order, of:

- MAC_I

- MAC_P

- BSSID

- RSNIE_I

- PNonce

- INonce

- Lifetime

- DH_I

- If all checks succeed, the SMK handshake is complete from the Peer STA side.

8.5.9.2 SMK handshake error conditions

If the STA_P does not receive a valid SMK Message 3 or a 4-Way STK Message 1 after sending the EAPOL request message to initiate the TDLS Peer Key rekey within a 2000 millisecond timeout, the STA_P shall invoke a direct link teardown procedure.

If the STA_I does not receive an SMK Message 2 from the STA_P, the STA_I shall attempt dot11RSNAConfigSMKUpdateCount transmits of the SMK Handshake Message 1 plus a final timeout. If the STA_I still has not received a response after these retries, it shall invoke a direct link teardown procedure. The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, then 200 milliseconds shall be used for all timeout values.

Upon receipt of the SMK Message 3, the STA_I will transmit Message 1 of the 4-Way STK Handshake to the STA_P. If the STA_I does not receive Message 2 of the 4-Way STK Handshake from the STA_P, it shall attempt dot11RSNAConfigSMKUpdateCount transmits of 4-Way STK Handshake Message 1, plus a final timeout. If STA_I still has not received a response after these retries, it shall invoke a direct link teardown procedure. The retransmit timeout value shall be 1000 milliseconds for the first timeout, half the listen interval for the second timeout, and the listen interval for subsequent timeouts. If there is no listen interval, then 1000 milliseconds shall be used for all timeout values.

8.5.9.3 STKSA rekeying

Rekeying is always initiated by the STA_I. When needed, the STA_P sends an EAPOL request message to the STA_I to request rekeying. The STA_P shall wait a minimum of one half the IEEE 802.1X timeout after the STSL setup before initiating a TDLS Peer Key rekey procedure. To perform rekeying, there are two cases:

1) If the SMK timer has not expired, the STAs will initiate a 4-Way Handshake to create a new STK. The 4-Way Handshake is always initiated by STA_I. In this case, STA_P should not delete any existing STKSA prior to verifying Message 3 of the 4-Way Handshake with STA_I for this session.

2) If the SMK has expired, STA_I shall not use an existing STKSA and shall start the SMK Handshake followed by a 4-Way Handshake to create new keys.

The format of the EAPOL-Key request message in case 1 from STA_P to STA_I is as follows:

Request Message:	STA_P → STA_I

EAPOL-Key(1,1,0,0,1,0,0,0, MIC, PMKID KDE)

The request message uses the following values for each of the EAPOL-Key frame fields:

Descriptor Type = N - see 8.5.2

Key Information:

Key Descriptor Version = 1 (RC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128)

Key Type = 1 (PTK)

SMK Message = 0

Install = 0

Key Ack = 0

Key MIC = 1

Secure = 1

Error = 0

Request = 1

Encrypted Key Data = 0

Reserved = 0

Key Length = 0

Key Replay Counter = request replay counter of peer STA

Key Nonce = 0

EAPOL-Key IV = 0

Key RSC = 0

Key MIC = MIC computed over the body of this EAPOL-Key frame

Key Data Length = Length of Key Data field in octets

Key Data = SMKID in SMKID KDE

8.5.9.4 Error Reporting

Error reporting messages are defined in this sub-clause and used to report errors whenever STAs detect an error during the SMK Handshake.

The STA, upon receipt of the error messages defined in this sub-clause, shall tear down the direct link with the other STA and clear all the TDLS Peer Key states.

The format of EAPOL-Key request message for reporting an error message is as follows:

Error Message: EAPOL-Key(1,1,0,0,0,1,0, 0, MIC, Error KDE, MAC Address KDE).

The request message uses the following values for each of the EAPOL-Key frame fields:

Descriptor Type = N - see 8.5.2

Key Information:

Key Descriptor Version = 1 (RC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128)

Key Type = 0 (Group/SMK)

SMK Message = 1 (SMK)

Install = 0

Key Ack = 0

Key MIC = 1

Secure = 1

Error = 1

Request = 1

Encrypted Key Data = 0

Reserved = 0

Key Length = 0

Key Replay Counter = request EAPOL replay counter

Key Nonce = 0

EAPOL-Key IV = 0

Key RSC = 0

Key MIC = MIC computed over the body of this EAPOL-Key frame

Key Data Length = Length of Key Data field in octets

Key Data = Error KDE (different types defined in Table 64), MAC Address KDE

8.5.9.4.1 Error ERR_CPHR_NS

This error message is sent whenever a STA finds that it does not support any of the ciphersuites proposed by the other STA. In response to this error, the STA creates an Error KDE with error type ERR_CPHR_NS and sends the message back to the other STA. The MAC address KDE contains the MAC address of the other

STA.

8.5.8.4.4 Error ERR_NO_STSL

This error message is sent whenever a STA finds that it does not have an existing direct link with the other STA. In response of this error, the STA creates an Error KDE with error type ERR_NO_STSL and sends the message back to the other STA. The MAC address KDE contains the MAC address of the other STA.

10 Layer management

10.3 MLME SAP interface

Insert a new subclause at the end of clause 10.3 as follows:

10.3.z1 Management of tunneled direct link setup

The following MLME primitives support the signaling of tunneled direct link setup. Figure z8 depicts the TDSL setup process. The figure illustrates the basic protocol, which is only an example and is not meant to be exhaustive of all possible protocol uses.

Figure z8—Tunneled Direct Link Setup Protocol Exchange

EDITORIAL NOTE—Add TDLS Setup Confirm.

10.3.z1.1 MLME-TDLSSETUPREQUEST.request

10.3.z1.1.1 Function

This primitive requests that a TDLS Setup Request frame be sent to the candidate peer STA.

10.3.z1.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.request(

PeerSTAAddress,

TDLSSetupRequest

TDLSRequestTimeout

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TDLS Setup process. |
| TDLSSetupRequest | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |
| TDLSRequestTimeout | Integer | ≥ 0 | Specifies a time limit (in TU) after which the TDLS Setup procedure is terminated. |

10.3.z1.1.3 When Generated

This primitive is generated by the SME to request that a TDLS Setup Request frame be sent to the candidate peer STA.

10.3.z1.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Setup Request action management frame. The STA then attempts to transmit this to the candidate peer STA.

10.3.z1.2 MLME-TDLSSETUPREQUEST.confirm

10.3.z1.2.1 Function

This primitive reports the result of a TDLS Setup procedure.

10.3.z1.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.confirm(

TDLSSetupResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSSetup- Response | As defined in TDLS Setup Response element | As defined in TDLS Setup Response element | Specifies service parameters for the TDLS Setup. |

10.3.z1.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-TDLSSETUP.request and indicates the results of the request.

This primitive is generated when the MLME-TDLSSETUP.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a TDLS Setup Response frame from the AP.

10.3.z1.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

10.3.z1.3 MLME-TDLSSETUPREQUEST.indication

10.3.z1.3.1 Function

This primitive indicates that a TDLS Setup Request frame was received from a candidate peer STA.

10.3.z1.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUP.indication(

PeerSTAAddress,

TDLSSetupRequest

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TDLS Setup Request frame was received. |
| TDLSSetup- Request | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |

10.3.z1.3.3 When Generated

This primitive is generated by the MLME when a valid TDLS Setup Request frame is received.

10.3.z1.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.z1.

10.3.z1.4 MLME-TDLSSETUPRESPONSE.request

10.3.z1.4.1 Function

This primitive requests that a TDLS Setup Request frame be sent to the initiating STA.

10.3.z1.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.request(

PeerSTAAddress,

TDLSSetupRequest

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TDLS Setup process. |
| TDLSSetupRequest | As defined in TDLS Setup Request element | As defined in TDLS Setup Request element | Specifies the proposed service parameters for the TDLS Setup. |
| ResultCode | Enumeration | SUCCESS, REFUSED, INVALID_ PARAMETERS, TIMEOUT |  |

10.3.z1.4.3 When Generated

This primitive is generated by the SME to request that a TDLS Setup Response frame be sent to the initiating STA.

10.3.z1.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a TDLS Setup Response action management frame. The STA then attempts to transmit this to the initiating STA.

10.3.z1.5 MLME-TDLSSETUPRESPONSE.confirm

10.3.z1.5.1 Function

This primitive reports the result of a TDLS Setup procedure.

10.3.z1.5.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.confirm(

TDLSSetupResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TDLSSetup- Response | As defined in TDLS Setup Response element | As defined in TDLS Setup Response element | Specifies service parameters for the TDLS Setup. |

10.3.z1.5.3 When Generated

This primitive is generated by the MLME as a result of an MLME-TDLSSETUP.request and indicates the results of the request.

This primitive is generated when the MLME-TDLSSETUP.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a TDLS Setup Response frame from the AP.

10.3.z1.5.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the Element Status and may use the reported data.

10.3.z1.6 MLME-TDLSSETUPRESPONSE.indication

10.3.z1.6.1 Function

This primitive indicates that a TDLS Setup Request frame was received from a candidate peer STA.

10.3.z1.6.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TDLSSETUPRESPONSE.indication(

PeerSTAAddress,

TDLSSetupResponse

ResultCode

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a TDLS Setup Request frame was received. |
| TDLSSetup- Request | As defined in TDLS Setup Request element | As defined in TDLS Setup Response element | Specifies the proposed service parameters for the TDLS Setup. |
| ResultCode | Enumeration | SUCCESS, REFUSED, INVALID_ PARAMETERS, TIMEOUT |  |

10.3.z1.6.3 When Generated

This primitive is generated by the MLME when a valid TDLS Setup Response frame is received.

10.3.z1.6.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.z1.

Add TDLS Teardown

11 MLME

Insert the following after 11.2.1.11:

11.2.1.12 Power Management with TDLS

Peer STAs may be capable to use the direct link for data transmission when one of them is in peer power save mode. Support for Peer PSM and operation in Peer PSM are two different optional capabilities, Peer PSM AP mode and Peer PSM client mode, correspondingly.

When a peer of a STA supports the Peer PSM AP mode, a STA supporting the Peer PSM client mode can enter the Peer PSM by sending a frame with the power management bit set. Such a STA is called a STA in Peer PSM client mode. The Peer PSM client mode may be supported when dot11PeerPSMClientModeImplemented is true.

When a peer of a STA is in the Peer PSM, the STA shall buffer MSDUs for the peer locally without sending them to the access point and deliver the buffered MSDUs over the direct link. Such a STA is referred to as a STA in Peer PSM AP mode. The Peer PSM AP mode may be supported when dot11PeerPSMAPModeImplemented is true. A STA which supports Peer PSM AP mode shall signal this capability through the use of the Peer PSM AP mode subfield in the Extended Capabilities Information field in TDLS Setup Request and Response frames. The STA in Peer PSM AP mode shall be in active mode.

The behavior of a STA in Peer PSM AP mode with respect to traffic buffered for a peer STA in client mode is the same as a regular U-APSD AP, with the following exception. When an AC becomes backlogged and no Service Period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in AP mode sends a unicast Peer Traffic Indication frame to the peer STA in client mode through the access point, indicating the backlogged AC(s).

The behavior of a STA in Peer PSM client mode with respect to traffic buffered at the peer STA in AP mode is the same as a regular U-APSD STA, with the following exception. The indication that traffic is buffered at the peer STA in AP mode is obtained from received Peer Traffic Indication frames from the peer STA in AP mode. To reduce the number of Peer Traffic Indication frames transmitted for a continuous uni-directional traffic stream without return traffic, a new Service Period may be started within Peer PSM Indication Window after the occurrence of a Service Interval during which a Data MPDU was received.

A STA in Peer PSM client mode may configure a STA in Peer PSM AP mode to use U-APSD using two methods. First, a STA in Peer PSM client mode may set individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in TDLS Setup Request and Response frames. Alternatively, a STA in Peer PSM client mode may designate one or more AC as trigger-enabled and one or more AC as delivery-enabled by sending an ADDTS Request frame per AC to the STA in Peer PSM AP mode.

The Peer PSM Indication Window field is optionally present in the Peer Traffic Indication frame. The default Peer PSM Indication Window is equal to DTIM interval.

Insert a new subclause at the end of clause 11 as follows:

11.z1 Tunneled Direct Link Setup

Tunneled Direct Link Setup (TDLS) is characterized by the fact that the signaling frames are encapsulated in Data frames, which allows them to be transmitted through any access point transparently. Therefore, a direct link can be setup using any access point. The access point does not need to be direct link aware, nor does it have to support any of the capabilities which will be used on the direct link. TDLS also includes an option either to enter Peer Power Save Mode (Peer PSM) remaining on the direct link or to suspend receiving over the direct link, so that the station can enter a power save mode.

A STA may transmit a Link RCPI Measurement Request to an (intended) peer STA to obtain an indication of the RCPI values at the peer STA. The Link RCPI measurement request and report are sent to the peer STA directly. The RCPI information may be used to decide whether to switch over to a direct link for communication with the peer STA.

To setup a direct link, the initiator STA sends a TDLS Setup Request to the intended peer STA. If the peer STA accepts the direct link, it responds with a TDLS Setup Response frame with status code 0 (Successful). If the peer STA does not accept the direct link, it responds with a TDLS Setup Response with a status code other than 0. If there is no response within the set timeout, the initiator STA should conclude that the intended peer STA does not support TDLS and the setup procedure is terminated. The initiator then sends a TDLS Setup Confirm to the peer STA to confirm the receipt of the TDLS Setup Response. RSNIE, SMK Message FTIE and DH IE shall not be included in a TDLS setup message when the STA has no RSNA with the AP. If security is required, the TDLS setup messages shall include the SMK handshake. When the TDLS Setup Handshake has been completed, both STAs shall accept frames received over the direct link.

After a successful response, the initiating STA further prepares the direct link for Data transmissions by starting the 4-way Handshake.

After transmitting a last Data frame through the AP path and before transmitting the first Data frame over the direct link, a STA may send a TDLS DL Path Switch Request. The first Data frame transmitted over the direct link should be transmitted after receipt of the TDLS DL Path Switch Response in this case, possibly after additional delay to account for potential EDCA related reordering inside the AP. This avoids potential reordering of frames between the AP path and the direct link. The STA may also use a message exchange which is part of the direct link setup or the peer key handshake for this purpose.

When a STA intends to disable its direct Rx path, for instance to suspend receiving over the direct link, it sends a TDLS AP Path Switch Request with the appropriate reason code to the peer STA. Upon receipt of a TDLS AP Path Switch Request, the receiving STA shall cease transmissions over the direct link as soon as possible. When no further Data frames will be transmitted over the direct link, the responding STA shall send a TDLS AP Path Switch Response. The requesting STA may enter a power save mode after receiving the TDLS DL Path Switch Response.

A STA may request a peer STA to enable its direct Rx path by sending a TDLS DL Path Switch Request, with the appropriate reason code. Upon receipt of the TDLS DL Path Switch Request, the receiving STA may enable the direct Rx path and respond with a TDLS DL Path Switch Response with the appropriate reason code. The requesting station shall not transmit frames on the direct link before receiving a TDLS DL Path Switch Response indicating that the Rx path was enabled (reason code 0).

When a STA intends to enter a power save mode, it may suspend the direct link by shutting down the direct Rx path, or enter the Peer PSM by sending a frame with the power management bit set. See 11.2.1.12.

To tear down a direct link, the STA sends a TDLS Teardown Request to the peer STA, after which the STA shall not transmit on the direct link any longer. Upon receipt of the TDLS Teardown Request, the peer STA shall disable the direct Rx and Tx paths and destroy the related security parameters, and then respond with a TDLS Teardown Response. Upon receipt of the TDLS Teardown Response, the STA which initiated the teardown shall disable the direct Rx path and destroy the related security parameters.

If channel switching is supported (dot11TDLSChannelSwitchingImplemented is set to True), the STA sets the TDLS Channel Switching capability bit to 1 and includes a Supported Regulatory Classes element in TDLS Setup Request and Response frames.

The target channel is determined by the STA which initiates a channel switch, from the set of regulatory classes supported by both peer STAs. The target channel and regulatory class are specified in the TDLS Channel Switch Request frame. The Country and Coverage Class settings on the target channel are the same as in the BSS to which both peer STAs are currently associated. Both STAs are entitled to request for a channel switch.

The events occurring for a channel switch are:

- backoff – TDLS Channel Switch Request – SIFS – ACK1 – […] – backoff – TDLS Channel Switch Response – SIFS – ACK2 – SwitchTime | ProbeTime – backoff – first transmission

Both STAs shall be listening on the new channel not later than SwitchTime after the end of ACK2 (this instance is marked as “|” in the above frame exchange sequence). After switching to the new channel, a STA shall perform a clear channel assessment (CCA) until a frame sequence is detected by which it can correctly set its NAV, or until a period of time equal to at least ProbeTime has transpired (this combined event is indicated as “ProbeTime” in the above frame exchange sequence). A first transmission on the new channel shall be preceded by a random backoff, which shall start at the end of the ProbeTime. If no successful frame exchange has occurred on the off-channel within SwitchTimeout after the end of ACK2, the stations shall go back to the base channel. The values for SwitchTime, ProbeTime and SwitchTimeout are specified in Table z12.

Table z12—Parameter Values

| Parameter | Value |
| --- | --- |
| SwitchTime | ≤ TBD us |
| ProbeTime | ≥ TBD us |
| SwitchTimeout | TDB us |

Before switching to an off-channel (i.e. a channel which is not the base channel), the stations shall be in PS mode with the AP. To this end, a STA which receives a TDLS Channel Switch Request may enter PS mode with the AP prior to sending the TDLS Channel Switch Response. This possible event is indicated with “[…]” in the above frame exchange sequence.

Because there is a backoff between the request and response, there is a (small) probability that two stations issue a TDLS Channel Switch Request at more or less the same time. To reduce the probability for this event to occur, a pending TDLS Channel Switch Request should be cancelled when a TDLS Channel Switch Request is received. When two TDLS Channel Switch Requests still cross, then both TDLS Channel Switch Responses will be executed sequentially if they contain result code 0 (accepted). If a TDLS Channel Switch Response does not imply a channel switch (because the stations already are on the requested channel), then there is no prior switch and ProbeTime and both stations simply continue what they were doing on the channel.

If the base channel is a DFS channel, then STAs should take care to reside on the base channel enough to receive channel switch announcements by the AP.

On the off-channel, the channel access rules which govern the off-channel shall be observed at any time, including possible DFS, TPC and radar scanning requirements.

Switching back to the base channel shall always be accepted, because the base channel is the only channel at which communications with the AP can take place.

A TDLS Path Switch Request frame and a TDLS Teardown Request frame shall not be transmitted on the off-channel. This avoids that such frames are implicitly also a Channel Switch request, which would potentially complicate their behavior.

TDLS channel switching shall not be invoked for coverage classes other than 0. This effectively limits channel switching to indoor use only.

The off-channel shall be an empty channel. Once beacon transmissions are detected on the off-channel, it shall be changed. The latest scan performed by both peer stations of an intended off-channel shall not be older than 5 minutes.

When active on an off-channel, peer stations shall respond to received Probe Requests with a Probe Response indicating ‘off-channel' as the SSID. The timestamp may contain a random value, while other contents of the Probe Response may be copied from the APs Beacon or Probe Response.

Annex A

A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

A.4.3 IUT configuration

Insert the following entry to the end of the IUT configuration table:

| Item | IUT configuration | References | Status | Support |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| *CFz | Is Tunneled Direct Link Setup supported? |  | O | Yes, No, N/A |  |  |

Add a new clause at the end of 4.3 as follows:

A.4.z1 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| TDLS1 | Tunneled Direct Link Setup | 7.2.2.1, 11.z1 | CFz:M | Yes, No. N/A |
| TDLS1.1 | TDLS Setup | 7.2.2.1.1, 7.2.2.1.2 | CFz:M | Yes, No. N/A |
| TDLS1.2 | TDLS Teardown | 7.2.2.1.3, 7.2.2.1.4 | CFz:M | Yes, No. N/A |
| TDLS1.3 | TDLS DL Path Switch | 7.2.2.1.5, 7.2.2.1.6 | CFz:M | Yes, No. N/A |
| TDLS1.4 | TDLS AP Path Switch | 7.2.2.1.7, 7.2.2.1.8 | CFz:M | Yes, No. N/A |
| TDLS1.5 | TDLS Peer Key Handshake | 8.5.9 | CFz:M | Yes, No. N/A |
| TDLS1.6 | Link RCPI Request/Report | 7.3.2.21.11, 7.3.2.22.11 | CFz:M | Yes, No. N/A |
| TDLS1.7 | Peer PSM (AP mode) | 7.2.2.1.10 | CFz:O | Yes, No. N/A |
| TDLS1.8 | Peer PSM (client mode) | 7.2.2.1.10 | CFz:O | Yes, No. N/A |
| TDLS1.9 | TDLS Channel Switching | 7.2.2.1.11, 7.2.2.1.12 | CFz:O | Yes, No, N/A |

Annex D

Insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11TunneledDirectLinkSetupImplemented TruthValue

dot11PeerPSMAPModeImplemented TruthValue,

dot11PeerPSMClientModeImplemented TruthValue,

dot11PeerPSMIndicationWindow INTEGER

dot11TDLSChannelSwitchingImplemented TruthValue }

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11TunneledDirectLinkSetupImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Tunneled Direct

Link Setup.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }

dot11PeerPSMAPModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM AP mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMClientModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting Peer PSM client mode.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}

dot11PeerPSMIndicationWindow OBJECT-TYPE

SYNTAX INTEGER (1...256)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute indicates the minimum interval in Beacon Intervals

between successive Peer Traffic Indication frames.

This value is transmitted in the Peer Traffic Indication frame.

The default value of this attribute is equal to one DTIM Interval.”

::= { dot11StationConfigEntry <ANA>}

dot11TDLSChannelSwitchingImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates that the station

implementation is capable of supporting TDLS Channel Switching.

The default value of this attribute is FALSE.”

::= { dot11StationConfigEntry <ANA>}