IEEE P802.11
Wireless LANs

| TGv Teleconference and Ad-Hoc Minutes April, May 2008 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1332 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the meeting notes from TGv teleconferences held: April 1st and April 8th, 2008.

April 1, 2008 Teleconference

Agenda, from e-mail notice:

1. Call to Order, Patent Notification

2. Editor Update

3. Comment Resolution
- Co-located Interference proposed resolutions, see https://mentor.ieee.org/802.11/file/08/11-08-0233-02-000v-co-located-interference-reporting.doc 
- General category comments, see https://mentor.ieee.org/802.11/file/08/11-08-0445-00-000v-lb-123-comment-resolutions-general.xls

4.Tentative Agenda for April 8th Con call – TCLAS, STA Statistics, FBMS, General comments

5. Adjourn


Please review the documents at the following links prior to the call:

-  IEEE Patent Policy - http://standards.ieee.org/board/pat/pat-slideset.ppt
-  Affiliation FAQ - http://standards.ieee.org/faqs/affiliationFAQ.html
-  Anti-Trust FAQ - http://standards.ieee.org/resources/antitrust-guidelines.pdf 
-  Ethics - http://www.ieee.org/portal/cms_docs/about/CoE_poster.pdf

Notes – Tuesday, April 1st, 2008

Attendees: Jari Jokela, Emily Qi, Dorothy Stanley, Allan Thomson, Qi Wang

1. Chair called meeting to order: 12:05 Eastern

Chair called attention to the patent policy slides. Are there any questions on the slides?

None

Chair asked: Are there any patent claim(s)/patent application claim(s) and/or the holder of patent claim(s)/patent application claim(s) that the participant believes may be essential for the use of that standard?

None brought forward

Any additions to the proposed agenda?

Allan has worked on the FBMS comments, agreed to go through those comments, see https://mentor.ieee.org/802.11/file/08/11-08-0439-00-000v-lb123-comments-fbms.xls .

2. Editor’s report.

Emily is working on incorporating the editorial comments, and Draft 2.01 will incorporate the editorial comment resolutions.

3. Comment resolutions

We discussed the proposed addition of transmit interference indication from https://mentor.ieee.org/802.11/file/08/11-08-0233-02-000v-co-located-interference-reporting.doc .

-  The current TGv Draft 2.0 text relates to receive operation, making the assumption that a station can schedule the transmit operation independently of a peer.

- Transmit operation was discussed in the past, but not included, due the belief that the the peer would
schedule its own transmit operation, and that it was unlikely that a station would be able to
receive, but not transmit, making the receive indication sufficient.
- Agreed to ask the commenter to describe a use case or application for the proposed addition, e.g. a real-life use case showing how the indication of receive interference is not sufficient, and transmit interference is used/required.

Jari will propose resolutions for the co-located interference category comments, target for discussion on the April 22 or May 6th conference call.

[Post meeting update: e-mail sent to the commenter 4/1/2008. This comment appears to be related to Co-located Interference category CID, 361, 362 or 368.]

Allan reviewed the FBMS “accepted” comments, see https://mentor.ieee.org/802.11/file/08/11-08-0439-00-000v-lb123-comments-fbms.xls.

- CID 506 – Agreed, with minor wording edits: change “belong to” to “correspond to” and delete duplicated first words of definition

- CID 622 – Agreed with proposed resolution.

- CID 915 – Agreed, add note to indicate change in multiple places

- CID 917 – Agreed with proposed resolution

- CID 521 – Agreed with proposed resolution

- CID 213, 792, 985 – Change to “counter”, slight wording change

- CID 522 – Agreed with proposed resolution

- CID 523 – Agreed with proposed resolution

- CID 524 – Agreed, with change of “Counter” to “Counters”

- CID 988 – Agreed, with wording change to “is shared by all FBMS streams that use the same delivery interval”

- CID 527– Agreed with proposed resolution

- CID 529 – Agreed with proposed resolution

- CID 568, 986 – Agreed with proposed resolution

- CID 569 – Agreed with proposed resolution

- CID 531 – Agreed with proposed “accept”, add “As in comment” as proposed resolution.

- CID 534 – Agreed with proposed resolution

- CID 538 – Agreed with proposed resolution

- CID 1006 – Agreed with proposed resolution

- Line item 534, (is CID 1035, 08-0265r2 will include CID number) - Change to “counter”

- CID 539 – Agree that change needs to be made, same as CID 1006 in “General” category. Proposed resolution to CID 1006 includes needed text.

- CID 248 – Agreed with proposed resolution

- CID 1011 – Agreed, add “as in comment”

- CID 912 – Agreed with proposed resolution

- Continue next time with CID 45

Allan has uploaded a revised spreadsheet, see https://mentor.ieee.org/802.11/file/08/11-08-0439-01-000v-lb123-comments-fbms.xls .

Dorothy reviewed the comment resolutions that have been prepared to date on the General category, see https://mentor.ieee.org/802.11/file/08/11-08-0445-00-000v-lb-123-comment-resolutions-general.xls .

- CID 37 (also 423, 464, 602, 684) – leave as deferred, presentation planned in May

- CID 46 (also 833)– Agreed to “accepted”, change resolution to delete the Extended Capabilities rows from the indicated tables, as text is now duplicated from that in TGn.

- CID 47 – Agreed with proposed resolution

- CID 48 – Change to “deferred”, need text submission with new column contents.

- CID 58 – Agreed with proposed resolution

- CID 59 – Change to “accepted”, since TGn, TGy now use table format, as does TGv. Editorial note also needs to be updated.

- CID 60 – Agreed with proposed resolution, also editorial note needs to be updated.

- CID 105 – Change to “counter” with changes to include “TRANSMISSION FAILURE” in the result, and description.

- CID 106 – Change to “deferred”, need to investigate if MLME changes are required or not. If changes are required, a submission will be needed.

- CID 123 – Change to “deferred”, Emily to contact commenter to understand why all class 1 frames now being changed to “public action”.

- CID 125 – Agreed with proposed “counter” resolution, same as CID 108, already resolved.

- CID 126 – Agreed with leaving “deferred” for now. Mail sent to commenter for additional clarification. Unclear why “supplicant STA and “Authenticator” terms should be used. Also Emily to follow-up with the commenter.

- CID 295 – Agree with proposed resolution.

- CID 302 – Agree with “decline” disposition, change reason to indicate that the commenter’s approach was previously tried, and it introduced confusion, which seems to have been removed with the current approach.

- CID 307 – Agree with “counter” disposition, vendor specific extensions were added in Draft 2.0, and are continuing to be added if missing, add reference to discussed LB123 CID 529.

- CID 429 – Agree to leave as “deferred” for now. E-mail was sent to the commenter to identify specific instances to be corrected, want to document the changes made for this comment. Qi to ping the commenter again.

- CIDs 458, 459 – Qi to ping the commenter for specifics.

- Continue next time with CID 461.

Dorothy will upload a revised spreadsheet, incorporating the agreed changes.

4. Agenda for April 8th teleconference, same time, bridge info

Continued comment resolutions: Ganesh is planning to have the TCLAS resolutions ready. Also continue with FBMS and General categories, STA Statistics if time allows.

5. Adjourn at 14:05 Eastern.

April 8, 2008 Teleconference

Agenda, from e-mail notice:

1. Call to Order, Patent Notification

2. Editor Update

3. Comment Resolution

4.Tentative Agenda for April 15th Con call – Continued comment resolution 

5. Adjourn


Please review the documents at the following links prior to the call:

-  IEEE Patent Policy - http://standards.ieee.org/board/pat/pat-slideset.ppt
-  Affiliation FAQ - http://standards.ieee.org/faqs/affiliationFAQ.html
-  Anti-Trust FAQ - http://standards.ieee.org/resources/antitrust-guidelines.pdf 
-  Ethics - http://www.ieee.org/portal/cms_docs/about/CoE_poster.pdf

Notes – Tuesday, April 8th , 2008

Attendees:  Dean Armstrong, Mark Rison, Dorothy Stanley, Allan Thomson, Ganesh Venkatesan, Qi Wang

1. Chair called meeting to order: 12:05 Eastern

Chair called attention to the patent policy slides. Are there any questions on the slides?

None

Chair asked: Are there any patent claim(s)/patent application claim(s) and/or the holder of patent claim(s)/patent application claim(s) that the participant believes may be essential for the use of that standard?

None brought forward

Any additions to the proposed agenda?

Agreed to discuss the collocated interference comment in 233r2, since Mark and Dean are on the call, then finish the 5 proposed “accept” FBMS comments in 439r1, then TCLAS comments, since Mark is on the call and many of the comments are his.

2. Editor’s report.

Emily was not on the call.

3. Comment resolutions

We discussed the proposed addition of transmit interference indication from https://mentor.ieee.org/802.11/file/08/11-08-0233-02-000v-co-located-interference-reporting.doc. 
On the prior call, members present asked for more info from the commenter, showing use cases where transmit indication would be needed. Dean and Mark provided the following explanation:

- Most commonly, periodic transmit inhibition interference occurs for IEEE 802.11 in the case of a co-located Bluetooth transceiver, where the 802.11 transmitter is inhibited (using hardware coexistence signalling) to protect Bluetooth voice data in slots that are reserved for Synchronous Connection Oriented (SCO) or Extended SCO (eSCO) reception. In these periods it may be possible to address the co-located STA with frames not requiring a control response, and the minor change we propose provides a mechanism which allows this.

- Another example is the case of a co-located IEEE 802.16e (Mobile WiMAX) transceiver where the 802.11 transmitter is periodically inhibited (again, using coexistence signalling) to protect IEEE 802.16e reception of downlink sub-frames.

- Note that in both cases above it is assumed that front-end overload interference prevents simultaneous 802.11 transmission and reception on the co-located transceiver despite any frequency separation which may be achieved through non-overlapping bands of operation or through a mechanism such as Bluetooth Adaptive Frequency Hopping (AFH). This assumption is in line with common experiences of co-locating such technologies.

- Though the performance gains given by the mechanism proposed are not likely to be of great magnitude, coexistence is an area where small gains all add up to make a difference. Therefore it seems sensible to ensure that the Co-located Interference Reporting mechanisms being created in 802.11v allow the functionality we propose - particularly when it comes at the cost of only a single bit field which may be treated as reserved (set to zero on transmission, ignored on reception) by STAs not wishing to use this functionality. As noted in 08-233r2, if it is felt that the range of the Interference Index field cannot be reduced then there are other fields in which this new bit could be incorporated.

Discussion:

- Still not clear that there is value in say the AP knowing that the STA cannot transmit. AP will re-transmit a frame if the ACK is not received, even if AP has knowledge.

- What will the AP do differently? Have avoided requiring any scheduling behavior.

- No difference between this additional bit and the receive indication already included in this regard.

- Last week – asked the question: Are there cases when a STA can receive but not transmit? Have our answer: Cases are collocation with BT and 802.16. Document this info for Jari as input to a proposed resolution; plan to review proposed resolution on April 22 or May 6 along with the other co-located interference comments.

Allan reviewed the remaining FBMS Comments that are proposed “accepted”, see https://mentor.ieee.org/802.11/file/08/11-08-0439-01-000v-lb123-comments-fbms.xls .

- CID 45 – Agreed with proposed resolution.

- CID 103 – Agreed with proposed resolution, add a note in the resolution indicating the page and line number (page 105, line 7)

- CID 107 – Agreed with the proposed resolution, add a note in the resolution indicating page 165, line 35.

- CID 115 – Agreed with proposed resolution, Add a note to indicate page 166, line 21.

- CID 116 – Agreed with proposed resolution. Comment calls for incorporation of TGn changes to the baseline text. Emily will be doing this as part of the editorial update. It looks like the changes TGv made are fairly straightforward additions to the new base. Dorothy will follow-up with Emily to confirm.

Ganesh reviewed comments in the TCLAS category see https://mentor.ieee.org/802.11/file/08/11-08-0455-00-000v-lb123-tclas-resolutions.xls .

- CID 340 – Agreed with proposed resolution, grammar fix: “are be set” to “are set”

- CID 341 – Agreed with proposed resolution.

- CID 342 – Agreed with proposed resolution.

- CID 343 – Similar to CID 345, asks for clarification of support for non-TCP/UDP streams. Defer, discuss at ad-hoc.

- CID 64 – Comments about the ordering within an octet, versus ordering of multiple octets. Text under discussion is from the base spec. Disagree that we should fix the base text in TGv.  Agreed resolution: Declined, forward to TGmb.

- CID 63 – Change to Counter, Delete page 32, lines 37-38, as the text duplicates text on Page 33, lines 21-22.

- CID 344 – Agreed with proposed “counter” resolution. Remove bit references in dashed list items. Re-word to describe a condition, and to remove the double negative, along the lines of “The TCLAS element is rejected if one of the following conditions occur”, or similar.

- CID 345 – Change resolution to “deferred” for now. Don’t quote RFC 790, as it is out of date. Do need references to UDP and TCP documents, Mark checked RFC 768, which has the number on page 3. RFC 793 does not have a reference. Mark to find a valid reference. Likely resolution is to “counter”, adding in the text proposed by the commenter. However have a concern with TGv changing this text, since previously, only TCP and UDP were discussed.  Have declined and counter text ready, discuss at ad-hoc.

- CID 346 – Agree with counter resolution. If using extension headers, the classifier will fail. Add a note on page 32 that “Frame classification when extension headers are used is not supported”

- CID 347 – Agree to “declined” resolution, with “CID will be forwarded to TGmb”

- CID 348 – Agree with proposed resolution and change, Ganesh to check off-line with Dave Stephenson. If Dave agrees, move forward, otherwise bring back for discussion.

- CID 349 – Agree to decline resolution with “CID will be forwarded to TGmb”

- CID 350 – Agree to decline resolution with “CID will be forwarded to TGmb”

- CID 351 – Comment deals with encoding of a 20 bit value, suggesting a better way to encode the value. Preference of the attendees is to adopt the commenter’s alternate suggestion. Check offline with implementers to see if there is any objection to adopting the commenter’s alternate suggestion. No change to functionality, only in the way 20 bits are encoded in 3 octets.

4. Agenda for April 8th teleconference, same time, bridge info

Continue on the next conference call with FBMS (counter, declined), remaining TCLAS and General category comments.

5. Adjourn at 14:05 Eastern.

- References: