IEEE P802.11
Wireless LANs

| LB 123 General Category Comment Resolution Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1332 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the proposed text to resolve LB 123 CIDs: 48, 105, 106,

LB 123 CID 48:

Update to the latest TGk draft

The format of table 7-26 has changed and there is potentially more information to supply regarding extensibility of elements.

(also shows CID 47: Remove "+n" syntax from Table 7-26)

Change Table 7-26 as shown below, inserting an “Extensible” column, and adding the entries in that column as shown.:

- Information Elements

Insert Element IDs <ANA> to <ANA> +18 into Table 26 and change the Reserved row accordingly:

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Event Request (see Error: Reference source not found) | <ANA> | 4 to 256 | Event Type Definitions |
| Event Report (see Error: Reference source not found) | <ANA> | 5 to 256 | Event Report Definition |
| Diagnostic Request (see Error: Reference source not found) | <ANA> | 4 to 256 | Diagnostic Type and Diagnostic Information Sub-elements |
| Diagnostic Report (see Error: Reference source not found) | <ANA> | 5 to 256 | Diagnostic Type and Diagnostic Information Sub-elements |
| Location Parameters (see Error: Reference source not found) | <ANA> | 2 to 256 | Location Sub-elements |
| Multiple BSSID (see Error: Reference source not found) | <ANA> | 3 to 256 |  |
| SSID List (see Error: Reference source not found) | <ANA> | 2 to 256 |  |
| Multiple BSSID-Index (see Error: Reference source not found) | <ANA> | 3 to 5 |  |
| FBMS Descriptor (see Error: Reference source not found) | <ANA> | 3 to 256 |  |
| FBMS Request (see Error: Reference source not found) | <ANA> | 4 to 256 | FBMS Sub-elements |
| FBMS Response (see Error: Reference source not found) | <ANA> | 3 to128 | FBMS Sub-elements |
| Traffic Generation (see Error: Reference source not found) | <ANA> | 3-5 |  |
| BSS Max Idle Period (see Error: Reference source not found) | <ANA> | 3 |  |
| TFS Request (see Error: Reference source not found) | <ANA> | 5 to 256 | TFS Sub-element |
| TFS Response (see Error: Reference source not found) | <ANA> | 2 to 256 | TFS Sub-element |
| Sleep Mode (see Error: Reference source not found) | <ANA> | 5 |  |
| TIM Broadcast Request (see Error: Reference source not found) | <ANA> | 3 |  |
| TIM Broadcast Response (see Error: Reference source not found) | <ANA> | 10 |  |
| Reserved | <ANA>, 220 |  |  |

LB123 CID 105

"This primitive is generated by the MLME when the request to transmit an Event Request frame completes."

It is unclear whether the confirm is synchronous - i.e. takes zero time,   in which case it merely checks the parameters;  or whether it waits for (re-)transmission of the management action frame to fail/succeed.

This lack of clarity will have different systems exhibiting different timing.

Applies to Event Request, Diagnostic Request, Location Request, FBMS, Co-located interference request

Proposed resolution: Include “TRANSMISSION FAILURE” as a ResultCode value in all of the .confirm primitives, and add an explanation that the confirm is sent when transmission failure occurs.

Modify the MLME-EVLREQUEST.confirm Result code and “when generated” on Page 118, line 41 as shown below:

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-EVLREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the event transaction. |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send an Event Request frame. |

- When generated

This primitive is generated by the MLME when transmission of the Event Request frame is acknowledged, (re) transmission of the Event Request frame fails, the Event Request contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-DIAGREQUEST.confirm result code values (page 135, line 59) as shown below:

- MLME-DIAGREQUEST.confirm

- Function

This primitive reports the result of diagnostic request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-DIAGREQUEST.confirm	(
Dialog Token,
ResultCode)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the diagnostic transaction. |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Diagnostic Request frame. |

- When generated

This primitive is generated by the MLME when transmission of the Diagnostic Request frame is acknowledged, (re) transmission of the Diagnostic Request frame fails, the Diagnostic Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-LOCATIONREQUEST.confirm on Page 130, line 35 as shown below:

- MLME-LOCATIONREQUEST.confirm

- Function

This primitive reports the result of a location request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONREQUEST.confirm	(
Dialog Token,
ResultCode,
TX Timestamp,
RX Timestamp)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Indicates the result of the corresponding MLME-LOCATION.request. |
| TX Timestamp | 10 octets as defined in Table v26 | 10 octets as defined in Table v26 | The TX Timestamp field contains the time at which the unicast Location Request frame was transmitted by a STA, defined to occur at the PHY-TXEND.indication of the transmitting Location Request frame. The TX Timestamp field is composed of the time fields defined in Table v26. |
| RX Timestamp | 10 octets as defined in Table v26 | 10 octets as defined in Table v26 | The RX Timestamp field contains the time at which the ACK frame in response to the Location Request frame was received by a STA, defined to occur at the PHY-RXEND.indication of the received ACK frame. The RX Timestamp field is composed of the time fields defined in Table v26. |

- When generated

This primitive is generated by the MLME when transmission of the Location Request frame is acknowledged, (re) transmission of the Location Request frame fails, the Location Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-FBMS.confirm on Page 145, line 23 as shown below:

- MLME-FBMS.confirm

- Function

This primitive reports the result of an FBMS procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- FBMS.confirm	(
ResultCode,
Peer MAC Address,
Dialog Token,
FBMSResponse)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Indicates the result of the corresponding MLME-FBMS.request. |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the FBMS transaction. |
| FBMSResponse | Set of FBMS Response elements | Set of FBMS Response elements | Specifies service parameters for the FBMS. |

- When Generated

This primitive is generated by the MLME when transmission of the FBMS Request frame is acknowledged, (re) transmission of the FBMS Request frame fails, the FBMS Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-CLINTERFERENCEREQUEST.confirm on Page 148, line 45 as shown below:

- MLME-CLINTERFERENCEREQUEST.confirm

- Function

This primitive reports the result of a co-located interference request.

Semantics of the service primitive

- The primitive parameters are as follows:

MLME-CLINTERFERENCEREQUEST.confirm(
Dialog Token,
ResultCode)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the co-located interference transaction. |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Co-located Interference Request frame. |

- When Generated

This primitive is generated by the MLME when the request to transmit a Co-located Interference Request frame completes.

This primitive is generated by the MLME when transmission of the Co-Located Interference Request frame is acknowledged, (re) transmission of the Co-located Interference Request frame fails, the Co-located Interference Request frame contains invalid parameters, or for unspecified failure reasons.

LB 123 CID 106:

There absolutely must be a 1:1 correspondance of .request and .confirm events across the MLME SAP.  It is clear from figure v106 that if the transmission of either the Location Configuration request or response frames eventually fails,   there will be no matching confirm.

Same comment in figures v107, 108, 110, 111, 112

Proposed resolution: Make consistent with CID 105, Include “TRANSMISSION FAILURE” as a ResultCode value in all of the .confirm primitives, and add an explanation that the confirm is indeed sent when transmission failure occurs

Note: no change is made to the figures, as the figures illustrate the basic protocol, and are not meant to be exhaustive of all possible ptorocol uses (page 134, lines 24-25). Do we want to make figure changes and have all the .confirm figures the same?

Modify the LocationCFG.confirm result code values (page 135, line 34) as shown below:

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONCFG.confirm	(
Dialog Token,
ResultCode,
Peer MAC Address,
Location Parameters)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Location Configuration Request frame. |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location configuration response shall be sent. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

Modify 10.3.52.2.3 as shown below:

- When generated

This primitive is generated by the MLME when transmission of the Location Configuration Request frame is acknowledged, (re) transmission of the Location Configuration Request frame fails, the Location Configuration Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the BSSTransitionManagement.confirm clause (page 143, line 59) as shown below:

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-BTM.confirm	(
ResultCode,
Peer MAC Address,
DialogToken,
BSSTransitionQueryReason,
StatusCode,
TargetBSSID)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-BTM.request. |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a BSS Transition Management Response frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this BSS Transition Management transaction as received in the BSS Transition Management Response frame. |
| StatusCode | Integer | 0 – 255 | As defined in 7.4.11.11. |
| TargetBSSID | MACAddress | Any valid individual MAC Address | The BSSID of the BSS that the STA indicated to roam to as received in the BSS Transition Management Response frame. |

- When Generated

This primitive is generated by the MLME when transmission of the BSS Transition Management Request frame is acknowledged, (re) transmission of the BSS Transition Management Request frame fails, the BSS Transition Management Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-TFS .confirm “When Generated text” (page 154, line 1) as shown below:

- MLME-TFS.confirm

- Function

This primitive reports the result of a TFS procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TFS.confirm	(
ResultCode,
PeerSTAAddress,
DialogToken,
TFSResponse,
VendorSpecific)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-TFS.request. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the TFS Response frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the TFS Request and Response transaction. |
| TFSResponse | A set of TFS Response elements | As defined in the TFS Response element | Specifies service parameters for the TFS. One or more TFS Response elements. |
| VendorSpecific | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the MLME when transmission of the TFS Request frame is acknowledged, (re) transmission of the TFS Request frame fails, the TFS Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-SLEEPMODE.confirm clause (page 159, line 51) as shown below:

- MLME-SLEEPMODE.confirm

- Function

This primitive reports the result of a request to send a Sleep Mode Response action frame.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-SLEEPMODE.confirm	(
ResultCode,
PeerSTAAddress,
DialogToken,
SleepMode,
TFSResponse,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-SleepMode.request. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Sleep Mode Response frame is received. |
| DialogToken | Integer | 0 – 255 | The Dialog Token to identify the Sleep Mode Request and Response transaction. |
| SleepMode | As defined in the Sleep Mode element | As defined in the Sleep Mode element | Specifies the proposed sleep mode service parameters for the Sleep Mode Response frame. |
| TFSResponse | A set of TFS Request elements | As defined in the TFS Response element | Specifies the proposed TFS service parameters for the Sleep Mode Response frame. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the MLME when transmission of the Sleep Mode Request frame is acknowledged, (re) transmission of the Sleep Mode Request frame fails, the Sleep Mode Request frame contains invalid parameters, or for unspecified failure reasons.

Modify the MLME-TIMBROADCAST.confirm clause (page 161, line 22) as shown below:

- MLME-TIMBROADCAST.confirm

- Function

This primitive reports the result of a TIM Broadcast procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-TIMBROADCAST.confirm(
ResultCode,
PeerSTAAddress,
Dialog Token,
TIMBroadcastResponse

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, TRANSMISSION FAILURE, MALFORMED REQUEST, REQUESTED INTERVAL TOO LONG, or OVERRIDDEN DUE TO LACK OF RESOURCES | Reports the outcome of a request to send a TIM Broadcast Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the TIM Broadcast process. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the TIM Broadcast request and response transaction. |
| TIMBroadcast- Response | As defined in TIM Broadcast Response element | As defined in TIM Broadcast Response element | Specifies service parameters for the TIM Broadcast. |

- When Generated

This primitive is generated by the MLME when transmission of the TIM Broadcast Request frame is acknowledged, (re) transmission of the TIM Broadcast Request frame fails, the TIM Broadcast Request frame contains invalid parameters, or for unspecified failure reasons.

References: