IEEE P802.11
Wireless LANs

| Proposed Resolutions to LB123 Comments on Multicast Diagnostics |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX, England | +44 1784 848770 | aashley@nds.com |

Abstract

This document contains proposed resolutions to LB123 comments CIDs 16, 450, 932 and 934

Modify clause 7.3.2.21.10a as indicated below:

- Multicast Diagnostics Request

Modify figure 7-72l1 as indicated:

|  | Randomization Interval | Measurement Duration | Group MAC Address | Multicast Triggered Reporting (optional) | Optional Subelements |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 3 | variable |  |
| Figure 7-62l1 -- Measurement Request field format for a Multicast Diagnostics Request |  |  |  |  |  |  |

In clause 7.3.2.21.10a modifythe following paragraph as indicated:

The Group MAC Address field contains the MAC address of the group addressed frames to which the request relates.  The Group MAC Address field can be set to all zeros to indicate that all group addressed frames, apart from the broadcast MAC address, are requested.

At the end of clause 7.3.2.21.10a add the following paragraph:

The Optional Sub-element field format contains zero or more Sub-elements, each consisting of a one octet

Sub-element ID field, a one octet Length field and a variable length Data field, as shown in Figure 7-62b.

The optional sub-elements are ordered by non-decreasing Sub-element ID.

Modify clause 7.3.22.10a  as indicated:

7.3.2.22.10a Multicast Diagnostics Report

The format of the Measurement Report field of a Multicast Diagnostics report is shown in Measurement Report field format for a Multicast Diagnostics Report.

|  | Measurement Start Time | Measurement Duration | Group MAC Address | Multicast Reporting Reason |
| --- | --- | --- | --- | --- |
| Octets: | 8 | 2 | 6 | 1 |
|  | Multicast Received MSDU Count | First Sequence Number | Last Sequence Number | Multicast Rate |
| Octets: | 4 | 2 | 2 | 2 |
| - Measurement Report field format for a Multicast Diagnostics Report |  |  |  |  |

The Measurement Start Time field is set to the value of the STA TSF timer at the time the measurement started. For a triggered Multicast Diagnostics report, this is the TSF value at the reporting STA when the trigger condition was met. For a multicast performance measurement, the start time occurs after a group addressed frame with a destination address equal to the value in the Group MAC Address field has been received, if at least one group addressed frame was received during the measurement duration. When the reason for sending the report is Performance Measurement and the Multicast Received MSDU Count is nonzero, the Measurement Start Time field is set to the value of the STA TSF timer at the time of the first group addressed MSDU received during the measurement interval.

The Measurement Duration field specifies the period over which the Multicast Diagnostic Report was generated. Contents of the Measurement Duration Field defines which reports use the Measurement Duration field and how the field value is defined for those reasons.

| - Contents of the Measurement Duration Field |  |
| --- | --- |
| Multicast Report Reason | Measurement Duration Field |
| Performance Measurement | The duration over which the measurement was performed, expressed in units of TUs. |
| Report Timeout Trigger | Not used. |

The Group MAC Address field contains the MAC address of the group addressed frames (the multicast group or broadcast address) to which the report relates.

11.20.2 Diagnostic and Multicast Diagnostic Reporting

In clause 11.20.2 (page 171 line 64 in draft 2.0), modify the paragraph as shown:

Every STA shall maintain an inactivity timer for every multicast diagnostic request accepted by the STA in which the Report Timeout Request field is set to 1. The inactivity timer at a

recipient is reset when MPDUs corresponding to the monitored group address are received. When a timeout of the multicast diagnostic timer is detected, the STA shall send a multicast diagnostic report with the Report Timeout Trigger in the Multicast Reporting Reason field set to one..

References: