IEEE P802.11
Wireless LANs

Normative Text for Channel Usage

Date: 2008-04-01

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Alex Ashley | NDS Ltd | One London Road, Staines, Middlesex, TW18 4EX | +441784848770 | aashley@nds.com |
| Michelle Gong | Intel Corporation | 2200 Mission College Santa Clara, CA 95054 | 408-765-7994 | Michelle.x.Gong@intel.com |
| Brian Hart | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 |  | brianh@cisco.com |
| Jari Jokela | Nokia | Visiokatu 3, Tampere, Finland | +358504860445 | Jari.jokela@nokia.com |
| Emily Qi | Intel Corporation | 2111 NE 25th Ave, Hillsboro OR | 503-264-7799 | Emily.h.qi@intel.com |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA 94588 | +1 925 474 2290 | matta@trapezenetworks.com |

Abstract

This submission contains the normative text for Channel Usage to provide channel/frequency usage recommendation for non-infrastructure networking.

This proposal provides comment resolution for LB123 CID #501 and CID#667, and the text is aligned with P802.11v-D2.0.

##### Motivation:

##### In general, a non-802.11-infrastructure network (e.g. IBSS, one-hop Mesh) can co-exist with an infrastructure network

##### The channel/frequency number that is selected by a non-802.11-infrastructure network is critical to the overall WiFi network performance

##### Guidance from the infrastructure on channel/frequency usage for non-802.11-infrastructure network can facilitate better network coexistence and improve overall network performance:

##### Coordinated channel planning

##### Reduce radio interference

##### Balance traffic load

##### Solution Summary:

##### Introducing Channel Usage IE

##### Provide recommended channel/frequency lists for different use cases

##### Allow flexibility for multiple applications and extensibility for future use cases

##### Non-AP STA can request channel usage information with or without having to associate to the infrastructure network

##### AP provides Channel Usage information that either IT recommended or the network infrastructure examined based on the interference level, traffic load or supported regulatory classes.

##### On the receipt of Channel Usage element in the Probe Response or Channel Usage Response frame, the recipient may use the channel usage information as part of channel selection processing to start a non-infrastructure network.

Proposed Changes to 802.11v D2.0:

3 Definitions

3.XX Non-Infrastructure mode: A BSS in which no access to a distribution system (DS) is available. Currently this includes Independent Basic Service Set (IBSS).

- Wireless LAN Network Management

- Overview

TGv editor: change the 3rd paragraph as follows:

The Wireless Network Management service includes:

- BSS Transition Management

- Channel Usage

- Co-located Interference Reporting

- Diagnostic Reporting

- Event reporting

- Location Services

- Maximum Multicast Rate processing

- Multicast Diagnostic reporting

- Multiple BSSIDs

- Proxy ARP

- Sleep Mode

- STA Diagnostics

- TIM Broadcast

- Traffic Filtering Service

- Traffic Generation

TGv editor: insert the following subclause after 5.2.11.2 as 5.2.11.3, and change following clause numbers correspondingly.

5.2.11.3 Channel Usage

The Channel Usage information provided by the AP to the non-AP STA is to advise the STA how to co-exist with the infrastructure network. The non-AP STAs may use as part of channel selection processing to start a non-infrastructure network.

7.2.3.8 Probe Request frame format

TGv editor: Insert the following rows in Table 7-14:

| Order | Information | Notes |
| --- | --- | --- |
|  | Channel Usage | The Channel Usage element may be present if dot11MgmtOptionChannelUsageEnabled is true. |
|  | Supported Regulatory Classes | The Supported Regulatory Classes element is present if the Channel Usage element is present. |

- Probe Response frame format

TGv editor: Insert the following rows in Table 7-15:

| Order | Information | Notes |
| --- | --- | --- |
|  | Channel Usage | The Channel Usage element is present if the Channel Usage element is present in the Probe Request frame and dot11MgmtOptionChannelUsageEnabled is true. |

### 7.3.2 Information Elements

TGv editor: Insert the following rows into Table 7-26:

| Information element | Element ID | Length (in octets) |
| --- | --- | --- |
| Channel Usage (see 7.3.2.81) | ANA | 4 to 256 |

- Extended Capabilities information element

TGv editor: Insert the following rows into Table 35a:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Channel Usage | The STA sets the Channel Usage field to 1 When dot11MgmtOptionChannelUsageEnabled is set to true and sets it to 0 otherwise. See 11.20.16. |

TGv editor: Insert the following after 7.3.2.80:

##### 7.3.2.81 Channel Usage element

The Channel Usage element defines the channel usage information for non-infrastructure networking. The format of the Channel Usage element is shown in Figure v-x1.

|  | Element ID | Length (2*n+1) | Usage Mode | Regulatory Class 1 | Channel Number 1 | ... | Regulatory Class n | Channel Number n |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 |

Figure v-x1. Channel Usage element format.

The Element ID field is equal to the Channel Usage value in Table 26.

The Length field is set to one plus twice the number of Regulatory Class/Channel Number pairs.

The Usage Mode field is set to a number that identifies the usage of the recommended channels listed in the Regulatory Class/Channel Number pair fields. The Usage Mode definitions are shown in Table v1-x1.

Table v-x1 Usage Mode definitions

| Value | Usage Mode |
| --- | --- |
| 0 | Reserved |
| 1 | Non-infrastructure |
| 2-255 | Reserved |

There are n Regulatory Class/Channel Number pairs, indicated by 1 through n. Each Regulatory Class/Channel Number pair comprises a Regulatory Class and a Channel Number. The Regulatory Class/Channel Number pair fields are not present if the Channel Usage element is included in the Probe Request frame and Channel Usage Request frame.

The Regulatory Class 1 through Regulatory Class n fields each contains an enumerated value from Annex J, specifying the regulatory class in which the paired Channel Number fields are valid.

The Channel Number 1 through Channel Number n fields each contains a single channel number.

Channel numbering is dependent on paired Regulatory Class according to Annex J.

The Channel Usage element may be included in Probe Request frames, as described in 7.2.3.8, Probe Response frames, as described in 7.2.3.9, Channel Usage Request frames, as described in 7.4.7a.8, and Channel Usage Response frames, as described in 7.4.7a.9. The use of the Channel Usage element and frames is described in 11.20.16.

### 7.4.7a Public Action frame details

Note 7.4.7a is defined in 802.11y D5.0.

### 7.4.7a.1 Public Action frame

Insert the following rows in table 7-57ab.

Table 7-57ab Public Action field values

| Action field value | Description |
| --- | --- |
| ANA | Channel Usage Request |
| ANA | Channel Usage Response |
| ANA+1-255 | Reserved |

Insert the following after 7.4.7a.7( last clause number used in 802.11y D.50):

##### 7.4.7a.8 Channel Usage Request frame format

The Channel Usage Request frame is sent by a non-AP STA to the AP to request the specified Channel Usage information.  The format of the Channel Usage Request frame is defined in Figure v-x6.

|  | Category | Action | Dialog Token | Channel Usage Elements | Supported Regulatory Classes Element | Vendor Specific Elements (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | variable |

Figure v-x6. Channel Usage Request frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Usage Request frame, as specified in 7.4.7a.1.

The Dialog Token field is set to a non-zero value chosen by the non-AP STA sending the Channel Usage Request frame to identify the request/response transaction.

The Channel Usage Element field includes one or more Channel Usage elements described in 7.3.2.81 to identify the request Usage Mode.

The Supported Regulatory Classes Element field includes a Supported Regulatory Classes element to indicate the support regulatory classes for the requested network type and consistent with the Country element in the Beacon frame of which the AP advertises. The Supported Regulatory Classes is described in 7.3.2.51.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

Editorial Note: 7.3.2.51 is defined in TGy D5.0.

##### 7.4.7a.9 Channel Usage Response frame format

The Channel Usage Response frame is sent by an AP STA in response to a Channel Usage Request frame, or autonomously. The format of the Channel Usage Response frame is shown in Figure v-x7.

The Channel Usage Response frame is sent by an AP in response to a Channel Usage Request frame.

|  | Category | Action | Dialog Token | Channel Usage Elements | Country Element (Optional) | Power Constraint Element (Optional) | Vendor Specific Elements (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable | 3 | Variable |

Figure vn. Channel Usage Response frame format.

The Category field is set to the value indicating the Wireless Network Management category, as specified in 7.3.1.11.

The Action field is set to the value indicating Channel Usage Response frame, as specified in 7.4.7a.1.

The Dialog Token field is set to the value in the corresponding Channel Usage Request frame, or zero for an unsolicited frame.

The Dialog token field is set to the nonzero value received in the Channel Usage Request frame if the Channel Usage Response frame is being transmitted in response to a Channel Usage Request frame. The Dialog token field is set to zero if the Channel Usage Response frame is being transmitted, but not in response to a Channel Usage Request frame.

The Channel Usage Element field includes one or more Channel Usage elements described in 7.3.2.81.

The Country Element field includes zero or one Country element described in 7.3.2.9. The Country element is included in the Country Element field when an unsolicited and/or group addressed Channel Usage Response frame is transmitted.

The Power Constraint Element field includes zero or one Power Constraint elements described in 7.3.2.16. The use of the Power Constraint element included in the Power Constraint Element field is described in 11.20.16.

The Vendor Specific Elements field contains zero or more Vendor Specific elements, as defined in 7.3.2.26.

- 10. Layer Management

- 10.3 MLME ASP Interface

TGv editor: Insert the following after 10.3.57

    - Channel Usage request

The following MLME primitives support the signaling of Channel Usage request. Figure v-x101 depicts the Channel Usage request process. The figure illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

Figure v-x101 Channel Usage request Protocol Exchange

      - MLME-CHANNELUSAGE.request

10.3.58.1.1 Function

This primitive requests the transmission of a Channel Usage Request frame be sent to an AP.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELUSAGE.request	(
PeerSTAAddress,
Dialog Token, 
ChannelUsage,
SSID,
SupportedRegulatoryClasses,

VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Channel Usage process. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the Channel Usage request and response transaction. |
| ChannelUsage | A set of Channel Usage element | A set of Channel Usage element | Specifies request types for the Channel Usage request. |
| SSID | Octet string | 0 – 32 octets | Specifies the desired SSID or the wildcard SSID. |
| SupportedRegulatoryClasses | As defined in Supported Regulatory Classes element | As defined in Supported Regulatory Classes element | Specifies the Supported Regulatory Classes information for the Channel Usage Request. |
| VendorSpecificInfo | A set of information Elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the SME to request that a Channel Usage Request frame be sent to the BSS which is advertising the SSID passed down in this primitive.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Usage Request action management frame. The STA then attempts to transmit this to the BSS which is advertising the SSID included in this primitive request. When wild card SSID is passed down the MLME-CHANNELREQUEST.request shall be transmitted to all BSSs in the current scan list as determined by the most recent MLME-SCAN.request.

- MLME-CHANNELUSAGE.confirm

- Function

This primitive reports the result of a Channel Usage procedure.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELUSAGE.confirm(
ResultCode,
PeerSTAAddress,
Dialog Token,
ChannelUsage, 
SSID,
Country,  
Power Constraint,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Channel Usage Request frame. |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Channel Usage process. |
| Dialog Token | Integer | 1 – 255 | The Dialog Token to identify the Channel Usage request and response transaction. |
| ChannelUsage | A set of Channel Usage element | A set of Channel Usage element | Specifies parameters for the Channel Usage. |
| SSID | Octet string | 0 – 32 octets | Specifies the SSID or the wildcard SSID used in the request. |
| Country | An information element | As defined in 7.3.2.9 | Zero or one Country information element. |
| Power Constraint | An information element | As defined in 7.3.2.16 | Zero or one Power Constraint information element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

- When Generated

This primitive is generated by the MLME as a result of an MLME-CHANNELUSAGE.request and indicates the results of the request.

This primitive is generated when the MLME-CHANNELUSAGE.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a Channel Usage Response frame from the AP.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.16.

- MLME-CHANNELUSAGE.indication

- Function

This primitive indicates that a Channel Usage Request frame was received from a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELUSAGE.indication(
PeerSTAAddress,
Dialog Token, 
ChannelUsage,
SSID,
SupportedRegulatoryClasses,

VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Channel Usage Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Usage request and response transaction. |
| ChannelUsage | A set of Channel Usage element | A set of Channel Usage element | Specifies request types for the Channel Usage request. |
| SSID | Octet string | 0 – 32 octets | Specifies the desired SSID or the wildcard SSID. |
| SupportedRegulatoryClasses | As defined in Supported Regulatory Classes element | As defined in Supported Regulatory Classes element | Specifies the Supported Regulatory Classes information for the Channel Usage Request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the MLME when a valid Channel Usage Request frame is received.

- Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 11.20.16.

- MLME-CHANNELUSAGE.response

- Function

This primitive is generated in response to an MLME-CHANNELUSAGE.indication requesting a Channel Usage Response frame is sent to a non-AP STA.

- Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANNELUSAGE.response(
PeerSTAAddress,
Dialog Token, 
ChannelUsage, 
SSID,
Country,
Power Constraint,
VendorSpecificInfo)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Channel Usage Request frame was received. |
| Dialog Token | Integer | 0 – 255 | The Dialog Token to identify the Channel Usage request and response transaction. |
| ChannelUsage | A set of Channel Usage elements | A set of Channel Usage elements | Specifies parameters for the Channel Usage. |
| SSID | Octet string | 0 – 32 octets | Specifies the desired SSID or the wildcard SSID. |
| Country | An information element | As defined in 7.3.2.9 | Zero or one Country information element. |
| Power Constraint | An information element | As defined in 7.3.2.16 | Zero or one Power Constraint information element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements |

- When Generated

This primitive is generated by the SME in response to an MLME-CHANNELUSAGE.indication requesting a Channel Usage Response be sent to a non-AP STA.

- Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Usage Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

Insert new subclause at the end of 11.20

### 11.20.16 Channel Usage Procedures

Channel Usage information is a set of channels provided by an AP to non-AP STAs for operation of a non-infrastructure network. The Channel Usage information provided by the AP to the non-AP STA is to advise the STA how to co-exist with the infrastructure network.

A STA that has a value of true for the MIB attribute dot11MgmtOptionChannelUsageEnabled is defined as a STA that supports use of Channel Usage and shall indicate this support by setting the Channel Usage bit of the Extended Capabilities element to 1 in transmitted Beacon, Association Request, Association Response, Reassociation Request, Reassociation Response, Probe Request, and Probe Response frames.

A non-AP STA that is not associated to an AP prior to using a non-infrastructure network may send a Probe Request frame including both Supported Regulatory Classes and Channel Usage elements.

A non-AP STA supporting use of Channel Usage may send a Channel Usage Request frame at any time after association to the AP that supports the use of Channel Usage to request the Channel Usage information for supported regulatory classes.

Upon the receipt of Channel Usage element in the Probe Request frame, the AP supporting use of Channel Usage shall send a Probe Response frame including one or more Channel Usage elements. Upon receiving a Channel Usage Request frame, the AP supporting use of Channel Usage shall send a Channel Usage Response frame including one or more Channel Usage elements. Channel Usage elements shall only include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with the Country element in the Beacon or Probe Response frame.

The AP may send an unsolicited group addressed or individually addressed Channel Usage Response frame to the STAs that have requested use of Channel Usage if the corresponding Channel Usage information needs to be updated. The Country information element shall be included in the unsolicited and/or group addressed Channel Usage Response frame. The Power Constraint information element may be included in the Channel Usage Response frame.

On the receipt of Channel Usage element in the Probe Response or Channel Usage Response frame, the recipient may use

- the channel usage information as part of channel selection processing to start a non-infrastructure network.

- the Power Constraint element, if present, as part of determining its maximum transmit power for transmissions for the non-infrastructure network.

NOTE 1— If either a recommended regulatory class, or a recommended channel, or both are not supported or understood by the recipient, or if the operating country of the sender is unknown, the recipient should discard the corresponding recommendation.

NOTE 2— The STA that has not requested use of Channel Usage should discard the unsolicited group addressed Channel Usage Response frame.

Insert the following rows to A.4.14

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM17 | Channel Usage | 11.20.16; 7.3.2.81; 7.4.7a.8;7.4.7a.9 | CFv:O | Yes, No, N/A |

# Annex D

Insert the following entries at the end of the Dot11WirelessMgmtOptionsEntry sequence list:

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionPresenceEnabledTruthValue,

dot11MgmtOptionFBMSImplementedTruthValue,

dot11MgmtOptionFBMSEnabledTruthValue,

dot11MgmtOptionEventsEnabledTruthValue,

dot11MgmtOptionDiagnosticsEnabledTruthValue,

dot11MgmtOptionMultiBSSIDImplementedTruthValue,

dot11MgmtOptionMultiBSSIDEnabledTruthValue,

dot11MgmtOptionTFSImplementedTruthValue,

dot11MgmtOptionTFSEnabledTruthValue, dot11MgmtOptionSleepModeImplementedTruthValue,

dot11MgmtOptionSleepModeEnabled TruthValue,

dot11MgmtOptionTIMBroadcastImplemented	 TruthValue,

dot11MgmtOptionTIMBroadcastEnabled TruthValue,

dot11MgmtOptionProxyARPEnabled TruthValue,

dot11MgmtOptionBSSTransitionEnabled TruthValue,

dot11MgmtOptionChannelUsageImplemented	TruthValue,

dot11MgmtOptionChannelUsageEnabled	TruthValue}

On page 155, as part of Dot11WirelessMgmtOptionsEntry, add the followingMIB objects:

dot11MgmtOptionChannelUsageImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Channel Usage when the dot11WirelessManagementImplemented is TRUE.”

DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionChannelUsageEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Channel Usage is enabled. Channel Usage is disabled otherwise.”

DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry 17 }