IEEE P802.11
Wireless LANs

| 802.11 TGn LB124 Submission |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: YYYY-MM-DD |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB124 comments assigned to the author:

The changes marked in this document are based on TGn Draft version D4.00.

They relate to:

- TGn Ad-hoc: PHY

- Comment group:  PHY Intro

# Comments

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 6302 | 247.16 | 20.2.2 | "NOTE 2—Channel spacings of 5 or 10 MHz indicated for L_DATARATE do not apply to an HT STA using NONHT-DUP-OFDM, {OFDM and CH_OFFSET of value CH_OFF_20U}, {OFDM and CH_OFFSET of value CH_OFF_20L} modulations." I don't see how a device operating with a clause-20 PHY can transmit any 5 or 10 MHz PPDU. We have not considered how an HT device manages the coexistence of 5 or 10 MHz channels. It is possible to have a device that is capable of operation in two modes: .11a/j (5, 10MHz permitted) or .11n (20, 40 permitted), but not one that on a packet-by-packet basis was able to switch between them. This device can be made without the need for any standards support. Consider the question - can a device operating in a 5MHz band transmit an A-MPDU? The answer is no. The HT PAR specifies 2.4 and 5GHz band operation only. A device operating in some other band that requires 5 MHz operation is not acting an an HT device - it doesn't advertise an HT Cabilities element, it supports none of the HT MAC features and none of the HT PHY features. Therefore the HT PHY need not, and indeed cannot support 5MHz or 10MHz channels, as they are outside the scope of the PAR./ This device may become an HT device when operating in 2.4 or 5GHz, in which case it will support Clause 20 PHY operation. While operating as an HT device, it cannot support 5 or 10 MHz channel width operation. Don't forget - the Clause 20 PHY SAP is an abstract description of an interface required to provide Clause 20 operation. It is not the implementation interface for a device capable of switching between multiple modes of operation. | Remove the cited text, and the specification of rates for 5 and 10MHz operation in the L_DATARATE parameter. |

Proposed Resolution:  Counter (Accept in principle)

See resolution in document 11-08/0492r0, which provides edits that match the proposed change.

Proposed Edits:

TGn Editor: Change Table 20-1 as follows:

| - TXVECTOR and RXVECTOR parametersinput.doc(#3153,2646, 988) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
|  |  |  | See NOTE 1 |  |
| FORMAT |  | Determines the format of the PPDU. Enumerated Type: NON_HT indicates Clause 15, 17, 18 or 19 PPDU formats, or non-HT duplicated PPDU format. In this case the modulation is determined by the NON_HT_MODULATION parameter. HT_MF indicates HT-mixed (#5400) format. HT_GF indicates HT-greenfield (#5400) format. | Y | Y |
| NON_HT_ MODULATION | FORMAT is NON_HT | Enumerated Type: ERP-DSSS ERP-CCK ERP-OFDM ERP-PBCC DSSS-OFDM OFDM NON_HT_DUP_OFDM (#5761) | Y | Y |
|  | Otherwise | Not present(# 2645) |  |  |
| L_LENGTH | FORMAT is NON_HT | Indicates the length of the PSDU in octets in the range 1-4095. This value is used by the PHY to determine the number of octet transfers that occur between the MAC and the PHY. | Y | Y |
|  | FORMAT is HT_MF | Indicates the value in the Length field of the Non-HT SIGNAL Field in the range 1-4095. This use is defined in 9.13.4 (L_LENGTH and L_DATARATE parameter values for HT-mixed (#5400) format PPDUs (#1705, 34)). This parameter may be used for the protection of more than one PPDU as described in 9.13.5 (L-SIG TXOP protection). | Y | Y |
|  | FORMAT is HT_GF | Not present | N | N |
| L_DATARATE | FORMAT is NON_HT (#5761) (#5230) | Indicates the rate used to transmit the PSDU in Mb/s. Allowed values depend on the value of the NON_HT_MODULATION parameter thus: ERP-DSSS: 1 and 2 ERP-CCK: 5.5 and 11 ERP-PBCC: 5.5, 11, 22 and 33 DSSS-OFDM, ERP-OFDM, NON_HT_DUP_OFDM: 6, 9, 12, 18, 24, 36, 48 and 54 OFDM: 6, 9, 12, 18, 24, 36, 48, and 54 | Y | Y |
|  | FORMAT is HT_MF | Indicates the data rate value that is in the Non-HT SIGNAL Field. This use is defined in 9.13.4 (L_LENGTH and L_DATARATE parameter values for HT-mixed (#5400) format PPDUs (#1705, 34)).(#2648) | Y | Y |
|  | FORMAT is HT_GF | Not Present | N | N |
| LSIGVALID (#2254) | FORMAT is HT_MF | True if L-SIG Parity is valid False if L-SIG Parity is not valid | N | Y |
|  | FORMAT is not HT_MF | Not Present | N | N |
| SERVICE | FORMAT is NON_HT and NON_HT_MODULATION is one of: (#5761) - DSSS-OFDM, - ERP-OFDM, - OFDM (#799) | Scrambler initialization, 7 null bits + 9 reserved null bits(# 726) | Y | N |
|  | FORMAT is HT_MF or HT_GF | Scrambler initialization, 7 null bits + 9 reserved null bits(# 727) | Y | N |
|  | Otherwise | Not present | N | N |
| TXPWR_LEVEL |  | The allowed values for the TXPWR_LEVEL parameter are in the range from 1 to 8. This parameter is used to indicate which of the available TxPowerLevel attributes defined in the MIB shall be used for the current transmission. | Y | N |
| RSSI |  | The allowed values for the RSSI parameter are in the range from 0 through RSSI maximum. This parameter is a measure by the PHY of the power observed at the antennas used to receive the current PPDU. RSSI shall be measured during the reception of the PLCP preamble. In HT-mixed (#5400) format the reported RSSI shall be measured during the reception of the HT LTFs. RSSI is intended to be used in a relative manner, and it shall be a monotonically increasing function of the received power. | N | Y |
| PREAMBLE_TYPE | FORMAT is NON_HT and NON_HT_MODULATION(# 621) is one of: - UPPER-20-ERP-DSSS, - UPPER-20-ERP-CCK, - UPPER-20-ERP-PBCC, - UPPER-20-DSSS-OFDM, - LOWER-20-ERP-DSSS, - LOWER-20-ERP-CCK, - LOWER-20-ERP-PBCC, - LOWER-20-DSSS-OFDM, - ERP-DSSS, - ERP-CCK, - ERP-PBCC, - DSSS-OFDM (#799)(# 333). | Enumerated type: SHORTPREAMBLE, LONGPREAMBLE | Y | Y |
|  | Otherwise | Not present | N | N |
| MCS | FORMAT is HT_MF or HT_GF | The MCS field selects the modulation and coding scheme used in the transmission of the packet. The value used in each modulation and coding scheme is the index defined in Error: Reference source not found. Integer: range 0 to 76. Values of 77 to 127 are reserved. The interpretation of the MCS index is defined in Error: Reference source not found. | Y | Y |
|  | Otherwise | Not present | N | N |
| REC_MCS | FORMAT is HT_MF or HT_GF | The MCS that the STA's receiver recommends. | N | O |
|  | Otherwise | Not Present | N | N |
| CH_BANDWIDTH | FORMAT is HT_MF or HT_GF | The CH_BANDWIDTH parameter indicates whether the packet is transmitted using 40 MHz or 20 MHz channel width. Enumerated type: HT_CBW20 for 20 MHz and 40 MHz Upper and 40 MHz Lower modes HT_CBW40 for 40 MHz(#5232) | Y | Y |
|  | FORMAT is NON_HT | Enumerated type: NON_HT_CBW40 for non-HT duplicate format (#6147) NON_HT_CBW20 for all other non-HT formats (#5761, 6147) | Y | Y |
| CH_OFFSET | (#5761) | The CH_OFFSET parameter indicates which portion of the channel is used for transmission. Refer to Error: Reference source not found for valid combinations of CH_OFFSET and CH_BANDWIDTH. (#5048) Enumerated type: CH_OFF_20 indicates the use of a 20 MHz channel (that is not part of a 40 MHz channel). CH_OFF_40 indicates the entire 40 MHz channel. CH_OFF_20U indicates the upper 20 MHz of the 40 MHz channel CH_OFF_20L indicates the lower 20 MHz of the 40 MHz channel. | Y | N |
| LENGTH | FORMAT is HT_MF or HT_GF | Indicates the length of an HT PSDU in the range 0-65535 octets. A value of zero indicates a Null Data Packet that contains no data symbols after the HT preamble (see Error: Reference source not found)(# 728). | Y | Y |
|  | Otherwise | Not present | N | N |
| SMOOTHING | FORMAT is HT_MF or HT_GF | Indicates whether frequency-domain smoothing is recommended as part of channel estimation. (See NOTE 2.)(#1673) Enumerated type: SMOOTHING_REC indicates that smoothing is recommended. SMOOTHING_NOT_REC indicates that smoothing is not recommended. | Y | Y |
|  | Otherwise | Not present | N | N |
| SOUNDING | FORMAT is HT_MF or HT_GF | Indicates whether this packet is a sounding packet. Enumerated type: SOUNDING indicates this is a sounding packet. NOT_SOUNDING indicates this is not a sounding packet. | Y | Y |
|  | Otherwise | Not present | N | N |
| AGGREGATION | FORMAT is HT_MF or HT_GF | Indicates whether the PSDU contains an A-MPDU. Enumerated type: AGGREGATED indicates this is a packet with A-MPDU aggregation. NOT_AGGREGATED indicates this is a packet without A-MPDU aggregation | Y | Y |
|  | Otherwise | Not present | N | N |
| STBC | FORMAT is HT_MF or HT_GF | Indicates the difference between the number of space time streams () and the number of spatial streams () indicated by the MCS. Set to 0 indicates no STBC (). Set to 1 indicates . Set to 2 indicates . Value of 3 is reserved. | Y | Y |
|  | Otherwise | Not present | N | N |
| FEC_CODING(#1810) | FORMAT is HT_MF or HT_GF | Indicates whether Binary Convolutional Code (BCC) or Low Density Parity Check (LDPC) encoding is used. Enumerated type: BCC_CODING indicates Binary Convolutional Code. LDPC_CODING indicates Low Density Parity Check code. | Y | Y |
|  | Otherwise | Not present | N | N |
| SHORT_GI | FORMAT is HT_MF or HT_GF | Indicates whether a short Guard Interval (GI) is used in the transmission of the packet. Enumerated type: LONG_GI indicates short GI is not used in the packet. SHORT_GI indicates short(# 1292,5234) GI is used in the packet. | Y | Y |
|  | Otherwise | Not present | N | N |
| NUM_EXTEN_SS | FORMAT is HT_MF or HT_GF | Indicates the number of extension spatial streams that are sounded during the extension part of the HT training in the range 0-3. | Y | Y |
|  | Otherwise | Not present | N | N |
| ANTENNA_SET | FORMAT is HT_MF or HT_GF | The ANTENNA_SET parameter indicates which antennas of the available antennas are used in the transmission. The length of the field is 8 bits. A 1 in bit position n, relative to the LSB, indicates that antenna n is used.(# 729) At most 4 bits out of 8 may be set to 1. This field is only present if antenna selection is applied. | O | N |
|  | Otherwise | Not present(#1901; ANTENNA_SET_ON parameter removed) | N | N |
| N_TX | FORMAT is HT_MF or HT_GF | The N_TX parameter indicates the number of transmit chains. | Y | N |
|  | Otherwise | Not present | N | N |
| EXPANSION_MAT | EXPANSION_MAT_TYPE is COMPRESSED_SV | Contains a set of compressed beamforming matrices as defined in Error: Reference source not found. The number of elements depends on the number of spatial streams and the number of transmit chains. | Y | N |
|  | EXPANSION_MAT_TYPE is NON_COMPRESSED_SV | Contains a set of non-compressed beamforming matrices as defined in Error: Reference source not found. The number of complex elements is where is the total number of subcarriers, is the number of columns and is the number of rows in each matrix. | Y | N |
|  | EXPANSION _MAT_TYPE is CSI_MATRICES | Contains a set of CSI matrices as defined in Error: Reference source not found. The number of complex elements is where is the total number of subcarriers, is the number of columns and is the number of rows in each matrix. | Y | N |
|  | Otherwise.(#2656) | Not present | N | N |
| EXPANSION_MAT_TYPE | EXPANSION_MAT is present. | Enumerated type: (#2656 -- removed NO_EXPANSION_MAT) COMPRESSED_SV indicates that EXPANSION_MAT is a set of compressed beamforming matrices. NON_COMPRESSED_SV indicates that EXPANSION_MAT is a set of non-compressed beamforming matrices. CSI_MATRICES indicates that EXPANSION_MAT(# 801) is a set of channel state matrices. | Y | N |
|  | Otherwise | Not present | N | N |
| CHAN_MAT | CHAN_MAT_TYPE is COMPRESSED_SV. | CHAN_MAT contains a set of compressed beamforming matrices as defined in Error: Reference source not found based on the channel measured during the training symbols of the received PPDU. The number of elements depends on the number of spatial streams and the number of transmit chains. | N | Y |
|  | CHAN_MAT_TYPE is NON_COMPRESSED_SV. | CHAN_MAT contains a set of non-compressed beamforming matrices as defined in Error: Reference source not found based on the channel measured during the training symbols of the received PPDU. The number of complex elements is where is the total number of subcarriers, is the number of columns and is the number of rows in each matrix. | N | Y |
|  | CHAN_MAT_TYPE is CSI_MATRICES. | CHAN_MAT contains a set of CSI matrices as defined in Error: Reference source not found based on the channel measured during the training symbols of the received PPDU. The number of complex elements is where is the total number of subcarriers, is the number of columns and is the number of rows in each matrix. | N | Y |
|  | Otherwise.(#2657) | Not present. | N | N |
| CHAN_MAT_TYPE | FORMAT is HT_MF or HT_GF. | Enumerated type: COMPRESSED_SV indicates that CHAN_MAT is a set of compressed beamforming vector matrices. NON_COMPRESSED_SV indicates that CHAN_MAT is a set of non-compressed beamforming vector matrices. CSI_MATRICES indicates that CHAN_MAT is a set of channel state matrices. | N | Y |
|  | Otherwise | Not present | N | N |
| RCPI |  | This parameter is a measure of the received RF Power averaged over all the receive chains in the data portion of a received frame.(#2658) Refer to Error: Reference source not found for the definition of RCPI. (#5235) | N | Y |
| SNR(#812) | CHAN_MAT_TYPE is CSI_MATRICES | This parameter is a measure of the received SNR per chain. SNR indications of 8 bits are supported. SNR shall be the dB representation of linearly averaged values over the tones represented(#3241) in each receive chain as described in 7.3.1.27 (CSI Report field) | N | Y |
|  | CHAN_MAT_TYPE is COMPRESSED_SV or NON_COMPRESSED_SV | This parameter is a measure of the received SNR per stream. SNR indications of 8 bits are supported. SNR shall be the sum of the dB values of SNR per tone divided by the number of tones represented(#3241) in each stream as described in 7.3.1.28 (Non-compressed Beamforming (#2825) Report field) and 7.3.1.29 (Compressed Beamforming (#2825) Report field) | N | Y |
| NOTE 1(# 2659)—In the TXVECTOR and RXVECTOR columns, the following apply: Y = Present N = Not present O = Optional NOTE 2—Setting of the smoothing bit is defined in Error: Reference source not found.(#1673) |  |  |  |  |