IEEE P802.11
Wireless LANs

| Normative Text Proposal for LB#123 Comment Resolution on Co-Located Interference Reporting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jing Zhu | Intel | 2111, NE 25th Ave., Hillsboro, OR 97124 | +1 (503) 265 7073 | jing.z.zhu@intel.com |
| Emily Qi | Intel |  |  |  |

Abstract

This submission contains the normative text to resolve IEEE 802.11v LB#123 comments with CID 99, 312, 493, 494, 495, 497, 807, and 810.

The text is aligned with P802.11v-D2.01

7.3.2 Information Elements

Add a new raw into Table 7-26 as follows:

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Co-located Interference Report (see 7.3.2.80) | <ANA> | 21 |

Insert the new section 7.3.2.80 into 7.3.2 as follows:

7.3.2.80 Co-located Interference Report element

The Co-located Interference Report element defines information about the characterstics of the reported co-located interferences. The Co-located Interference Report element is defined in Figure vx1.

The Element ID field is equal to the Co-Located Interference Report value in Table 7-26.

The Length field is 19.

Move Figure v111 of 7.4.11.15 to here, and make changes as follows:

|  | Element ID | Length | Report Period | Interference Level | Interference Level Accuracy/Interference Index |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |
|  | Interference Interval | Interference Burst Length | Interference Start Time | Interference Center Frequency | Interference Bandwidth |
| Octets | 2 4 | 2 4 | 4 | 2 | 2 |

Figure vx1: Co-located Interference Report element format

Move the 6th and beyond paragraphs of 7.4.11.15 to here, and make changes as follows:

The Report Period field indicates how often the STA automatically reports the co-located interference. The

field is in units of 100 TUs. If the Report Period is 0, then the reporting is not periodic, and a report is generated when the STA detects a change in the co-located interference. See 11.20.10 for further details.

The Interference Level field is a 2's complement signed integer indicating the level of the co-located interference in units of dBm averaged over a 4 μs period during an interference period. When the interference level is unknown, the field is set to 127dBm  When the interference level is equal or greater than 126dBm, the field is set to 126dBm. If no co-located interference is present the field is set to -128 dBm. The interference level is referenced to the antenna connector used for reception. For more than one antenna see Antenna Connector definition (see 3.7b).

EDITORIAL NOTE—Antenna Connector (3.7b) is defined in TGk D9.0.

The Interference Level Accuracy/Interference Index field is shown in Figure vx2.

Move Figure v112 of 7.4.11.15 to here:

|  | B0-B3 | B4-B7 |
| --- | --- | --- |
|  | Expected Accuracy | Interference Index |
| Octets: | 1 | 1 |

Figure vx2—Interference Level Accuracy/Interference Index field format

The Expected Accuracy field represents an unsigned integer indicating the expected accuracy of the estimate of interference in dB (95% confidence interval). If accuracy is unknown then the Expected Accuracy field is set to 15.

The Interference Index field indicates the interference index that is unique for each type of interference source. The field set to 0 indicates that no co-located interference is present. See 11.20.10 for further details.

The Interference Interval field indicates the interval between two successive periods of interference in microseconds. When the interval between two successive periods of interference is variable the field is set to 65535 232-1.When the interval between two successive periods of interference is equal or greater than 65534 232-2 the field is set to 65534 232-2. If no co-located interference is present the field is set to 0.

The Interference Burst Length field indicates the duration of each period of interference in microseconds. When the duration of each period of interference is variable the field is set to 65535 232-1. When the duration of each period of interference is equal or greater than 65534 232-2 the field is set to 65534 232-2. If no co-located interference is present the field is set to 0.

The Interference Start Time field contains the least significant 4 octets (i.e. B0-B31) of the TSF timer at the start of the next interference burst. When either the Interference Interval or the Interference Burst Length fields are set to 232-1, this field indicates the average duty cycle. The average duty cycle value is defined as Round-to-Integer ((65535 232 – 1)[average interference burst length (microsecond)]/[average interference interval (microsecond)]). When the interference is non-periodic the Interference Start Time field is set to 0. If no co-located interference is present the field is set to 0.

The Interference Center Frequency field indicates the center frequency of interference in 2MHz. When center requency is unknown or interference covers the whole channel, the center frequency of the channel is reported. If no co-located interference is present the field is set to 0. The Interference Bandwidth field indicates the bandwidth at the -3dB roll-off point of the interference signal in 5kHz. When bandwidth of the interference signal is unknown, the field is set to 65535. When bandwidth of the interference signal is equal or greater than 65534 the field is set to 65534. If no co-located interference is present the field is set to 0.

7.4.11.15 Co-located Interference Response frame format

Change Fig. v110 of 7.4.11.15 as follows:

|  | Category | Action | Dialog Token | Response Info Co-located Interference Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1variable |

Figure v110 – Co-located Interference Response frame format

Change the fifth paragraph of 7.4.11.15 as follows:

The Response Info field is shown in Figure v111. Each Co-located Interference Response frame may include

up to 16 separate Response Info fields. The Response Info field format is shown in Figure v111 and the total length of single Response Info field is 15 octets. The Co-located Interference Report Elements field contains one or more Co-located Interference Report elements to indicate the characteristics of the reported co-located interferences, as defined in 7.3.2.80.

Move the sixth paragraph and beyond of 7.4.11.15 to 7.3.2.80

Move Figure v111 and Figure v112 of 7.4.11.15 to 7.3.2.80

- Semantics of the service primitive

Change the first paragraph as follows:

The primitive parameters are as follows:

MLME- CLINTERFERENCERESPONSE.request (

Peer MAC Address,

Dialog Token,

Response Info Co-located Interference Report)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Co-located Interference Response shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the co-located interference transaction. |
| Response Info Co-located Interference Report | Integer Set of Co-located Interference Report elements | As defined in the Co-located Interference Response frame Set of Co-located Interference Report elements | Specifies the co-located interference characterictics. |

- Semantics of the service primitive

Change the first paragraph as follows:

The primitive parameters are as follows:

MLME- CLINTERFERENCERESPONSE.indication	(
Peer MAC Address,

Dialog Token,

Response Info Co-located Interference Report)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location response shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Response Info Co-located Interference Report | Integer Set of Co-located Interference Report elements | As defined in the Co-located Interference Response frame Set of Co-located Interference Report elements | Specifies the co-located interference characteristics. |

- Co-located Interference Reporting

Change the fifth paragraph as follows:

The reporting STA shall use the Interference Index field in Co-located Interference Response frame to indicate different types of interference. The reporting STA shall select unique Interference Index value for each co-located interference source. For example if the reporting STA has knowledge of co-located interference originating from two interference sources the reporting STA shall report both type of interference using separate Response Info fields  Co-located Interference Report elements having separate Interference Index field. Both Response Info fields Co-located Interference Report elements can be sent in the same Co-located Interference Response frame and both can have the same Report Period.