IEEE P802.11
Wireless LANs

| TGz LB127 CID 17-36 (7.1.3.1.6) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 7, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document addresses TGz LB127 CID 17-36, which are related to subclauses 7.1.3.1.6.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7 Frame formats

7.1 MAC frame formats

Change 7.1.3.1.6 as shown:

7.1.3.1.6 Power Management field

The Power Management field is 1 bit in length and is used to indicate the power management mode of a STA. The value of this field remains constant in each frame from a particular STA within a frame exchange sequence defined in 9.12. The value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

A value of 1 indicates that the STA will be in PS mode or Peer PS mode. A value of 0 indicates that the STA will be in active mode. This field is always set to 0 in frames transmitted by an AP.

Change Table 4 as shown:

Table 4—QoS Control field

| Applicable Frame (sub) Types | Bits 0-3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8-15 |
| --- | --- | --- | --- | --- | --- |
| QoS CF-Poll and QoS CF-Ack+ CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP limit |
| QoS Data+CF-Poll and QoS Data+CF-Ack+CF-Poll frames sent by HC | TID | EOSP | Ack Policy | A-MSDU Present | TXOP limit |
| QoS Data and QoS Data+CF-Ack frames sent by HC or by a non-AP STA acting in Peer PSM AP mode | TID | EOSP | Ack Policy | A-MSDU Present | AP PS Buffer State |
| QoS Null frames sent by HC | TID | EOSP | Ack Policy | Reserved | AP PS Buffer State |
| QoS Data and QoS Data+CF-Ack frames sent by non-AP STAs not acting in Peer PSM AP mode | TID | 0 | Ack Policy | A-MSDU Present | TXOP duration requested |
|  | TID | 1 | Ack Policy | A-MSDU Present | Queue size |
| QoS Null frames sent by non-AP STAs | TID | 0 | Ack Policy | Reserved | TXOP duration requested |
|  | TID | 1 | Ack Policy | Reserved | Queue size |

| 17 | Alexander | Tolpin | 7.1.3.1.6 | 10 | 38 | T | Y | Peer PS Mode refers to TDLS as defined in 3.z2. I assume the statement "A value of 1 indicates that the STA will be" should be related to traditional PS Mode (as defined in IEEE802.11-2007.pdf ) as well as to Peer PSM. However, the current text specifies that the mentioned bit is related to Peer PSM only. Actually, I do not see the any reason to include the word "Peer" in 7.1.3.1.6 | Replace "A value of 1 indicates that the STA will be in (Peer) PS mode." to "A value of 1 indicates that the STA will be in traditional PS mode or in Peer PSM, or remove '(Peer)' from this line. | Counter – see 11-08-xxxx-00-000z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18 | Allan | Thomson | 7.1.3.1.6 | 10 | 38 | T | Y | The statement "the STA will be in (Peer) PS mode" appears to change the sentence to ONLY Peer PS mode. Assuming the intent is to change the sentence to include Peer PS Mode as well as normal PS Mode the sentence needs work. | Correct sentence to something like "will be in PS mode or Peer PS mode" | Accept – see 11-08-xxxx-00-000z |
| 19 | Andrew | Myles | 7.1.3.1.6 | 10 | 38 | T | Y | The text states, "A value of 1 indicates that the STA will be in (Peer) PS mode". It appears this means "PS mode or Peer PS mode" but it is unclear. | Change sentence to, "A value of 1 indicates that the STA will be in Peer PS or PS mode" | Accept – see 11-08-xxxx-00-000z |
| 20 | Ganesh | Venkatesan | 7.1.3.1.6 | 10 | 38 | T | Y | [KS] (Peer) should not be in parantheses, it is specifying meaning for a field under specific circumstances. | Change "STA will be in PS mode or in Peer PSM mode." | Accept – see 11-08-xxxx-00-000z |
| 21 | Ganesh | Venkatesan | 7.1.3.1.6 | 10 | 36 | T | N | "A value of 1 indicates that the STA will be in (Peer) PS mode." Setting the bit to 1 means that the STA is in power save mode. This includes PS (between the STA and the associated AP), Peer PSM or both. It is better to either drop the addition "peer" or state all choices explicitly. | Remove the addition of "(Peer)" or restate as follows: "A value of 1 indicates that the STA will be in Power Save mode: Power Save mode with the associated AP and/or Peer PSM with the non-AP STA with which the STA has a Direct Link established." | Counter – see 11-08-xxxx-00-000z |
| 22 | Jarkko | Kneckt | 7.1.3.1.6 | 10 | 38 | T | Y | The same power management field should not define the power management for AP - STA link and for DLS link. Links should have independent power management links. The PS power management mode is used also to enable scanning or other unspecified operations and handling both links with the same power management field complicates their handling | Add separate power management fields for DLS and AP-STA interfaces. | Reject – The meaning of the field is the same when sent to an AP or when sent to a non-AP STA which supports the AP-side of U-APSD (i.e. peer PSM in AP mode). |
| 23 | Keith | Amann | 7.1.3.1.6 | 10 | 38 | T | Y | The amendment adds the word "Peer" within the sentence "A value of 1 indicates that the STA will be in PS mode". This change implies that the use of the PS bit is then restricted to DLS in some fashion, and therefore creates an interpretation that would be overly restrictive and result in incompatibilities with existing equipment. | Remove the word "Peer" as was added. | Counter – see 11-08-xxxx-00-000z |
| 24 | Osama | Aboul-Magd | 7.1.3.1.6 | 10 | 38 | T | Y | The use of the term "Peer" is confusing. It may give the impression of an ad hoc arrangement. There is the need for some clarification and discussion. | It is probably adequate to include in Chapter 5 some discussion on the DLS service and TDLS. Also needed is highlighting relationship to the general IEEE 802.11 architecture and relationship to ad hoc and infrastructure modes of operation | Counter – see 11-08-xxxx-00-000z |
| 25 | Padam | Kafle | 7.1.3.1.6 | 10 | 38 | T | Y | It will be a good to define different power management fields for AP - STA link and for DLS link. Using same power management field complicates specific operations such as scanning. | Use separate power management fields for DLS and AP-STA interfaces. | Reject – The meaning of the field is the same when sent to an AP or when sent to a non-AP STA which supports the AP-side of U-APSD (i.e. peer PSM in AP mode). |
| 26 | Adrian | Stephens | 7.1.3.5 | 11 | 1 | T | Y | I don't recognise the starting point of Table 4 as being amended by the TGn draft. And there are no editing instructions to clarify the edit. | Show edits based on TGn as your baseline. | Accept – see 11-08-xxxx-00-000z |
| 27 | Allan | Thomson | 7.1.3.5 | 11 | 1 | T | Y | The row for QoS data frames sent by non-AP has additional text to state "to an AP". What is a non-AP is sending a frame to another non-AP not using direct link? Such as a IBSS? | Clarify Qos control field | Counter – see 11-08-xxxx-00-000z |
| 28 | Andrew | Myles | 7.1.3.5 | 11 | 1 | T | Y | Table 4 is modified to add a line for TDLS.. However, in doing so it removes the specification for a non-AP STA in an IBSS | Fix the indicated problem | Accept – see 11-08-xxxx-00-000z |
| 29 | Bill | Marshall | 7.1.3.5 | 11 | 1 | T | y | original text in third row of this table is "QoS data frames sent by non-AP STAs". Is the intent to delete the "STAs"? If so, show it with strikethrough. I suspect this was not the intent. | show "STAs" as being unchanged from the base document | Accept – see 11-08-xxxx-00-000z |
| 30 | Ganesh | Venkatesan | 7.1.3.5 | 11 | 1 | T | Y | [KS] Table 4: 3rd row, 1st Col: non-AP is incomplete | Change "non-AP STA to an AP" | Counter – see 11-08-xxxx-00-000z |
| 31 | Ganesh | Venkatesan | 7.1.3.5 | 11 | 1 | T | Y | [KS] Table 4: 4th row, 1st Col: To be consistent with previous rows, it is worthwhile to add the parties involved in this exchange. | Change "...send between non-AP STAs over the direct link". | Counter – see 11-08-xxxx-00-000z |
| 32 | Henry | Ptasinski | 7.1.3.5 | 11 | 1 | T | Y | “to an AP” is incorrect. QoS data frames can be sent between non-AP STAs in an IBSS without using TDLS | Change to “QoS data frames sent by non-AP STAs when not using DLS” and change “frames sent over the direct link” to “frames sent by non-AP STAs when using DLS” | Counter – see 11-08-xxxx-00-000z |
| 33 | Matthew | Fischer | 7.1.3.5 | 11 | 1 | T | Y | Applicable frame types column, last row of the table, there is no mention of whether the transmitter is a non-AP STA or an AP or either. I guess that it is generally assumed that DLS STA are non-AP, but is this always true? | Clarify. | Accept – see 11-08-xxxx-00-000z |
| 34 | Osama | Aboul-Magd | 7.1.3.5 | 11 | 1 | T | Y | regarding the new raw added to Table 4, it is not clear how a frame sent over the direct link will be differentiated from, for example, a frame that is sent by non-AP station to AP. | Need clarification. | Counter – see 11-08-xxxx-00-000z |
| 35 | Thomas | Kolze | 7.1.3.5 | 11 | 1 | T | Y | Last row of table, applicable frame types column, it is not specified if the transmitter is a non-AP STA or an AP or either. It appears to be assumed that DLS STA are non-AP, correctly or not. | Make explicit. | Counter – see 11-08-xxxx-00-000z |
| 36 | Tomoko | Adachi | 7.1.3.5 | 11 | 1 | T | Y | The last row in Table 4. In 7.1.3.5.2, it is described that EOSP is used by the HC. The frames in the last row are not sent by HC, so setting EOSP in Bit 4 should be a mistake. | Change the entry of Bit 4 in the last row of Table 4 from "EOSP" to "Reserved". | Counter – see 11-08-xxxx-00-000z |