IEEE P802.11
Wireless LANs

| TGz LB127 CID 70-168 (7.2.2.1.x) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 8, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document addressed TGz LB127 CID 70-168, which relate to subclauses 7.2.2.1.x.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Replace 7.2.2.1.1 through 7.2.2.1.10 with the following:

7.2.2.1.1 TDLS Setup Request frame format

The Information field of a TDLS Setup Request frame contains the information shown in Table z2.

Table z2—Information for TDLS Setup Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Dialog Token | The Dialog Token contains a unique nonzero value for the conversation between the peer STAs involved in this request. |
| 2 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | RSNIE_I | The RSNIE for Initiator STA is included if security is required on the direct link. |
| 5 | SMK Message 1 FTIE | The SMK Message 1 FTIE is included if security is required on the direct link. |
| 6 | DH_I | The Public value for Initiator STA is included if security is required on the direct link. |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.2 TDLS Setup Response frame format

The Information field of a TDLS Setup Response frame contains the information shown in Table z3.

Table z3—Information for TDLS Setup Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Request. |
| 3 | Association Request frame body without RSN element | The Association Request frame body is specified in 7.2.3.4. Only present for Status Code 0 (Successful). |
| 4 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. Only present for Status Code 0 (Successful). |
| 5 | RSNIE_P | The RSNIE of Peer STA is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 6 | SMK Message 2 FTIE | The SMK Message 2 FTIE is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 7 | DH_P | The Public value for Peer STA is included if security is required on the direct link and the Status Code is 0 (Succesful). |
| 8 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. Only present for Status Code 0 (Successful). |

7.2.2.1.3 TDLS Setup Confirm frame format

The Information field of a TDLS Setup Response frame contains the information shown in Table z4.

Table z4—Information for TDLS Setup Confirm frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Setup Response. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | SMK Message 3 FTIE | The SMK Message 3 FTIE is included if security is required on the direct link. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.4 TDLS Teardown Request frame format

The Information field of a TDLS Teardown frame contains the information shown in Table z5.

Table z5—Information for TDLS Teardown Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.5 TDLS Teardown Response frame format

The Information field of a TDLS Teardown frame contains the information shown in Table z6.

Table z6—Information for TDLS Teardown Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Teardown Request. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.6 TDLS DL Path Switch Request frame format

The Information field of a TDLS DL Path Switch Request frame contains the information shown in Table z7.

Table z7—Information for TDLS DL Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

Reason for the Path Switch Request frame

7.2.2.1.7 TDLS DL Path Switch Response frame format

The Information field of a TDLS DL Path Switch Response frame contains the information shown in Table z9.

Table z9—Information for TDLS DL Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Suspend frame. |
| 3 | Result | The Result field (1-octet) indicates the result of the Path Switch request and is set to one of values in Table z10. |
| 4 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

frame

7.2.2.1.8 TDLS AP Path Switch Request frame format

The Information field of a TDLS AP Path Switch Request frame contains the information shown in Table z11.

Table z11—Information for TDLS AP Path Switch Request frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Reason Code | The Reason Code is defined in 7.3.1.7. |
| 2 | Dialog Token | The Dialog Token contains a unique value for the conversation between the peer STAs involved in this request. |
| 2 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 5 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.9 TDLS AP Path Switch Response frame format

The Information field of a TDLS AP Path Switch Response frame contains the information shown in Table z12.

Table z12—Information for TDLS AP Path Switch Response frame

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Status Code | The Status Code is defined in 7.3.1.9. |
| 2 | Dialog Token | The Dialog Token is copied from the corresponding TDLS Path Switch Request frame. |
| 3 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 4 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

7.2.2.1.10 Peer Traffic Indication

The Information field of a Peer Traffic Indication frame contains the information shown in Table z13.

Table z13—Peer Traffic Indication

| Order | Information | Notes |
| --- | --- | --- |
| 1 | AC_BK traffic available | 1 octet field which is set to 0 if AC_BK is empty and set to 1 if traffic is available in AC_BK. Values 2-255 are reserved. |
| 2 | AC_BE traffic available | 1 octet field which is set to 0 if AC_BE is empty and set to 1 if traffic is available in AC_BE. Values 2-255 are reserved |
| 3 | AC_VI traffic available | 1 octet field which is set to 0 if AC_VI is empty and set to 1 if traffic is available in AC_VI. Values 2-255 are reserved |
| 4 | AC_VO traffic available | 1 octet field which is set to 0 if AC_VO is empty and set to 1 if traffic is available in AC_VO. Values 2-255 are reserved |
| 5 | Peer PSM Indication Window | 1 octet field which indicates the minimum interval between successive Peer Traffic Indication frames sent to the same peer, expressed in Beacon Intervals. |
| 6 | Link Identifier | The Link Identifier is specified in 7.3.2.z1. |
| 7 | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements |

The Peer Traffic Indication frame indicates the power save buffer state at the STA buffering data for a peer STA in Peer PSM client mode.

Each of the ACs traffic available fields is 1 octet in length. It is set to zero value if the corresponding AC (AC0, AC1, AC2, AC3) is empty and set to 1 if traffic is available for the specific AC. Values 2-255 are reserved.

In subclause 11.2.1.12,  replace the fourth paragraph with the following paragraph:

The behavior of a STA in Peer PSM AP mode with respect to traffic buffered for a peer STA in client mode is the same as a regular U-APSD AP, with the following exception. When traffic becomes available for an AC becomes backlogged and no Service Period has occurred for that AC for a period of Peer PSM Indication Window prior to the arrival of the new traffic, the STA in AP mode sends a unicast Peer Traffic Indication frame to the peer STA in client mode through the access point, indicating the backlogged AC(s) for which traffic is available.

In subclause 11.2.1.12, change the final paragraph as follows:

The Peer PSM Indication Window field is optionally present in the Peer Traffic Indication frame. The default Peer PSM Indication Window is equal to the DTIM interval.

In subclause 11.z1, add the following sentence:

TDLS frames shall be sent through the AP, which means that the A1 field is set to the BSSID.

In subclause 7.3.1.7, add the following reason codes to Table 7-22, and update the reserved values accordingly:

Table 7-22—Reason codes

| Reason code | Meaning |
| --- | --- |
| <ANA> | Path switch requested for unspecified reason |
| <ANA> | Path switch requested due to change in power save mode |
| <ANA> | Path switch requested due to change in link state |

In subclause 7.3.1.9, add the following status codes to Table 7-23, and update the reserved values accordingly:

Table 7-23—Status codes

| Status code | Meaning |
| --- | --- |
| <ANA> | Path switch request denied for unspecified reason |
| <ANA> | Path switch request denied because of entering power save mode |
| <ANA> | Path switch request denied because of link state |

| 70 | Adrian | Stephens | 7.2.2.1.1 |  |  | T | Y | For good reason, management frames consist of an optional fixed part followed by a number of elements. The TDLS setup frame consists of a fixed part (link identifier), a variable part (Association Request), a fixed part (Dialog Token) and then a number of elements. Same comment for 7.2.2.1.2 (TDLS Setup Response frame) | Reorder so that all fixed fields occur before variable fields. | Accept – see 11-08-xxxx-00-000z |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 | Alex | Ashley | 7.2.2.1.1 | 12 | 8 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS Setup Request frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 72 | Alex | Ashley | 7.2.2.1.1 | 12 | 6 | T | Y | When is RSNIE of Peer STA optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 73 | Alex | Ashley | 7.2.2.1.1 | 12 | 6 | T | Y | When is SMK Message 1 FTIE optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 74 | Allan | Thomson | 7.2.2.1.1 | 12 | 8 | T | Y | What does "sent through the AP" mean? As received on the AP radio and retransmitted? Same comment applies to all other sub-sections in 7.2.2.1.x | Provide clear definition of what "through" means. | Accept – see 11-08-xxxx-00-000z |
| 75 | Allan | Thomson | 7.2.2.1.1 | 12 | 8 | T | Y | Sentence states "shall". This belongs in clause 11. Same comment applies to all other sub-sections in 7.2.2.1.x | Fix sentence and move shall to clause 11 | Accept – see 11-08-xxxx-00-000z |
| 76 | Bill | Marshall | 7.2.2.1.1 | 12 | 6 | T | y | SMK Message 1 (according to 8.5.9.1) includes a Lifetime, but Lifetime is not defined as part of the FTIE. | Include a TIE in this frame for the key lifetime |  |
| 77 | Brian | Hart | 7.2.2.1.1 | 12 | 5 | T | Y | Language is vague here and subsections 7.2.2.1.1-7.2.2.1.10. "The TDLS setup request frame contains" => "The Information field of the TDLS setup request contains …" | fix, 10x | Accept – see 11-08-xxxx-00-000z |
| 78 | Brian | Hart | 7.2.2.1.1 | 12 | 5 | T | Y | Length of dialog not specified; length of all fields/elements should be specified; parsing of frame not possible since end of AssocReq frame body cannot be determined; when length is added, rules for extensibility of fields/elements need to be defined. Extensibliity of frame impossible due to optional trailing field - e.g. convert to subelement. IMHO, dialog token should apply at a regular place near the start of the list of fields. Apply to subsections 7.2.2.1.1-7.2.2.1.10 | fix, 10x | Counter – See CID 70. |
| 79 | Brian | Hart | 7.2.2.1.1 | 12 | 6 | T | Y | "Public value" is very vague => "Public value of SecurityStuff" for DH_I and DH_P. Apply to subsections 7.2.2.1.1-7.2.2.1.10 | fix, Nx |  |
| 80 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text states, "The Dialog Token contains a unique value for this conversation." Conversation is not a defined term. The term unique also needs further definition--unique to what entities? | Clarify the text. | Accept – see 11-08-xxxx-00-000z |
| 81 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text states that the Association Request frame body is included but doesn't state which fields and elements are to be included. Moreover, the association request frame body includes an RSN IE and TDLS Setup request includes one too (the RSNIE_I). | The text is ambiguous and should be clarified. The text (not necessarily in this clause) should clearly explain the purpose for having two different RSN IEs. | Accept – see 11-08-xxxx-00-000z |
| 82 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text describes a field called DH_I, which is defined as a "Public value for Initiator STA". However, the public value is never defined in the text. Note that this is just one instance of the term "public value"; it is used for several fields in this draft, so this comment applies to multiple clauses in the document. | Define "public value". |  |
| 83 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text states, "The Dialog Token contains a unique value for this conversation." Conversation is not a defined term. The term unique also needs further definition--unique to what entities? | Clarify the text. | Accept – see 11-08-xxxx-00-000z |
| 84 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text states that the Association Request frame body is included but doesn't state which fields and elements are to be included. Moreover, the association request frame body includes an RSN IE and TDLS Setup request includes one too (the RSNIE_I). | The text is ambiguous and should be clarified. The text (not necessarily in this clause) should clearly explain the purpose for having two different RSN IEs. | Accept – see 11-08-xxxx-00-000z |
| 85 | Dave | Stephenson | 7.2.2.1.1 | 12 | 6 | T | Y | The text describes a field called DH_I, which is defined as a "Public value for Initiator STA". However, the public value is never defined in the text. Note that this is just one instance of the term "public value"; it is used for several fields in this draft, so this comment applies to multiple clauses in the document. | Define "public value". |  |
| 86 | David | Hunter | 7.2.2.1.1 | 12 | 6 | T | Y | Why does the whole Assoc Request frame body have to be included in the TDLS Request frame? Doesn't the receiver already know the SSID? And the source and destination MAC addresses are already in the Data Frame header. | Replace the Assoc Request body field with the needed fields -- Capability Info, Supported Rates, Extended Supported Rates. Anything else? | Counter – The contents of the Association Request is clarified to exclude the RSN element. |
| 87 | David | Hunter | 7.2.2.1.1 | 12 | 6+ | T | Y | Need a TDLS timeout value, similar to the DLS timeout value. | Insert field for "TDLS Timeout Value", with the Notes value being the same as for DLS: defined in 7.3.1.1.3 -- or make a local copy of 7.3.1.1.3. |  |
| 88 | Dorothy | Stanley | 7.2.2.1.1 | 11 |  | T | N | Delete the sentences, in this and subsequent clauses "The xx frame shall be sent through the AP", as it's really not needed - at least it looks like all of the TDLS frames are sent to the AP, that's the definition of the protocol. | As in comment | Accept – see 11-08-xxxx-00-000z |
| 89 | Ganesh | Venkatesan | 7.2.2.1.1 | 12 | 6 | T | Y | [KS] Table z2: Where is "Dialog Token" defined? | Give reference, or define this element | Decline – The dialog token field is defined in 7.3.1.12 of the base standard. |
| 90 | Ganesh | Venkatesan | 7.2.2.1.1 | 12 | 6 | T | Y | [KS] Dialog token is mentioned to be unique for the conversation…How do we prevent STA using same value for a different conversation? How do we prevent 2 STAs from differentiating 2 simultaneous conversations, when both generate a locally unique number which may be same? There is other context that may help identify this case, but makes for a simpler design if we make this globally unique. Or, make the, unique per all conversations from a STA. | Change "Dialog Token contains gloabally unique value for this conversation". | Accept – see 11-08-xxxx-00-000z |
| 91 | Ganesh | Venkatesan | 7.2.2.1.1 | 12 | 6 | T | Y | [KS] Vendor-specific fields have been omitted from TDLS Setup message format, and from all other messages defined in this draft. Vendor specific IEs allow for vendor differentiation and should be included, also as in current 802.11 messages. | Add vendor-specific IE to all message formats defined in the amendment. | Accept – see 11-08-xxxx-00-000z |
| 92 | Jesse | Walker | 7.2.2.1.1 | 12 | 6 | T | Y | I am confused. The association frame body already contains an RSN IE. Why is a second needed? | Please take greater care spelling out which fields from the Association frame body are actually needed. | Accept – see 11-08-xxxx-00-000z |
| 93 | Matthew | Fischer | 7.2.2.1.1 | 12 | 6 | T | Y | The Association Request frame body is a variable-size field. How is a a receiver of the TDLS Request frame supposed to know where the end of the assreq frame body field ends and the Dialog Token field starts? | Somehow provide length information for this field, or maybe just put the Dialog Token as Order 1 and shift the other fields accordingly (e.g. LinkID becomes 2 and assreq becomes 3). In this way, identifying the end of the assreq frame body is unimportant, since it ends with elements, which are self-describing, and it is followed by more elements. The problem with the existing ordering is that you cannot tell if the dialog token is a dialog token or another element. | Accept – see 11-08-xxxx-00-000z |
| 94 | Meiyuan | Zhao | 7.2.2.1.1 | 12 | 6 | T | Y | DH_I IE is redundant, since it's already included in SMK Message 1 FTIE | Remove row 6 from Table z2 |  |
| 95 | Osama | Aboul-Magd | 7.2.2.1.1 | 12 | 8 | T | Y | The term "shall be sent through the AP" is used in many places. I would suggest adding more explanation, i.e. who originates the frame, the role of the AP (maybe nothing), and so on. | Need clarification. | Counter – Quoted text moves to section 11.z1. |
| 96 | Thomas | Kolze | 7.2.2.1.1 | 12 | 6 | T | Y | The Association Request frame body is a variable-size field. Need to provide means for receiver of the TDLS Request frame to determine where the end of the frame body field ends and the Dialog Token field starts. | Provide length information for this field, or explore other solutions. | Accept – see 11-08-xxxx-00-000z |
| 97 | Tomoko | Adachi | 7.2.2.1.1 | 12 | 6 | T | N | Usually in other action frames, value zero has special meaning or not allowed to use. Do we allow to use zero for the dialog token here? | Clarify. | Yes |
| 98 | Tomoko | Adachi | 7.2.2.1.1 | 12 | 6 | T | Y | "… for this conversation."? | Change it to "… for each setup transaction." | Accept – see 11-08-xxxx-00-000z |
| 99 | Tomoko | Adachi | 7.2.2.1.1 | 12 | 8 | T | Y | "The TDLS Setup Request frame shall be sent through the AP." More explanation, please. | Describe in detail how the frame is sent to the peer STA or refer to an appropriate subclause (11.7 seems to be a good place to refer to). The same change should be made in 7.2.2.1.2-7.2.2.1.9. | Accept – see 11-08-xxxx-00-000z |
| 100 | Alex | Ashley | 7.2.2.1.2 | 13 | 1 | T | Y | When is RSNIE_P optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 101 | Alex | Ashley | 7.2.2.1.2 | 13 | 1 | T | Y | When is SMK Message 2 FTIE optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 102 | Alex | Ashley | 7.2.2.1.2 | 13 | 1 | T | Y | When is DH_P optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 103 | Alex | Ashley | 7.2.2.1.2 | 13 | 3 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS Setup Response frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 104 | Bill | Marshall | 7.2.2.1.2 | 13 | 1 | T | y | Do you really intend the TDLS Setup Response frame to contain an Association Request frame body, or should this be the Association Response frame body? | please change, or clarify the intent of having as Association Request frame in both the TDLS Setup Request and in the TDLS Setup Response. If the intent is that both STAs send an Association Request frame to each other, then the TDLS Setup Response should contain both an Association Request and an Association Response, and the TDLS Setup Confirm would contain the Association Response from the originator. | Counter – Both messages indeed should contain an Association Request frame body. An additional Association Response is not required because it would contain redundant information. However, the contents of the Association Request have been clarified per CID 81. |
| 105 | Bill | Marshall | 7.2.2.1.2 | 13 | 1 | T | y | SMK Message 2 (according to 8.5.9.1) includes a Lifetime, but Lifetime is not defined as part of the FTIE | Include a TIE in this frame for the key lifetime |  |
| 106 | David | Hunter | 7.2.2.1.2 | 12 | 7 | T | Y | If "DH" stands for Diffe-Hellman, then need to say somwhere how and when Diffe-Hellman is used. | Either replace "DH" with some other name or describe the usage of Diffe-Hellman. |  |
| 107 | David | Hunter | 7.2.2.1.2 | 13 | 1+ | T | Y | Why does the whole Assoc Request frame body have to be included in the TDLS Response frame? The initiator of the TDLS knows which link this is about via the link ID, so why does it need to receive the whole Assoc Request frame back? | Replace the Assoc Request body field with the needed fields -- Capability Info, Supported Rates, Extended Supported Rates. Anything else? | Counter – See CID 81. |
| 108 | Ganesh | Venkatesan | 7.2.2.1.2 | 13 | 1 | T | Y | [KS] Table z3: Text says it contain an Association Request frame body. Should be Association Response? | Change table z3, row 3 to Association Response | Decline – See CID 104. |
| 109 | Jesse | Walker | 7.2.2.1.2 | 13 | 1 | T | Y | I am confused. The association frame body already contains an RSN IE. Why is a second needed? | Please take greater care spelling out which fields from the Association frame body are actually needed. | Accept – see 11-08-xxxx-00-000z |
| 110 | Jesse | Walker | 7.2.2.1.2 | 13 | 1 | T | Y | Why is an Association Request Body (instead of a Response) included in the response message? | Please either rationalize this construction or utilize the appropriate fields of an Association Response instead of a Request. | Both messages contain an Association Request frame body because the same set of information needs to be conveyed. See also CID 104 and CID 81. |
| 111 | Kevin | Hayes | 7.2.2.1.2 | 13 | 1 | T | Y | Is the Diaglog Token present if Status != 0? | Explain how the frame will appear when Status != 0. | Accept – see 11-08-xxxx-00-000z |
| 112 | Meiyuan | Zhao | 7.2.2.1.2 | 13 | 1 | T | Y | DH_P IE is redundant, since it's already included in SMK Message 2 FTIE | Remove row 7 from Table z2 |  |
| 113 | Alex | Ashley | 7.2.2.1.3 | 13 | 8 | T | Y | When is SMK Message 3 FTIE optional? | Provide definition of when this field is present | Accept – see 11-08-xxxx-00-000z |
| 114 | Alex | Ashley | 7.2.2.1.3 | 13 | 10 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS Setup Confirm frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 115 | Bill | Marshall | 7.2.2.1.3 | 13 | 8 | T | y | SMK Message 3 also contains RSNIE_I, Lifetime, and DH_I (according to 8.5.9.1). | add RSNIE_I, Tid for Lifetime, and DH_I to Table z4. |  |
| 116 | David | Hunter | 7.2.2.1.3 | 13 | 5+ | T | Y | Since the Peer expects SMK Message 3 back, why not send it a Status, even if something goes wrong? | Add a Status Code field, as the second field, parallel to the Response frame's Status Code field, and with the same description as is used in Table z3. | Accept – see 11-08-xxxx-00-000z |
| 117 | Tomoko | Adachi | 7.2.2.1.3 | 13 | 7 | T | Y | "The TDLS Setup Response frame contains the information shown in Table z4." This subclause if for TDLS Setup Confirm frame format. In the first place, do we need this frame? | Remove the TDLS Setup Confirm frame from the draft. | Decline – The confirm frame contains the third part of the security handshake. |
| 118 | Adrian | Stephens | 7.2.2.1.4 |  |  | T | Y | Is there any reason why 7.2.2.1.4 and 7.2.2.1.6 reverse the order of the Dialog Token and Reason fields? | Make them consistent. | Accept – see 11-08-xxxx-00-000z |
| 119 | Alex | Ashley | 7.2.2.1.4 | 14 | 3 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS Teardown Request frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 120 | Ganesh | Venkatesan | 7.2.2.1.4 | 14 | 3 | T | Y | [KS] Why Teardown shall be sent only via the AP? There may be design reasons to do so, but I would suggest that these teardown frames be sent either via AP or direct link. STAs do not know best channels for teardown, so better to give the option to the STA. Same for TDLS Teardown Response frame. | Change "The TDLS Teardown Request frame may be sent through the AP or via direct link." Same change in 7.2.2.1.5 | Decline – As a general rule, TDLS frames are sent through the AP. The direct path may not always be enabled, while the overhead of sending these frames through the AP is small because they are sent infrequently. It is better to have a clean and simple rule in t his case. |
| 121 | Kevin | Hayes | 7.2.2.1.4 | 14 | 1 | T | Y | "unique value for this conversation" Unique with respect to what? Other conversations or other frames in the same conversation? | Please explain. | Accept – see 11-08-xxxx-00-000z |
| 122 | Michelle | Gong | 7.2.2.1.4 | 14 | 1 | T | Y | "The reason code is defined in 7.3.1.7" Are the reason codes defined in 7.3.1.7 sufficient? For instance, a peer STA may tear down the direct link due to insufficient RSSI or low data rate. | Define more reason codes in 7.3.1.7 that are relevant to TDLS. |  |
| 123 | Michelle | Gong | 7.2.2.1.4 | 14 | 3 | T | Y | Why is TDLS Teardown Request frame sent through the AP? Since the request is sent to tear down the direct link, isn't it better, i.e. less delay and more efficient, to sent it over the direct link? If there are important technical reasons that are unknown to me, please add a note to clarify. If not, please remove this requirement. | Either clarify why this frame has to be sent through the AP or allow the TDLS Teardown Request frame being sent over the direct link. | Decline – See CID 120. |
| 124 | Stephen | Emeott | 7.2.2.1.4 | 13 | 12 | T | Y | If this message is to be allowed over a direct RX path, there should be an option (MIC field) for secure teardown message(s) for STA that have completed the TDLS peer key handshake. If the teardown message is not permitted on the direct RX path, then clause 11.z1 should say so. | Make one of the two suggested changes | Accept – see 11-08-xxxx-00-000z |
| 125 | Tomoko | Adachi | 7.2.2.1.4 | 14 | 1 | T | Y | "… for this conversation."? | Change it to "… for each teardown transaction." | Accept – see 11-08-xxxx-00-000z |
| 126 | Alex | Ashley | 7.2.2.1.5 | 14 | 10 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS Teardown Response frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 127 | Bill | Marshall | 7.2.2.1.5 | 14 | 8 | T | y | TDLS Teardown Response frame needs a Status code | Add a Status code to the frame | Accept – see 11-08-xxxx-00-000z |
| 128 | David | Hunter | 7.2.2.1.5 | 14 | 8+ | T | Y | A Response frame needs a Status Code field. | Add a Status Code field, as the second field, parallel to the Response frame's Status Code field, and with the same description as is used in Table z3. | Accept – see 11-08-xxxx-00-000z |
| 129 | Michelle | Gong | 7.2.2.1.5 | 14 | 10 | T | Y | Why is TDLS Teardown Response frame sent through the AP? Since the response is sent to acknowledge the tearing down of the direct link, isn't it better, i.e. less delay and more efficient, to sent it over the direct link? If there are important technical reasons that are unknown to me, please add a note to clarify. If not, please remove this requirement. | Either clarify why this frame has to be sent through the AP or allow the TDLS Teardown Response frame being sent over the direct link. | Accept – See CID 120. |
| 130 | Alex | Ashley | 7.2.2.1.6 | 14 | 17 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS DL Path Switch Request frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 131 | Bill | Marshall | 7.2.2.1.6 | 14 | 15 | T | y | "Reason" field in this frame is confusing, with Reason codes being defined in 7.3.1.7. | Either use reason codes as defined in 7.3.1.7, or change the name of this field to something else | Accept – see 11-08-xxxx-00-000z |
| 132 | Michelle | Gong | 7.2.2.1.6 | 15 | 1 | T | Y | Reason 1 is "Change in power save mode". Does it mean a STA 1) changing from one power save mode to another or 2) changing from active mode to power save mode? Why can chaning power save mode trigger path switch request? | Please clarify |  |
| 133 | Michelle | Gong | 7.2.2.1.6 | 15 | 1 | T | Y | Reason 2 "Change in link state" is not clear. What is link state? What link state is changed? | Define "link state" and clarify what is "Change in link state" |  |
| 134 | Michelle | Gong | 7.2.2.1.6 | 15 | 1 | T | Y | I don't think the reason codes defined in Table z8 cover all scenarios. There would be other scenarios when a STA wants to do a path change. One example I can think of is related to energy efficiency. It would be more energy efficient if a STA only needs to maintain the AP path, esp. when the STA doesn't have much to transmit. Yet, sending over the direct link could be more bandwidth efficient. Therefore, a STA may want to switch from the direct link to the AP path when it has had no data to transmit/receive for a period of time. | Please identify usage scenarios when a STA may want to send out a path switch request and define corresponding reason code for such usage scenarios. |  |
| 135 | Tomoko | Adachi | 7.2.2.1.6 | 14 | 15 | T | Y | "… for this conversation."? | Change it to "… for each DL path switch transaction." | Accept – see 11-08-xxxx-00-000z |
| 136 | Tomoko | Adachi | 7.2.2.1.6 | 14 | 15 | T | Y | The last row in Table z7 is similar to the ordinary reason code and confusing. | Change the name of the field to something else. Make the same change to the one in Table z11. | Accept – see 11-08-xxxx-00-000z |
| 137 | Alex | Ashley | 7.2.2.1.7 | 15 | 9 | T | Y | Clause 7 should not define behaviour. | Remove "The TDLS DL Path Switch Response frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 138 | Ganesh | Venkatesan | 7.2.2.1.7 | 15 | 7 | T | Y | [KS] Instead of defining a new field "Result", use the status code fields, and add new values to that table | Use Status Codes instead of defining a new field Result | Accept – see 11-08-xxxx-00-000z |
| 139 | Michelle | Gong | 7.2.2.1.7 | 15 | 10 | T | Y | Reason 2 is "Reject because of the link status". It's worthwhile to be more specific as to what link status would cause a "Reject" | Be more specific on the link status. For instance, what link status would cause a reject? (A good link status certainly wouldn't) Also, how is link status measured and determined? |  |
| 140 | Adrian | Stephens | 7.2.2.1.8 |  |  | T | N | "The Dialog Token contains a unique value for this conversation." Strictly, it doesn't. The name space is of finite size and cannot uniquely identify any particular "conversation" | Replace "contains a unique value for" with "identifies" Make this change globally. | Accept – see 11-08-xxxx-00-000z |
| 141 | Alex | Ashley | 7.2.2.1.8 | 16 | 3 | T | Y | Clause 7 should not define behaviour. | Remove "The The TDLS AP Path Switch frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 142 | Bill | Marshall | 7.2.2.1.8 | 16 | 1 | T | y | "Reason" field in this frame is confusing, with Reason codes being defined in 7.3.1.7. | Either use reason codes as defined in 7.3.1.7, or change the name of this field to something else | Accept – see 11-08-xxxx-00-000z |
| 143 | Michelle | Gong | 7.2.2.1.8 | 16 | 3 | T | Y | Why is TDLS AP Path Switch request frame sent through the AP? Since the request is sent when the direct link exists, isn't it better, i.e. less delay and more efficient, to sent it over the direct link? If there are important technical reasons that are unknown to me, please add a note to clarify. If not, please remove this requirement. | Either clarify why this frame has to be sent through the AP or allow this frame being sent over the direct link. | Accept – See CID 120 |
| 144 | Tomoko | Adachi | 7.2.2.1.8 | 16 | 1 | T | Y | "… for this conversation."? | Change it to "… for each AP path switch transaction." | Accept – see 11-08-xxxx-00-000z |
| 145 | Alex | Ashley | 7.2.2.1.9 | 16 | 10 | T | Y | Clause 7 should not define behaviour. | Remove "The The TDLS AP Path Switch Response frame shall be sent through the AP." and put it in clause 11. | Accept – see 11-08-xxxx-00-000z |
| 146 | Bill | Marshall | 7.2.2.1.9 | 16 | 8 | T | y | TDLS Path Switch Response frame needs a Status code | Add a Status code to the frame | Accept – see 11-08-xxxx-00-000z |
| 147 | David | Hunter | 7.2.2.1.9 | 14 | 8+ | T | Y | A Response frame needs a Status Code field. | Add a Status Code field, as the second field, parallel to the Response frame's Status Code field, and with the same description as is used in Table z3, OR add a Result field similar to that in tables z9 and z10.. | Accept – see 11-08-xxxx-00-000z |
| 148 | Henry | Ptasinski | 7.2.2.1.9 | 16 | 9 | T | Y | For symmetry with other frame formats, and for future expansion, TDLS AP Path Switch Response should have a Result code field. | Add a Result code field to the frame. Define result value 0 as Accept. Reserve all other values. | Accept – see 11-08-xxxx-00-000z |
| 149 | Michelle | Gong | 7.2.2.1.9 | 16 | 10 | T | Y | Why is TDLS AP Path Switch Response frame sent through the AP? Since the response is sent when the direct link exists, isn't it better, i.e. less delay and more efficient, to sent it over the direct link? If there are important technical reasons that are unknown to me, please add a note to clarify. If not, please remove this requirement. | Either clarify why this frame has to be sent through the AP or allow this frame being sent over the direct link. | Accept – See CID 120 |
| 150 | Michelle | Gong | 7.2.2.1.9 | 16 | 8 | T | Y | Why is there no reason code defined for AP Path Switch Response? Is it too restrictive for a peer STA to have to accept the AP Path Switch? This also relates to the reasons why a STA may want to switch to an AP path. For instance, if a STA doesn't have much traffic to send over the direct link, it may choose to switch to the AP path for energy conservation or other purposes. But its peer STA should be able to make an independent decision based on its own traffic condition or power save states. In a peer-to-peer scenario, two STAs need to collaborate on decision making. | Please either clarify why a peer STA has to accept the Path Switch Request or allow a peer STA to reject the Request and provide a reason code. | Accept – see 11-08-xxxx-00-000z |
| 151 | Adrian | Stephens | 7.2.2.1.10 |  |  | T | N | I don't like the use of the term "backlogged" which implies a blockage. The name of the ACn backlogged fields doesn't match their definition. | Choose a better name (e.g. ACn traffic available) or change the definition. | Accept – see 11-08-xxxx-00-000z |
| 152 | Adrian | Stephens | 7.2.2.1.10 |  |  | T | N | Why are these "backlogged" fields specified in an octet? Which non-zero value is used for "otherwise"? | Define value 0 as empty, 1 as non-empty and 2-255 as reserved. Or move into a bit field and save 3 octets. | Accept – see 11-08-xxxx-00-000z |
| 153 | Adrian | Stephens | 7.2.2.1.10 | 17 | 1 | T | Y | "1 octet field which indicates the minimum interval between successive Peer Traffic Indication frames sent to the same peer, expressed in Beacon Intervals. (Optional)" How is the optionality of this field expressed? | Define the value of zero to mean "unspecified" and in 33.19 replace "present" with "specified". | Accept – see 11-08-xxxx-00-000z |
| 154 | Alex | Ashley | 7.2.2.1.10 | 16 | 19 | T | Y | The baseline does not use the terms AC0 .. AC3. It uses the terms AC_VI, AC_VO, AC_BE and AC_BK. I suggest using the same names as the baseline. | Rename AC0, AC1, AC2 and AC3 to AC_BE, AC_BK, AC_VI, AC_VO. Look in clause 7.3.2.29 table 7-36 for how to map from the numeric values to AC_VO, AC_VI, etc | Accept – see 11-08-xxxx-00-000z |
| 155 | Alex | Ashley | 7.2.2.1.10 | 17 | 4 | T | Y | The baseline does not use the terms AC0 .. AC3. It uses the terms AC_VI, AC_VO, AC_BE and AC_BK. I suggest using the same names as the baseline. | Rename AC0, AC1, AC2 and AC3 to AC_BE, AC_BK, AC_VI, AC_VO. Look in clause 7.3.2.29 table 7-36 for how to map from the numeric values to AC_VO, AC_VI, etc | Accept – see 11-08-xxxx-00-000z |
| 156 | Artur | Zaks | 7.2.2.1.10 | 16 | 19 | T | Y | Table z13, Column Information: AC0, AC1, AC2 are not defined | Define AC0, AC1 and AC2 | Accept – see 11-08-xxxx-00-000z |
| 157 | Artur | Zaks | 7.2.2.1.10 | 17 | 1 | T | Y | Table z13, Column Information: AC0, AC1, AC2 are not defined | Define AC0, AC1 and AC2 | Accept – see 11-08-xxxx-00-000z |
| 158 | Brian | Hart | 7.2.2.1.10 | 16 | 19 | T | Y | 1 byte for a boolean! | "convert ACx backlogged" to a bitfield or use the extra bits within the existing fields | Decline –the octet definition allows for possible future extension to express buffer size, while the additional overhead is small. |
| 159 | Ganesh | Venkatesan | 7.2.2.1.10 | 17 | 4-6 | T | Y | "It is set to zero value if the corresponding AC (AC0, AC1, AC2, AC3) is empty and to a non-zero value otherwise." (a) "set to zero" conveys the same idea as "set to zero value" (b) is there a relation between the non-zero value and how full/empty the corresponding buffer is? | Replace "set to zero value" with "set to zero" Describe what the non-zero value is and how it relates to the state of the buffer. | Counter – See CID 152 |
| 160 | Henry | Ptasinski | 7.2.2.1.10 | 17 | 5 | T | Y | “a non-zero value” should indicate which non-zero value to use, or what different non-zero values mean. | Clarify the interpretation of non-zero values in the Acs backlogged fields. | Counter – See CID 152 |
| 161 | Joseph | Lauer | 7.2.2.1.10 | 16 | 19 | T | Y | Why do we use a whole octet for each AC if all we are only discriminating between 0 and a non-zero value? | Combine the backlogged fields into one octet or justify the use of one octet for each AC. | Counter – See CID 158 |
| 162 | Keith | Amann | 7.2.2.1.10 | 17 | 5 | T | Y | The definition of each "Acs backlogged" field indicates that a non-empty AC will be signaled with a "non-zero value". This is excessively broad for no apparent reason since the field name appears to indicate that this is a binary value. | Replace the text "a non-zero value" with "a value of one (1)". | Accept – see 11-08-xxxx-00-000z |
| 163 | Matthew | Fischer | 7.2.2.1.10 | 16 | 16 | T | Y | Handy, these undefined fields… | Provide some meaningful definition of these fields. Without it, they are worthless. Or at a miminum, do not waste all of these octets if there are only two values to each field. Put all of the AC backlogged indications into single bits within a single octet and define the values of 1 and 0 for each bit to have the meanings of empty and not, or vice versa. | Counter – See CID 152 and CID 158 |
| 164 | Michael | Bahr | 7.2.2.1.10 | 17 | 0 | T | N | According to Annex D, information 6 is the value of dot11peerPSMIndicationWindow, but this MIB variable is not referenced here | reference dot11peerPSMIndicationWindow in row 6 | Accept – see 11-08-xxxx-00-000z |
| 165 | Osama | Aboul-Magd | 7.2.2.1.10 | 16 | 19 | T | Y | What are AC0, AC1 and AC2? | explain | Accept – see 11-08-xxxx-00-000z |
| 166 | Thomas | Kolze | 7.2.2.1.10 | 16 | 16 | T | Y | Undefined fields. | Provide meaningful definition of these fields and/or collapse. | Counter – See CID 152 and CID 158 |
| 167 | Tomoko | Adachi | 7.2.2.1.10 | 16 | 19 | T | Y | "non-zero otherwise" Don't care what ever it is if it is not zero? | Change the wording to something more appropriate. There are 4 places in Table z13 and one place in page 17, line 5 to correct. | Accept – see 11-08-xxxx-00-000z |
| 168 | Tomoko | Adachi | 7.2.2.1.10 | 17 | 1 | T | Y | "(Optional)" in Table z13. Does it mean setting this information in the frame is optional? It seems so from 11.2.1.12. But why does its presence have to be optional? (Why does it needed to be hide sometimes?) | Clarify what is meant by "(Optional)". Or, if there is no important reason to hide it, delete "(Optional)". | Accept – see 11-08-xxxx-00-000z |