IEEE P802.11
Wireless LANs

| PHY Ad Hoc Annex J Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |

Abstract

This document proposes resolutions for comments from the PHY ad hoc: 6067, 6138.

Draft text change instructions based on D4.01.

Introduction

These are the CIDs addressed by this submission.

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6076 | Chan, Douglas | 473.26 | Annex J | Those channels that are subjected to DFS requirements (i.e., those with behavior limits set "4") for regulatory classes in the USA are not properly denoted. | Investigate and amend accordingly. | Counter – The TGn editor shall make the changes shown in document 11-08/0598r0. |
| 6138 | Ecclesine, Peter | 475.01 | Annex J | 11k added RCs 30-32 to table J-3, inserted regulatory classes should begin at 33. | change editing instruction and new classes to be numbered correctly for the amendment's baseline. | Counter – The TGn editor shall make the changes shown in document 11-08/0598r0. |

Discussion – not to be incorporated into the draft:

USA FCC references for 50-100 us DFS requirements may be checked at http://www.atcb.com/publicdocs/FCC-06-96A1.pdf and Japanese ARIB references may be obtained from http://www.arib.or.jp/english/html/overview/st_ej.html.

802.11 Annex J for the USA and Japan is outdated in that it omits latest DFS requirements. This draft makes appropriate updates where required.

TGn Editor: Please insert at least the following rows of table J.1 of IEEE 802.11:2007 into the 11n draft and change the rows as identified below

Table J.1—Regulatory classes in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1, 4 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1, 4 |

TGn Editor: Please change the following rows of table J.1 of D4.01 as identified below.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 5 | 40 | 52, 60 | 200 | — | 1 | 1,4,13 |
| 24 | 5 | 40 | 100, 108, 116, 124, 132 | 200 | — | 1 | 1,4,13 |
| 28 | 5 | 40 | 56, 64 | 200 | — | 1 | 1,4,14 |
| 29 | 5 | 40 | 104, 112, 120, 128, 136 | 200 | — | 1 | 1,4,14 |

Table J.3—Regulatory classes in Japan

TGn Editor: Please insert instructions for deleting row 32 of Table J.3 of the baseline document.

TGn Editor: Please change the following rows of table J.3 of D4.01 as identified below.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (dBm) | Transmit Power Limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32 | 5 | 20 | 52,56,60,64 | - | 10 | 10 | 1 | 1, 2, 3, 4, 5, 6 |
| 33 | 5 | 20 | 52,56,60,64 | - | 10 | 5 | 1 | 1, 2, 4, 5, 6 |
| 34 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 50 | 1 | 1, 3, 4, 5, 6 |
| 35 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 25 | 1 | 1, 4, 5, 6 |
| 36 | 5 | 40 | 36,44 | - | 5 | 5 | 1 | 1, 2,6,13 |
| 37 | 5 | 40 | 52,60 | - | 5 | 5 | 1 | 1, 2, 3, 4, 5, 6, 13 |
| 38 | 5 | 40 | 52,60 | - | 5 | 2.5 | 1 | 1, 2, 4, 5, 6, 13 |
| 39 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 13 |
| 40 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 13 |
| 41 | 5 | 40 | 40,48 | - | 5 | 5 | 1 | 1, 2, 6, 14 |
| 42 | 5 | 40 | 56,64 | - | 5 | 5 | 1 | 1, 2, 3, 4, 5, 6, 14 |
| 43 | 5 | 40 | 56,64 | - | 5 | 2.5 | 1 | 1, 2, 4, 5, 6, 14 |
| 44 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 14 |
| 45 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 14 |
| 46 | 4 | 40 | 184,192 | 24 | 25 | - | 2 | 5, 6, 7, 13 |
| 47 | 4 | 40 | 184,192 | 24 | 25 | - | 2 | 5, 6, 8, 13 |
| 48 | 4 | 40 | 184,192 | 24 | 25 | - | 3 | 5, 6, 7, 13 |
| 49 | 4 | 40 | 184,192 | 24 | 25 | - | 3 | 5, 6, 8, 13 |
| 50 | 4 | 40 | 184,192 | - | 5 | - | 1 | 5, 6, 8, 13 |
| 51 | 4 | 40 | 184,196 | 24 | 25 | - | 2 | 5, 6, 7, 14 |
| 52 | 4 | 40 | 184,196 | 24 | 25 | - | 2 | 5, 6, 8, 14 |
| 53 | 4 | 40 | 184,196 | 24 | 25 | - | 3 | 5, 6, 7, 14 |
| 54 | 4 | 40 | 184,196 | 24 | 25 | - | 3 | 5, 6, 8, 14 |
| 55 | 4 | 40 | 184,196 | - | 5 | - | 1 | 5, 6, 8, 14 |
| 56 | 2.407 | 40 | 1-9 | - | 5 | - | 4 | 10,13 |
| 57 | 2.407 | 40 | 5-13 | - | 5 | - | 4 | 10,14 |