IEEE P802.11
Wireless LANs

| PHY Ad Hoc Annex J Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-527-9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems | 170 West Tasman Drive, San Jose, CA 95134 | 408-525-3346 | brianh@cisco.com |

Abstract

This document proposes resolutions for comments from the PHY ad hoc: 6067, 6138.

Draft text change instructions based on D4.01.

Introduction

These are the CIDs addressed by this submission.

| CID | Commenter | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6076 | Chan, Douglas | 473.26 | Annex J | Those channels that are subjected to DFS requirements (i.e., those with behavior limits set "4") for regulatory classes in the USA are not properly denoted. | Investigate and amend accordingly. | Counter – The TGn editor shall make the changes shown in document 11-08/0598r3. |
| 6138 | Ecclesine, Peter | 475.01 | Annex J | 11k added RCs 30-32 to table J-3, inserted regulatory classes should begin at 33. | change editing instruction and new classes to be numbered correctly for the amendment's baseline. | Counter – The TGn editor shall make the changes shown in document 11-08/0598r3. |

Discussion – not to be incorporated into the draft:

802.11 Annex J for the USA is outdated in that it omits latest DFS requirements.  Furthermore, the changes of Annex J in D4.0.1 did not take into account of the changes made in TGk, which is TGn’s baseline.  This draft makes appropriate updates where required.

TGn Editor: Please insert at least the following rows of table J.1 of IEEE 802.11:2007 into the 11n draft and change the rows as identified below

Table J.1—Regulatory classes in the USA

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1, 4 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1, 4 |

TGn Editor: Please change the following rows of table J.1 of D4.01 as identified below.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (mW) | Transmit Power Limit (EIRP) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 5 | 40 | 52, 60 | 200 | — | 1 | 1,4,13 |
| 24 | 5 | 40 | 100, 108, 116, 124, 132 | 200 | — | 1 | 1,4,13 |
| 28 | 5 | 40 | 56, 64 | 200 | — | 1 | 1,4,14 |
| 29 | 5 | 40 | 104, 112, 120, 128, 136 | 200 | — | 1 | 1,4,14 |

Table J.3—Regulatory classes in Japan

TGn Editor: Please change row 30 of Table J.3 of D4.01 as identified below.   The edits highlighted in yellow are for the baseline incorporated by 11kD13.0.

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (dBm) | Transmit Power Limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3032 | 5.0 | 20 | 52,56,60,64 | 22- | 10 | 10 | 1 | 1, 2, 3, 4, 5, 6 |

TGn Editor: Remove row 30 of Table J.3 in D4.0.1 and edit the insert instructions as follows.  (FYI: The only changes here are in the Regulatory Class column’s numbering and the pink-highlight in the insert instructions.)

Insert the following entries in Table J.3 after the row for Regulatory class 2932, insert the NOTE at the end

of the table and adjust the numbering of the reserved row appropriately:

| Regulatory Class | Channel Starting Frequency (GHz) | Channel Spacing (MHz) | Channel set | Transmit Power Limit (dBm) | Transmit Power Limit (mW/MHz) | EIRP (mW/MHz) | Emissions Limits set | Behavior Limits set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 313 | 5 | 20 | 52,56,60,64 | - | 10 | 5 | 1 | 1, 2, 4, 5, 6 |
| 324 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 50 | 1 | 1, 3, 4, 5, 6 |
| 335 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | - | 10 | 25 | 1 | 1, 4, 5, 6 |
| 346 | 5 | 40 | 36,44 | - | 5 | 5 | 1 | 1, 2,6,13 |
| 357 | 5 | 40 | 52,60 | - | 5 | 5 | 1 | 1, 2, 3, 4, 5, 6, 13 |
| 368 | 5 | 40 | 52,60 | - | 5 | 2.5 | 1 | 1, 2, 4, 5, 6, 13 |
| 379 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 13 |
| 3840 | 5 | 40 | 100, 108, 116, 124, 132 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 13 |
| 3941 | 5 | 40 | 40,48 | - | 5 | 5 | 1 | 1, 2, 6, 14 |
| 402 | 5 | 40 | 56,64 | - | 5 | 5 | 1 | 1, 2, 3, 4, 5, 6, 14 |
| 413 | 5 | 40 | 56,64 | - | 5 | 2.5 | 1 | 1, 2, 4, 5, 6, 14 |
| 424 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 25 | 1 | 1, 3, 4, 5, 6, 14 |
| 435 | 5 | 40 | 104, 112, 120, 128, 136 | - | 5 | 12.5 | 1 | 1, 4, 5, 6, 14 |
| 446 | 4 | 40 | 184,192 | 24 | 25 | - | 2 | 5, 6, 7, 13 |
| 457 | 4 | 40 | 184,192 | 24 | 25 | - | 2 | 5, 6, 8, 13 |
| 468 | 4 | 40 | 184,192 | 24 | 25 | - | 3 | 5, 6, 7, 13 |
| 479 | 4 | 40 | 184,192 | 24 | 25 | - | 3 | 5, 6, 8, 13 |
| 4850 | 4 | 40 | 184,192 | - | 5 | - | 1 | 5, 6, 8, 13 |
| 4951 | 4 | 40 | 184,196 | 24 | 25 | - | 2 | 5, 6, 7, 14 |
| 502 | 4 | 40 | 184,196 | 24 | 25 | - | 2 | 5, 6, 8, 14 |
| 513 | 4 | 40 | 184,196 | 24 | 25 | - | 3 | 5, 6, 7, 14 |
| 524 | 4 | 40 | 184,196 | 24 | 25 | - | 3 | 5, 6, 8, 14 |
| 535 | 4 | 40 | 184,196 | - | 5 | - | 1 | 5, 6, 8, 14 |
| 546 | 2.407 | 40 | 1-9 | - | 5 | - | 4 | 10,13 |
| 557 | 2.407 | 40 | 5-13 | - | 5 | - | 4 | 10,14 |