IEEE P802.11
Wireless LANs

| TGz Draft 1.0 LB127 CID 229-372 (8.5.9.x) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: May 8, 2008 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206, Hillsboro OR 97006 | +1 503 264 3759 | Kapil.sood@intel.com |
| Menzo Wentink | Qualcomm | Straatweg 66, Breukelen, the Netherlands | +31-65-183-6231 | mwentink@qualcomm.com |

Abstract

This document addresses TGz LB127 CID 229-372, which relate to subclause 8.5.9.x.

| 229 | Liwen | Chu | 8 | 21 | 6 | T |  | When two TDLS peer STAs set up security association and enter power saving mode using the AP path, one way to keep security is that they use normal data frames to encapsulate the encrypted frames by TDLS key. The AP does not need to decrypt and encrypt the data frames. | Modify the draft to allow this TDLS peer security. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 230 | Jesse | Walker | 8.5.8.1.3 | 24 | 28 | T | Y | The Peer also needs to verify INonce, KeyId, and Lifetime fields | In the comment | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 231 | Adrian | Stephens | 8.5.9 |  |  | T | Y | "The TDLS Peer Key EAPOL-Key exchange provides a mechanism for obtaining the keys to be used for direct STA-to-STA communication. The initiator STA shall start a timer when it sends the first EAPOL-Key message and the peer STA shall do the same on receipt of the first EAPOL-Key message. On expiration of this timer, the STA shall transition to the STKINIT state." 1. Where is the value of this timer specified? 2. What is the STKINIT state, and which state machine enters this state? | Define the value of this time - perhaps by reference to a MIB variable. Reference the state machine description which includes this state. |  |
| 232 | Adrian | Stephens | 8.5.9 |  |  | T | Y | "A STA should use the TDLS Peer Key Handshake prior to transferring any direct STA-to-STA data frames. The STKSA should be deleted when the STA to STA connection is terminated." This is over general. | 1. Exclude the normal DLS case. 2. Exclude the case when one or more of the AP and both STAs do not support RSN. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc.. The TDLS security protocol is specific to Tunnel DLS, and applicable only when both STAs support RSN. |
| 233 | Bill | Marshall | 8.5.9 | 21 | 14 | T | y | initiator is not starting a PeerKey handshake, its starting a TDLS PeerKey handshake | change "starts the PeerKey Handshake" to "starts the TDLS PeerKey Handshake" | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc.. |
| 234 | Bill | Marshall | 8.5.9 | 21 | 25 | T | y | these are not assumptions, they are just notation | change line to "The following notation is used in the TDLS PeerKey description:" | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc.. |
| 235 | Bill | Marshall | 8.5.9 | 21 | 27 | T | y | as the time of the approval of this amendment, 802.11r won't exist -- all of its changes will be incorporated into the base document that TGz is amending | delete "of 802.11r-D7.0" | Accept. |
| 236 | Clint | Chaplin | 8.5.9 |  |  | T | Y | I'm uncomfortable with the term "public value" as used in this draft. It's such a generic term, and has absolutely no descriptive element in it. And why is the variable usually called "DH"? What does "DH" stand for? | "public key value"? | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc., where Diffie Hellman (DH) has been removed. |
| 237 | Ganesh | Venkatesan | 8.5.9 | 21 | 32 | T | Y | [KS] Both lines 32 and 33 have same STA-I | Change line 32 to Initiator STA an line 33 to Peer STA | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc.. |
| 238 | Ganesh | Venkatesan | 8.5.9 | 21 | 46 | T | Y | [KS] Clause 8.5.3 defines the 4-way handshake as between a supplicant and an Authenticator. There are certain security properties of the roles of these 2 end-points. In the TDLS peer key handshake, it is not clear which end point is playing which of the 2 roles. | Clarify the roles and assumptions of the 2 security endpoints. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 239 | Henry | Ptasinski | 8.5.9 | 21 | 13 | T | Y | “The AP must ...setup” implies that the AP is actively participating in the TDLS setup. | Change to “Prior to PeerKey setup, each STA shall establish an RSNA with the AP” | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 240 | Henry | Ptasinski | 8.5.9 | 21 | 16 | T | Y | BSSID is sent in the exchange, but never verified. | Add a requirement that the BSSID be validated (same in all messages and matches the current association). | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. where BSSID is bound to the keys. |
| 241 | Henry | Ptasinski | 8.5.9 | 21 | 18 | T | Y | “shall start a timer” doesn't provide any details about the timer. | Provide details here or a reference to clause 8.5.9.2 (assuming that's the timer referenced here). |  |
| 242 | Henry | Ptasinski | 8.5.9 | 21 | 22 | T | Y | “A STA should use the TDLS Peer Key Handshake prior to transferring ...” is insufficiently strict. | Change to “When using TDLS with an AP with which the STA has an RSNA, the STA shall use the TDLS Peer Key Handshake prior to transferring ...” | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 243 | Hideyuki | Suzuki | 8.5.9 | 21 | 10 | T | Y | Although TDLS Peer Key Handshake has been combined with 3-way TDLS setup handshake, so this description is inaccurate. | Rewrite this description appropriately. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 244 | Hideyuki | Suzuki | 8.5.9 | 21 | 10 | T | Y | Either STA should initiate the handshake first or both can initiate at the same time. | TDLS Peer Key Handshake needs to be fixed from the viewpoint of peer to peer model in order to support simultaneity. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 245 | Hideyuki | Suzuki | 8.5.9 | 21 | 17 | T | Y | TDLS Peer Key Handshake does not use EAPOL-Key exchange. | Rewrite the description of TDLS Peer Key Handshake appropriately. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 246 | Hideyuki | Suzuki | 8.5.9 | 21 | 20 | T | Y | STKINIT is the state of the PeerKey Handshake Supplicant key management state machine. | Define TDLS Peer Key Handshake state machine and use it. |  |
| 247 | Hideyuki | Suzuki | 8.5.9 | 21 | 45 | T | Y | To my understanding, all of the information which is necessary to build a STK are already exchanged through SMK Handshake, so the initiator STA might not explicitly need to initiate the 4-Way STK Handshake. | Rethink whether 4-Way handshake should be combined into 3-Way TDLS Setup Handshake then rewrite this description of the 4-Way STK Handshake appropriately. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. 4-way handshake is redundant and removed. |
| 248 | Hideyuki | Suzuki | 8.5.9 | 22 | 6 | T | Y | TDLS Peer Key Handshake has been combined with 3-way TDLS setup handshake, so the word "first message" should be modified appropriately. | "first message" should be changed to "TDLS Setup Request" or SMK Message 1. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 249 | Jesse | Walker | 8.5.9 | 21 | 10 | T | Y | The TDSL Peer Key Handshake seems to me to be needlessly heavyweight and non-extensible. It seems to me that a much lighter-weight protocol can be devised within the security model adopted by 802.11z. In particular, 802.11z assumes that the AP is trusted to not compromise the TDSL messages or utilize any of this communications to compromise the resulting DL key. | Under the trust assumptions 802.11z is already making about the AP, a design that (a) requires the AP to enforce RSN-protected links (already explicitly required on p.21 line 13-14) and (b) the Initiator and the Peer exchange random nonces used to derive a session key using the 802.11i KDF suffices. See 11-08--0476 as an example protocol meeting this description. Replacing 8.5.9 with 11-08-0476 or some like proposal would address my concerns with the existing 8.5.9. This will reduce the network and CPU bandwidth by orders of magnitude without degrading security one bit. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 250 | Jesse | Walker | 8.5.9 | 21 | 10 | T | Y | Since the intent is to reuse the 4-Way Handshake, which is controlled by the SME and 802.1X, the relation between the TDLS Peer Key Handshake on one hand and the SME and 802.1X has to be worked through in a way that will be interoperable. No text regarding these relationship exists currently | Add a model explaining how the SME invokes the TDLS Peer Key Setup protocol, how the necessary interactions with 802.1X happen, such as relaying the keys created by the protocol, occur |  |
| 251 | Jesse | Walker | 8.5.9 | 21 | 13 | T | N | "…data confidentiality…" 802.11 does not support any data confidentiality-only cipher suites. | "…data confidentiality and integrity…" | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 252 | Jesse | Walker | 8.5.9 | 21 | 13 | T | Y | "The AP must establish…prior to PeerKey setup." While I entirely condone the sentiment behind this, err, uh, requirement, there are two problems with describing the functionality this way. The first 802.11z is not suppoed to change APs, and they currently don't implement this check. The second is that the check needs to be at the STA, not the AP, to be effective from a security perspective. | Replace the offending sentence with something like "A STA shall establish an RSNA-protection association with the AP prior to initiating Peer Key setup. The STA shall not initiate or respond to the PeerKey setup messages unless the AP's RSN information element omits WEP-40 and WEP-104 as enabled pairwise and group cipher suites, and the 'No Pairwise' subfield of the RSN capabilities field is 0." | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 253 | Kevin | Hayes | 8.5.9 | 21 | 37 | T | Y | Add the following to the assumption list: The AP is trusted by both STA peers. | See comment. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 254 | Kevin | Hayes | 8.5.9 | 21 | 42 | T | Y | "This message exchange goes through the AP and is …" This is supposed to be a specification, not a loose colloquial description. | Please precisely describe how the message exhange is transported, describing the identity of all parties. Or delete the text. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc., where endpoints are identified by their MAC addresses. |
| 255 | Matthew | Fischer | 8.5.9 | 21 | 12 | T | Y | Does the phrase "direct link setup" now include TDLS? That relationship is implied in this sentence, but does it hold everywhere in this draft as well as in the baseline? | At a minimum, redefine DLS to include TDLS if that is appropriate, otherwise, make certain that references to direct links are explicit about which type of direct links are applicable (e.g. TDLS DLS vs non-TDLS DLS) | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc., where the security protocol clearly applies only to TDLS. |
| 256 | Matthew | Fischer | 8.5.9 | 21 | 12 | T | Y | The comment is intended to apply to all of the subclauses of 8.5.9. General grammar issues and ambiguous references to "STA" | Throughout 8.5.9 and all of its subclauses, find and fix missing articles, add adjectives to "STA" as appropriate, make verb conjugates match and make them correct for the context. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 257 | Meiyuan | Zhao | 8.5.9 | 21 | 14 | T | Y | It is assumed that the initiator STA starts the TDLS Peer Key Handshake. However, it is not clear from TDLS execution that there's only one initiator STA. It could be the two STAs initiate their own instance of TDLS simultaneously. If it is allowed to have both initiators to start peer key handshake, the two STAs end up with two sets of keys. Even with a mechanism to drop a set of key, the simultaneous execution of two Peer Key Handshake protocol instances can enable inter-leaving attacks. | Current design of peer key handshake is a hack of a client/server protocol for peer to peer communication. The design philosophy is incorrect. To make it work, at least carefully analyze TDLS protocol to clearly define roles to enable only one peer key handshake instance. A better approach is to re-design the peer key handshake protocol that is directly suitable for peer to peer setting. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc., where handling of simultaneous security handshakes is specified. |
| 258 | Nancy | Cam-Winget | 8.5.9 | 21 | 27 | T | Y | This first item does not need to be an assumption as TGz succeeds TGr so it should be implied that FT and FTIE are already defined in the base 802.11 specification. | Remove this line. | Accept in principle. |
| 259 | Nancy | Cam-Winget | 8.5.9 | 21 | 43 | T | Y | What PTK? If each Peer STA is communicating via the AP, then their messages are protected by different PTK's; e.g. the STA_I's PTK shared with the AP and the STA_P's PTK shared with the PTK. | Either remove or clarify this sentence. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 260 | Stephen | Emeott | 8.5.9 | 21 | 12 | T | Y | According to clause 11.z1, the peer key handshake seems to begin with the first message in the tunneled direct link setup | Change sentence to read "The TDLS Peer Key Handshake begins with the first message in the directl link setup procedure (11.z1). | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 261 | Stephen | Emeott | 8.5.9 | 21 | 17 | T | Y | The second paragraph refers to a TDLS Peer Key EAPOL-Key exchange, when the exchange seems to occur using ethertype 89-0d frames. | Remove references to the EAPOL-key frame and refer instead to the ethertype 89-0d frames | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. EAPOL-Key exchanged removed. |
| 262 | Stephen | Emeott | 8.5.9 | 21 | 43 | T | Y | The message exchange is protected "using either the PTK of the initiator STA or the PTK of the peer STA, depending upon whether the message is being transmitted to/from the AP" | Make the suggested change | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 263 | Tomoko | Adachi | 8.5.9 | 21 | 10 | T | Y | Is public key used? Can't common key cryptosystem be used to ease the implementation (reduce the amount of calculation)? | Use common key cryptosystem. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc., using symmetric key crypto. |
| 264 | Adrian | Stephens | 8.5.9.1 |  |  | T | N | "1406 pi" The operation and the value of pi are undefined. | Either replace pi with the Greek symbol of that name, or add a multiplication symbol and define "pi". | Accept. DH is removed. |
| 265 | Adrian | Stephens | 8.5.9.1 |  |  | T | Y | In documenting the value of the prime, is the most significant byte first or last. Is the most significant nibble first or last. Is it read left to right first, or top to bottom first? | Define how to interpret this block of characters as a number. | Accept. DH is removed. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 266 | Bill | Marshall | 8.5.9.1 | 22 | 28 | T | y | it is unclear what is meant by the first parameter shown in the FTIE. Values 0, 7, and 4 are used in the three messages of the sequence | add an explanation of this value. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 267 | Bill | Marshall | 8.5.9.1 | 22 | 31 | T | y | Lifetime and DH_P are not contained in the FTIE | move the right paren (ending the FTIE contents) to follow the BSSID | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using TIE and removed DH. |
| 268 | Bill | Marshall | 8.5.9.1 | 22 | 34 | T | y | Lifetime and DH_I are not contained in the FTIE | move the right paren (ending the FTIE contents) to follow the BSSID | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using TIE and removed DH. |
| 269 | Brian | Hart | 8.5.9.1 | 22 | 7 | T | Y | "Imitator"? Or initiator? | Check & fix if needed | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 270 | Ganesh | Venkatesan | 8.5.9.1 | 22 | 4 | T | Y | [KS] The use of a public-key exchange adds a new component to 802.11 security which has been avoided, largely due to the computational time and latency needed on low-power 802.11 devices. In addition, this requires a significant non-incremental component in traditional device supplicants. The security of DH approach is still susceptible to MITM attacks. These MITM attacks can be limited to those that can be launched on the AP (assuming 802.11i mechanisms exists on 2 links). So, DH approach is insufficient, and largely an overkill. | Define the security goals for the Peer-key handshake. Adopt the resolution presented in 11-08-0476-00 (version number may change). | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 271 | Ganesh | Venkatesan | 8.5.9.1 | 22 | 11 | T | Y | [KS] The use of a 1536-bit based group is insufficient for matching the strength of a 128-bit AES crypto. RFC 3526 states "For the 128-bit AES we need about a 3200-bit group [Orman01]". If DH is to be used, then use a higher-bit group. | If TGz decides to continue using DH scheme, then use a higher-bit group (4096-bit or higher) to match cryptographic strength of 128-bit AES. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 272 | Hideyuki | Suzuki | 8.5.9.1 | 22 | 28 | T | Y | FTIE does not have PNonce field or INonce field. Although FTIE has ANonce and SNonce field, each fields are dedicated for the value chosen by R1KH or S1KH. | Define new IE for SMK Handshake or modify the definition of FTIE appropriately. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using ANonce and Snonce fields of FTIE. |
| 273 | Jesse | Walker | 8.5.9.1 | 22 | 11 | T | Y | Why do we believe that a 1536 bit prime will meet the security needs of the application beyond 2015? When we have to increase the size of the Diffie-Hellman group to 3K bits = 384 octets (to meet FIPS 140-2 in 2010), the parameters will no longer fit in a single IE. Why do we believe that we can meet the power consumption requirements of battery powered devices with a prime this large (roughly 34 million cycles per operation)? Why is there no extensibility mechanism to replace this Diffie-Hellman group with other ones as need demands. While Diffie-Hellman is a lovely algorithm, no attempt has been made to justify its use for this application. | As far as I can tell, a non-Diffie-Hellman algorithms, such as the one suggested in 11-08-0476, will provide the same level of security at a computational cost that is ***4 ORDERS OF MAGNITUDE*** lower per key establishment. Replace the current Diffie-Hellman approach with a cheaper algorithm, such as that in 11-08-0476. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 274 | Jesse | Walker | 8.5.9.1 | 22 | 31 | T | Y | FTIE Mic Control field is 2 octets; MIC is 16 octets; PNonce is 32 octets; INonce is 32 octets; MAC_I is 6 octets + 2 octets overhead, MAC_P is 6 octets + 2 octets overhead; BSSID is 6 octets + 2 octets overhead; Lifetime is ?? octets + 2 octets overhead; DH_P is 192 octets + 2 octets overhead. 2+16+32+32+6+2+6+2+6+2+??+2+192+2 = 302+?? > 255 = maximum number of octets in an 802.11 IE. It is unclear why the FTIE needs to carry the DH_P, since 7.3.2.z3 defines a separate IE for this purpose. Should the Lifetime be given by the Timeout Interval IE (7.3.2.49 in 802.11r)? | Some surgery is required, even if TGz decides to stick with the current scheme instead of replacing it with something cheaper. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 275 | Jesse | Walker | 8.5.9.1 | 22 | 31 | T | N | While I understand that the document assumes that the AP will not launch man-in-the-middle attacks against the SMK protocol, it really bothers me a great deal that message 2 does not include both DH_I and DH_P. It seems as though this makes man-in-the-middle attacks even more attractive by failing to bind all of the parameters properly. It also gives the crypto community an even more attractive target for ridiculing our collective efforts publicly. Isn't avoidance of further public relations disasters over inept our continuing inept use of cryptography a consideration? | In the comment | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 276 | Jesse | Walker | 8.5.9.1 | 22 | 34 | T | N | It probably doesn't matter that the initiator repeat the DH_I in message 3--the two parties can discover they have agreed on this parameter from the MIC. | No change is required, but the 802.11 bit police are usually looking for ways to reduce message overheads... | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 277 | Jesse | Walker | 8.5.9.1 | 22 | 28 | T | Y | I am confused again. The updates P802.11z D1.0 makes to 7.3.2.46 does not include additions for the Lifetime and DH_I fields, yet the message format on line 28 indicates these are indeed subfields of FTIE. Why is DH_I needed here, as it has its own subfield defined in 7.3.2.z2? What is the Lifetime subfield, and how large is it? Note that 7.3.2.46 does NOT give you the option to elide the PNonce subfield to save space, as this message has done. | Please utilize data structures consistently | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 278 | Jesse | Walker | 8.5.9.1 | 22 | 28 | T | Y | I am even more confused. Since SMK messages have their own Ethertype, they really need their own extensible format. I don't object to utilizing 802.11 IEs to convey information, but the formats for this new application protocol are entirely implicit. (The formats in 7.2 are NOT extensible, because a legacy system isn't given a clue about how to deal with extensions developed after deployment) | Please create a new clause defining SMK messages as first class objects. Include a version distinguisher, and define the MANDATORY semantics around handling the version distinguisher, to make the protocol extensible in the future. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 279 | Joseph | Epstein | 8.5.9.1 | 22 | 14 | T | N | The formula should be given in proper mathematical symbols, rather than the pseudoformulation from the RFC. | Replace "pi" with the greek letter, show the exponentiations through superscripting, and replace the curly braces. The use of ^ can be considered unclear without an explanation. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 280 | Jouni | Malinen | 8.5.9.1 | 22 | 25 | T | Y | The SMK Handshake for TDLS seems to be using an unauthenticated Diffie-Hellman exchange to generate SMK. This is potentially open to man-in-the-middle attacks. Even though it is not mentioned in the draft, I would assume that the pre-existing encrypted connection between the AP and STAs is used as an implicit “authentication” for the SMK Handshake and DH. However, this may result in decreased security guarantees and potential misconception on how secure the direct link is. Would it be possible to end up using TKIP to protect the AP-STA links and negotiate CCMP for the direct link? In such a case, the SMK and STK derived for CCMP would depend on security of TKIP (i.e., if TKIP is broken, man-in-the-middle attack against the DH in SMK Handshake could be possible). I don't consider this to be major issue since the STAs have already agreed to use the AP for the data connection. However, I think it would be useful to explicitly spell out the assumptions about the AP security and risks involved if the STA-AP-STA link is using less secure encryption than the direct link. | Consider adding a paragraph that lists the assumptions on the AP behavior for the security of the tunneled SMK Handshake (including the use of the AP connection as an implicit authentication for DH and affect of TKIP as the STA-AP encryption on the security of key generation for possible CCMP protected direct link). | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed, and security assumptions added. |
| 281 | Matthew | Fischer | 8.5.9.1 | 22 | 7 | T | Y | Wrong word | Imitator should be initiator (not sure about case) | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 282 | Meiyuan | Zhao | 8.5.9.1 | 22 | 28 | T | Y | Message 1 should include the MAC_P as well to specify the recepient of the message. Otherwise, there's increase vulnerability to denial of service attack | Add MAC_P in message 1 of SMK handshake | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 283 | Meiyuan | Zhao | 8.5.9.1 | 22 | 4 | T | Y | The design of SMK Handshake uses a variant of Diffie-Hellman key exchange protocol. However, this design fails to provide authenticity of the two parties. MIC protection using the key derived from the secret key is not sufficient to provide authenticity. | Modify the design to provide authenticity. The design could be similar to STS protocol which provide party authenticity. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 284 | Meiyuan | Zhao | 8.5.9.1 | 22 | 31 | T | Y | To help authenticity and session binding, message 2 should include both DH_P and DH_I | Add DH_I in message 2 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 285 | Meiyuan | Zhao | 8.5.9.1 | 22 | 34 | T | Y | To help authenticity and session binding, message 3 should include both DH_P and DH_I | Add DH_P in message 3 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 286 | Nancy | Cam-Winget | 8.5.9.1 | 22 | 14 | T | Y | By the indication of a modulo prime indicated in this sentence, an individual who is knowledgeable in crypto could discern that some public key operation is to take place (and judging by the use of DH and RFC 3526, that it is possibly a Diffie Hellman exchange)….but nowhere, are these constructs introduced, nor is there any explanation for how this parameter applies and what it is trying to achieve. While we could extract this from further reading the message descriptions, an introductory paragraph of how the SMKSA is validated? uses this to derive keys? is required to make sense of this. | In the comment. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 287 | Stephen | Emeott | 8.5.9.1 | 22 | 27 | T | Y | The FTIE does not take Lifetime nor DH_I as arguments | Modify the message definition to put a ")" after BSSID. Same goes for the FTIE() arguments in message 2 and message 3 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using TIE and removed DH. |
| 288 | Bill | Marshall | 8.5.9.1.1 | 22 | 38 | T | y | lack of normative language in this clause | change "STA performs" to "STA shall perform" | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 289 | Bill | Marshall | 8.5.9.1.1 | 22 | 40 | T | y | RSNIE is not in this message | change to RSNIE_I |  |
| 290 | Bill | Marshall | 8.5.9.1.1 | 22 | 41 | T | y | Very awkward statement regarding the group cipher suit field | change to "The Group cipher suite shall be set to zeroes." | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 291 | Bill | Marshall | 8.5.9.1.1 | 22 | 45 | T | y | Key Nonce field is undefined | change "sent in Key Nonce field" to "sent in the Inonce field" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using Anonce/Snonce from FTIE. |
| 292 | Bill | Marshall | 8.5.9.1.1 | 21 | 34 | T | y | Sice the FTIE definition uses the terms ANonce and SNonce, there is potential confusion here since TGz is ugins INonce and PNonce. | make this association clear; suggest changing lines 34 and 35 to "INonce is the nonce generated by STA_I and stored in the SNonce field of the FTIE", and similar change to line 35 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using Anonce/Snonce from FTIE. |
| 293 | Bill | Marshall | 8.5.9.1.1 | 22 | 47 | T | y | "as part of Lifetime" doesn't specify where/how to include this information | clarify | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Lifetime clearly included in TIE. |
| 294 | Bill | Marshall | 8.5.9.1.1 | 22 | 49 | T | y | extraneous word "public" on this line | delete "public" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH deleted. |
| 295 | Bill | Marshall | 8.5.9.1.1 | 22 | 51 | T | y | lack of normative language in this clause | change "STA sends" to "STA shall send" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 296 | Bill | Marshall | 8.5.9.1.1 | 22 | 51 | T | y | peer STA_P is confusing | change to "to the peer STA through the AP" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 297 | Bill | Marshall | 8.5.9.1.1 | 22 | 52 | T | y | lack of normative language in this clause | change "STA starts" to "STA shall start" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 298 | Bill | Marshall | 8.5.9.1.1 | 22 | 3 | T | y | there may be several existing direct links. | change "against existing direct link" to "against existing direct links" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Simultaneous operation addressed. |
| 299 | Bill | Marshall | 8.5.9.1.1 | 22 | 3 | T | y | there may be several existing direct links. | change "If no direct link exists" to "If no matching direct link exists" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Simultaneous operations addressed. |
| 300 | Bill | Marshall | 8.5.9.1.1 | 22 | 3 | T | y | unclear pronoun "it" - which STA?? | change "it" to "the peer STA" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Clarified. |
| 301 | Bill | Marshall | 8.5.9.1.1 | 22 | 3 | T | y | lack of normative language in this clause | change "silently discards" to "shall silently discard" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 302 | Bill | Marshall | 8.5.9.1.1 | 22 | 6 | T | y | lack of normative language in this clause | change "If all check succeed" to "If all checks succeed, the peer STA shall" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 303 | Bill | Marshall | 8.5.9.1.1 | 22 | 8 | T | y | the random number isn't sent yet | change "sent" to "placed" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 304 | Bill | Marshall | 8.5.9.1.1 | 22 | 8 | T | y | Key Nonce field is undefined | change "in Key Nonce field" to "in the PNonce field" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using FTIE nonce fields. |
| 305 | Bill | Marshall | 8.5.9.1.1 | 22 | 10 | T | y | "used" is unclear | change "used" to "placed" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 306 | Bill | Marshall | 8.5.9.1.1 | 22 | 11 | T | y | extraneous word "public" on this line | delete "public" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 307 | Bill | Marshall | 8.5.9.1.1 | 22 | 21 | T | y | this is not a single large integer value, but rather an octet string. | change to 0x53 0x4d 0x4b 0x20 etc | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 308 | Bill | Marshall | 8.5.9.1.1 | 22 | 23 | T | y | "the initiator RSNIE" is unclear | change to RSNIE_I | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 309 | Bill | Marshall | 8.5.9.1.1 | 22 | 25 | T | y | lack of normative language in this clause | change "sends" to "shall send" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 310 | Bill | Marshall | 8.5.9.1.1 | 22 | 25 | T | y | handling of message 2 in 8.5.9.1.2 is checking a MIC, but the MIC is not being calculated | add the normative procedures to calculate the MIC | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 311 | Clint | Chaplin | 8.5.9.1.1 | 23 | 21 | T | Y | Defining a string as "0x534d4b204b65792044657269766174696f6e" is ambiguous and can lead to incompatibilities | define as "0x53 0x4d 0x4b 0x20 0x4b 0x65 0x79 0x20 0x44 0x65 0x72 0x69 0x76 0x61 0x74 0x69 0x6f 0x6e" | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 312 | Ganesh | Venkatesan | 8.5.9.1.1 | 22 | 49 | T | Y | [KS] The DH is computed using exponentiation operation, as opossed to the multiplication operation suggested on this line | If group decides to use DH, then change DH_I = g^A mod p | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 313 | George | Vlantis | 8.5.9.1.1 | 23 | 3 | T | Y | In the case of TDLS, the check of the MAC address in step (a) does not make a lot of sense. | Please clarify the method with TDLS. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MAC addresses are bound to key derivation. |
| 314 | Henry | Ptasinski | 8.5.9.1.1 | 22 | 42 | T | Y | The group cipher suite identifier use in the exchange should be well defined. | Define a new cipher suite to represent “no group-addressed data allowed on this link”, add it to table 7-32 in the base standard, and use it in the TDLS handshake. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Specify that TDLS does not use group communication. |
| 315 | Henry | Ptasinski | 8.5.9.1.1 | 22 | 52 | T | Y | “starts a timer” doesn't provide sufficient details about the timer. | Provide details here or a reference to clause 8.5.9.2 (assuming that's the timer referenced here). |  |
| 316 | Henry | Ptasinski | 8.5.9.1.1 | 23 | 5 | T | Y | The BSSID should be verified. | After step (a), add a check that the BSSID matches the current association. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. BSSID is tied into the key derivation. |
| 317 | Hideyuki | Suzuki | 8.5.9.1.1 | 23 | 3 | T | Y | TDLS Peer Key Handshake has been combined with 3-way TDLS setup handshake. So basically there is no TDLS direct link at this stage. | Remove this sentence or separate SMK Handshake from 3-Way TDLS Setup Handshake. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 318 | Jesse | Walker | 8.5.9.1.1 | 22 | 42 | T | Y | "…with any value…" I will continue to vote "Do Not Approve" as long as this is any value but the group cipher suite for the BSS; it will confuse other members of the BSS too much otherwise. I will also continue to vote "Do Not Approve" if the draft allows this to assume the values WEP-40 or WEP-104. Hasn't the industry suffered enough over WEP to not forbid its use for new functions? | Require the initiator to set the group cipher suite to the value equal to that used by the BSS. Also require that the initiator SHALL NOT initiate the SMK Handshake and the receiver SHALL reject SMK Handshake messages if the Group Cipher Suite is WEP-40 or WEP-104 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 319 | Jesse | Walker | 8.5.9.1.1 | 22 | 45 | T | Y | "…the Key Nonce field…" Clause 7.3.2.46 fails to define a KeyNonce field for the FTIE. | Please utilize data structures consistently | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Anonce/Snonce from FTIE are used. |
| 320 | Jesse | Walker | 8.5.9.1.1 | 22 | 47 | T | Y | "…of Lifetime." Clause 7.3.2.46 fails to define a Lifetime field for the FTIE | Please utilize data structures consistently | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. TIE is used for lifetime. |
| 321 | Jesse | Walker | 8.5.9.1.1 | 22 | 47 | T | Y | As a rule, we should make every effort so that our designs do not make denial-of-service any worse. The Lifetime fails to give an guidance avoiding this. | Define a minimum time for the Lifetime paramater. 5 minutes (300 seconds) seems like a reasonable lower bound--if anyone needs to establish keys any faster than this, something is wrong!!! | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 322 | Jesse | Walker | 8.5.9.1.1 | 22 | 49 | T | Y | "…a random integer A < p" | Require A to be at least 224 bits in length (at least one non-zero bit for bits >= 223, where the bits are numbered 0,1,2,...,n). The custom today is to make A 160 bits in length. This size will be too small to provide assurance against attacks based on the Special Number Seive by 2010. 224 bits will give us a safety margin until about 2020, although it will make performance suck even more than it would otherwise. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH Removed. |
| 323 | Jesse | Walker | 8.5.9.1.1 | 23 | 3 | T | Y | Two other checks should be required: verify that the lifetime is not too short and verify that DH_I is actually a member of the group instead of a value generated randomly by an attacker; if you really, really want to do Diffie-Hellman, do it right, or else you're wasting everyone's time. This check can be done using the elementary identity DH_I^p mode p = 1. | In the comment | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 324 | Jesse | Walker | 8.5.9.1.1 | 23 | 6 | T | Y | What happens if one of the checks fails? | Specify mandatory failure processing | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Falure processing specified in detail. |
| 325 | Jesse | Walker | 8.5.9.1.1 | 23 | 22 | T | Y | You'll find it is useful in real life to generate a key identifier and include this in the protocol--indeed, the 4-Way Handshake REQUIRES that you do this. | A perfectly good key identifier is Truncate-128(SHA-256(BSSID ¦¦ MAC_I ¦¦ MAC_P ¦¦ INonce ¦¦ PNonce)), on the assumption that at least one of <INonce, PNonce> has been generated randomly as per the spec. Messages 2 and 3 should convey this key identifier, so that the two parties know that they agree on the key to use. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Key ID introduced. |
| 326 | Jouni | Malinen | 8.5.9.1.1 | 22 | 49 | T | Y | Diffie-Hellman uses exponentiation, not multiplication, modulo p. | Replace “DH_I = gA mod p” with “DH_I = gA mod p” (i.e., use subscript for 'A'). | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 327 | Kevin | Hayes | 8.5.9.1.1 | 22 | 47 | T | Y | "sent as part of Lifetime" How is it sent? Is it sent in my Lifetime? In your Lifetime? | Please precisely describe how the lifetime of the SMKSA is communicated. Or delete the sentence. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using TIE. |
| 328 | Kevin | Hayes | 8.5.9.1.1 | 22 | 49 | T | Y | gA mod p | gA mod p | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 329 | Kevin | Hayes | 8.5.9.1.1 | 23 | 3 | T | Y | "Verify…against existing direct link." What is meant by this? | Please precisely describe what to verify. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 330 | Kevin | Hayes | 8.5.9.1.1 | 23 | 6 | T | Y | "If all checks succeed" What checks? The one that is not precisely described in line 3 above? Please list any other checks and if no others, then lose the plural on 'checks'. | See comment. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Failure cases specified. |
| 331 | Kevin | Hayes | 8.5.9.1.1 | 23 | 26 | T | Y | The calculation of the MIC is not explained or even mentioned here. Yet the next step says to check that is is correct. What gives? | Add precise description of MIC calculation. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 332 | Liwen | Chu | 8.5.9.1.1 | 23 | 3 | T | Y | On receipt of SMK Handshake Message 1, the peer STA verifies the initiator MAC address against existing direct link. But actually SMK messages can be carried in TDLS setup messages. This should be fixed. | Use the method proposed in 11-08/290 to fix the problem. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MAC addresses are bound inot this protocol. |
| 333 | Matthew | Fischer | 8.5.9.1.1 | 22 | 38 | T | Y | grammar issues with these sentences - missing articles in quite a few places and the STA nouns could use a few adjectives to clarify which STA -also, the verbs at each subbullet are inconsistent in conjugation | Find and fix missing articles, add adjectives to "STA" as appropriate, make verb conjugates match and make them correct for the context. Note that subbullet d) includes both a noun and verb in its predicate, while the previous subbullets use the common noun of the preceding sentence - make all subbullets consistent for improved readability. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 334 | Stephen | Emeott | 8.5.9.1.1 | 22 | 47 | T | Y | The Lifetime should be packed in some type of IE | Make the suggested change | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Using TIE |
| 335 | Stephen | Emeott | 8.5.9.1.1 | 22 | 36 | T | Y | The construction of message 1-3 should be more concisely described, similar to how the construction of the STKSA rekeying EAPOL-Key message is described in 8.5.9.3 to avoid any confusion about the fields in the frame. | Make the suggested change | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Message clearly defined. |
| 336 | Jesse | Walker | 8.5.9.1.1 | 23 | 9 | T | Y | "…a smaller lifetime…" | Again, there needs to be some normative prohibition against picking a lifetime that is too small, e.g., below 300 seconds. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 337 | George | Vlantis | 8.5.9.1.2 | 23 | 35 | T | Y | In the case of TDLS, the checks of the MAC addresses in steps (a) and (b) do not make a lot of sense. | Please clarify the method with TDLS. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MAC addresses are bound into key derivation. |
| 338 | Henry | Ptasinski | 8.5.9.1.2 | 23 | 30 | T | Y | “starts a timer” doesn't provide sufficient details about the timer. | Provide details here or a reference to clause 8.5.9.2 (assuming that's the timer referenced here). |  |
| 339 | Henry | Ptasinski | 8.5.9.1.2 | 23 | 37 | T | Y | The BSSID should be verified. | After step (a), add a check that the BSSID matches the current association and the BSSID provided in Message 1. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. BSSID tied into the key derivation. |
| 340 | Hideyuki | Suzuki | 8.5.9.1.2 | 23 | 35 | T | Y | TDLS Peer Key Handshake has been combined with 3-way TDLS setup handshake. So basically there is no TDLS direct link at this stage. | Remove this sentence or separate SMK Handshake from 3-Way TDLS Setup Handshake. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 341 | Jesse | Walker | 8.5.9.1.2 | 23 | 39 | T | Y | There are again checks missing. | Require the initiator to check that (a) the Lifetime is not too small, (b) that the receiver has not increased the lifetime originally proposed, and (c) that DH_P is actually a group element by verifying DH_P^p mod p = 1. It wouldn't hurt to check the key identifier, too, although presently this has not been defined and included in the message. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Checks are added. |
| 342 | Jesse | Walker | 8.5.9.1.2 | 23 | 41 | T | Y | What happens if one of the checks fails? | Specify mandatory failure processing | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Mandatory behavior specified. |
| 343 | Jesse | Walker | 8.5.9.1.2 | 24 | 13 | T | N | I would include the (presently undefined) KeyID in the MIC calculation as well--it will make debugging easier for implementers and doesn't hurt security one bit | In the comment | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. All binding ingredients of KeyID are included in the MIC. |
| 344 | Jesse | Walker | 8.5.9.1.2 | 24 | 15 | T | Y | What happens if the MIC check fails? | Specify mandatory failure processing | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 345 | Keith | Amann | 8.5.9.1.2 | 23 | 35 | T | Y | The text states that on receipt of message 2 the initiator STA performs a check verifying whether a direct link exists, and if not then it silently discards the message. This is a chicken and egg problem since the direct link doesn't exist until the negotiation of the security message is completed. | Clarify the text to indicate that "no direct link exists" means that the station is also not in the process of attempting to construct a direct link with the peer. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Messages 2 and 3 are MIC-verfied and failure cases identified clearly. |
| 346 | Keith | Amann | 8.5.9.1.2 | 23 | 35 | T | Y | The text indicates that in the event of an error the messages should be silently discarded, but clause 8.5.8.4.4 states that when a station detects that it does not have a direct link established it should respond with an appropriate error message. | Clarify these paragraphs, and any others that are related, to clearly indicate what a stations normative behavior should be in the case of an error (i.e. it should or should not send an error response), particularly as it relates to situations where no direct link exists. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Behavior clarified. |
| 347 | Kevin | Hayes | 8.5.9.1.2 | 24 | 2 | T | Y | Firstly, should probably use the '¦¦' notation rather than a bulleted list. Secondly, why not do the MIC on the while EAPOL-Key frame, as is done for other protocols in this standard? | Please precisely describe the calculation of the MIC which shall include the entire frame. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MIC calculation specified. |
| 348 | Liwen | Chu | 8.5.9.1.2 | 23 | 35 | T | Y | On receipt of SMK Handshake Message 2, the initiator STA verifies the initiator MAC address against existing direct link. But actually SMK messages can be carried in TDLS setup messages. This should be fixed. | Use the method proposed in 11-08/290 to fix the problem. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MAC addresses bound to key derivation. |
| 349 | Meiyuan | Zhao | 8.5.9.1.2 | 24 | 5 | T | Y | The ordering of information for MIC calculation should be different for messages sent by STA_I and STA_P. The current design uses the same ordering, which enables reflection attack. | Change the ordering of MIC calculation in message 2 to: "MAC_P MAC_I BSSID RSNE_P PNonce INonce Lifetime DH_P DH_I". And change the ordering of MIC calculation in message 3 to: "MAC_I MAC_P BSSID RSNE_I INonce PNonce Lifetime DH_I DH_P". | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Ordering changed as per Link Identifier. |
| 350 | George | Vlantis | 8.5.9.1.3 | 24 | 25 | T | Y | In the case of TDLS, the checks of the MAC addresses in steps (a) and (b) do not make a lot of sense. | Please clarify the method with TDLS. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. MAC is calculated within the key derivation. |
| 351 | Henry | Ptasinski | 8.5.9.1.3 | 24 | 27 | T | Y | The BSSID should be verified. | After step (a), add a check that the BSSID matches the current association and the BSSID provided in Message 2. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. BSSID added to key derivation. |
| 352 | Jesse | Walker | 8.5.9.1.3 | 24 | 28 | T | Y | What happens if any of these checks fail? | Specify mandatory failure processing | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Failure cases specified. |
| 353 | Liwen | Chu | 8.5.9.1.3 | 24 | 25 | T | Y | On receipt of SMK Handshake Message 3, the peer STA verifies the initiator MAC address against existing direct link. But actually SMK messages can be carried in TDLS setup messages. This should be fixed. | Use the method proposed in 11-08/290 to fix the problem. | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 354 | Adrian | Stephens | 8.5.9.2 |  |  | T | Y | This subclause contains a number of "magic numbers" - i.e. values probably chosen as a best guess of this group on the value of timeouts. However, these values may turn out to be poorly chosen for whatever reason, and should be manageable in future. | Replace each magic number (e.g., 2000, 1000, 200) with a mib variable reference. Add mib variables for each of these magic numbers. |  |
| 355 | Clint | Chaplin | 8.5.9.2 | 25 | 2 | T | Y | "The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval 3 for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, 4 then 200 milliseconds shall be used for all timeout values." | "The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, then 200 milliseconds shall be used for all timeout values other than the first." or "The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, then 2000 milliseconds shall be used for all timeout values." |  |
| 356 | Henry | Ptasinski | 8.5.9.2 | 25 | 2 | T | Y | The listen interval is provided by a STA to the AP. Peers in TDLS do not have any knowledge of each other's listen interval. | Define the time in absolute terms, or relative to some value already known by both sides (e.g. DTIM). |  |
| 357 | Hideyuki | Suzuki | 8.5.9.2 | 24 | 48 | T | Y | To my understanding, all of the information which are necessary to build a STK are already shared through the SMK Handshake(SMK, PNonce, INonce, MAC_P and MAC_I). So the initiator STA might not explicitly need to initiate the 4-Way STK Handshake. | Need careful consideration on whether TDLS PeerKey Handshake and/or 4-Way handshake should be combined into 3-Way TDLS Setup Handshake or not, then modify the description of TDLS PeerKey Handshake appropriately. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. 4-way handshake removed. |
| 358 | Hideyuki | Suzuki | 8.5.9.2 | 24 | 50 | T | Y | "direct link teardown procedure" is not defined in this draft. | Define the "direct link teardown procedure" for TDLS. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. FTIE is included in teardown to indicate session identifier. |
| 359 | Hideyuki | Suzuki | 8.5.9.2 | 25 | 1 | T | Y | "direct link teardown procedure" is not defined in this draft. | Define the "direct link teardown procedure" for TDLS. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. FTIE is included in teardown to indicate session identifier. |
| 360 | Jouni | Malinen | 8.5.9.2 | 25 | 2 | T | Y | “The listen interval” is used as a timeout for SMK handshake. However, 802.11z does not specify which listen interval (or well, whose.. peer or initiator?) is used here. I would assume this text was just copied from the RSN 4-way handshake timeouts that the Authenticator uses. However, in case of TDLS PeerKey handshake, the initiator or peer STA do not really know the listen interval of the other STA and there is not much point in using STA's own listen interval as a timeout for retry in this case. Unless the STA can somehow figure out the other STA's listen interval, it would be better to just define a fixed timeout for TDLS PeerKey handshake. Furthermore, it does not look correct to replace the 2000 millisecond timeout with a 200 millisecond timeout if “there is no listen interval” (whatever that would mean in TDSL..) | Replace “The retransmit timeout value shall be 2000 milliseconds for the first timeout, the listen interval for the second timeout, and twice the listen interval for subsequent timeouts. If there is no listen interval, then 200 milliseconds shall be used for all timeout values.” with “The retransmit timeout value shall be 2000 milliseconds.” Similarly, on line 10, replace “The retranmit timeout value shall be 1000 millisecond sfor the first timeout, half the listen interval for the second timeout, and the listen interval for subsequent timeouts. If there is no listen interval, then 1000 milliseconds shall be used for all timeout values.” with “The retransmit timeout value shall be 1000 milliseconds.” |  |
| 361 | Meiyuan | Zhao | 8.5.9.2 | 25 | 9 | T | Y | The behavior of setting different values at different circumstances is confusing. It says "If the STA_I still has not received a response after these retries, it shall invoke a direct link teardown procedure. The retransmission timeout value shall be 1000 milliseconds...." Does this refer to the retransmission of STK handshake message or TDLS tear down message? If it's former, it's not consistent with base standard where the timeout value is 100 milliseconds. If it's latter, it's not consistent with the previous paragraph where the value is 2000 millisecond. Further the values of subsequent retires are different in the two paragraphs as well. | Clarify the setting of timeout values. |  |
| 362 | Meiyuan | Zhao | 8.5.9.2 | 25 | 6 | T | Y | STA_I sends message 3, not receives message 3. In this paragraph, the roles of STA_I and STA_P are contradictory from the specification. | Change the paragraph to: "Upon receipt of the SMK Message 3, the STA_P shall transmit Message 1 of the STK Handshake to the STA_I. If the STA_P does not receive Message 2 of the 4-Way STK…from the STA_I, it shall attempt …. If STA_P still has not received..." | Accept in principle. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 363 | Stephen | Emeott | 8.5.9.2 | 24 | 48 | T | Y | Its unclear why error handling for the rekey is described before the rekey proceduring in 8.5.9.3 is defined. | Relocate the material to the end of 8.5.9.3 | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 364 | Bill | Marshall | 8.5.9.3 | 25 | 35 | T | y | where is the value of "N" defined | please clarify | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. DH removed. |
| 365 | Clint | Chaplin | 8.5.9.3 | 25 | 25 | T | Y | "If the SMK has expired" | "If the SMK timer has expired" |  |
| 366 | Jesse | Walker | 8.5.9.3 | 25 | 17 | T | Y | What is "the 802.1X Timeout"? 802.1X, an external standard, defines numerous timeouts. How does the 802.11 MAC or SME access this value? Hint: from a specsmanship perspective, you only get to use interfaces defined in the 802.1X spec if you want to follow this tact, and they don't define one. | I think we will have to reference an 802.11i timer instead. These are maintained in the SME, which coordinates between the MAC and 802.1X. The 802.11i key management state machines in 8.5.5 and 8.5.6 are probably the right place to begin looking for the "right" timer. |  |
| 367 | Jesse | Walker | 8.5.9.3 | 25 | 21 | T | Y | 802.1X is a module external to 802.11. The base standard defines SME state machines in 8.5.5 and 8.5.6 to mediate between the MAC and 802.1X. IF 802.11z continues to use the 4-Way handshake, you will need to update at least the Supplicant (non-AP STA) state machine to express the new functionality defined herein. | In the comment | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. 4-way HS removed. |
| 368 | Jesse | Walker | 8.5.9.3 | 25 | 30 | T | Y | I am confused again. Line 25 says STA_I initiates the rekey, but line 30 seems to say that STA_P initiates. | Please make the text internally consistent | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 369 | Jouni | Malinen | 8.5.9.3 | 25 | 18 | T | Y | This paragraph seems to be in conflict with itself.. The first sentence states that rekeying is always initiated by the STA_i and the third sentence talks about STA_P initiating a TDLS Peer Key rekey procedure. STA_P is only requesting STA_I to initiate the rekeying procedure.. | Replace “before initiating” with “before requesting”. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. |
| 370 | Jouni | Malinen | 8.5.9.3 | 25 | 39 | T | Y | 802.11r and 802.11w introduce stronger MIC for EAPOL-Key frames to allow MD5 and SHA-1 -based mechanisms to be avoided. 802.11z should do the same for its use of EAPOL-Key. | Insert “or 3 (NIST AES key wrap with AES-128-CMAC)” into the end of Key Description Version description. This applies to all EAPOL-Key messages in the draft, not just the one on page 25 line 39. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Not using EAPOL-Key, and using negotiable MIC algorithm. |
| 371 | Jesse | Walker | 8.5.9.4 | 26 | 34 | T | Y | The use of the EAPOL-Key message implies an interaction with 802.1X via the SME. | Update the SME state machines in 8.5.5. and 8.5.6 to add any new DL specific details on how to accomplish this. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. EAPOL-Key removed. |
| 372 | Jesse | Walker | 8.5.9.4.1 | 27 | 27 | T | Y | Why is this needed? The devices can already discover this from the inclusion of RSNIEs in their setup messages. The error code appears to be nothing more than an opportunity to create new types of denial of service | Think about removing this error code. | Accept. Addressed with the acceptance of 11-08-0476-00-000z-TGz-Security-Proposal.doc. Error handling made consistent. |