IEEE P802.11
Wireless LANs

| Remove Characteristic Two Curves |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA 94089 United States of America | +1 408 227 4500 | dharkins@arubanetworks.com |

Abstract

It is required to remove support for characteristic two elliptic curves from the draft. Elliptic curves defined over a prime field continue to be supported and encouraged for use. Examples of such curves include those defined by the United States National Institute of Standards and Technology (NIST).

Editor—add the following paragraph to the end of section 7.3.1.1

7.3.1.1 Authentication algorithm number field

There are characteristic-two elliptic curves, F2m, (whose defining equation is written y2 + xy = x3 + ax2 + b) in the above registry but their use in this standard is undefined. Elliptic curves over a prime field, Fp, (whose defining equation is written y2 = x3 + ax + b) are used.

Editor—add the following paragraph to the beginning of 11B.2.3.1

11B.2.3.1 Elliptic Curve Finite Cyclic Groups

Only elliptic curves over a prime field, Fp, are permitted to be used with SAE in this standard. When generating the password element in section 11B.2.3.1.1, the defining equation y2 = x3 + ax + b is used to solve for a y-coordinate using a known x-coordinate.

References:

Federal Information Processing Standard (FIPS) 186-2