IEEE P802.11
Wireless LANs

| KDF mods and Test Vectors |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-May-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dan Harkins | Aruba Networks | 1322 Crossman ave, Sunnyvale, CA, 94089 United States of America | +1 408 227 4500 | dharkins@arubanetworks.com |

Abstract

Text to address comments 116 and 122 is presented. Test vectors are added for the KDF and the KDF is modified to take 128, 192 or 256 bit keys with which to derive more keys.

8.8.3 Key derivation function

The key derivation function for the mesh key hierarchy, KDF, is defined as follows:

Output = KDF-Length(KEY, X1, X2, …, Xn) where

Input:   KEY, a variable length key derivation key of at least 128 bits either 128, 192 or 256 bits

X1, X2, …, Xn a variable number of variable-length strings

Output: a Length-bit derived key

K = L(KEY, 0, 128)

result  “”

iterations  (Length+127)/128

do i = 1 to iterations

result  result || vPRF(K KEY, Length, i, X1, X2, … Xn, i)

od

return first Length bits of result, and securely delete all unused bits.

where vPRF(K, P1, P2, … Pm) is defined as

if m = 0 then return AES-128-CMAC-K, 0x01 01271)

S  AES-128-CMAC(K, 0x00 0128)

H.9 Mesh KDF Test Vectors

Derivation of a 768-bit key from a 128-bit, 192-bit and 256-bit key

first string

4d657368 204b6579 20446572 69766174

696f6e

meshID

11223344 55667788

mkd-nas-id

00112233 44556677 8899aabb ccddeeff

deaddada deaddada ffeeddcc bbaa9988

77665544 33221100

mkdid-id

b1abf0f0 0001

SPA

000b0e01 0203

---------------------------

input 128 bit key

fffefdfc fbfaf9f8 f7f6f5f4 f3f2f1f0

stretched 768-bit key:

dba01914 00f4af33 791af741 552257ed

e4cea244 b3086130 5cef49d3 a565901a

8bac6499 e1dc2386 dbf89e4b 2daa9021

67bf0603 a66917c6 37a71cf0 6c1a2d1e

0e945695 d4a36a6f 20dc9936 9102b430

c79dcdf5 3589694d 9a49c241 35fe46a9

---------------------------

input 192-bit key

fffefdfc fbfaf9f8 f7f6f5f4 f3f2f1f0

6f6e6d6c 6b6a6968

stretched 768-bit key:

4f6733e9 0b903aa4 d650fcfc 739ad7cd

bc9c94c3 acc98d48 0b6b76f0 4f343198

d2088d51 b0252a95 0af6d59e f67900c2

900988d3 1335e5e1 ccf9971a 646bd1c4

5f5b920d 3bf2805a 5ec6c6f5 8336c85a

baf3e2ad 72697685 f4b15086 3f38a279

---------------------------

input 256-bit key

fffefdfc fbfaf9f8 f7f6f5f4 f3f2f1f0

6f6e6d6c 6b6a6968 67666564 63626160

stretched 768-bit key:

c34e3e13 a0ac3ae8 be6fdeab fc8b5a47

99fb1f95 2150d5fa 8a9cf70d 145f1c86

3a9bd3f0 48e1c2bc 71974d37 875fde4e

e4481186 3ff5d8cf 34485f3b 932409f0

2a2d9fb0 7465465c 3afce721 68e34b58

eb643d48 ae8014e4 b7de8950 e8041a36

References: