IEEE P802.11
Wireless LANs

| Proposal for Extendend Capabilities for Interworking Service |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |

Abstract

Most of the amendments to 802.11 use two MIB variables to enable basic operation of new features.  For Interworking Service they would be referred to as dot11InterworkingServiceImplemented and dot11InterworkingServiceEnabled.  The former is a MIB variable, which typically is accessed via SNMP only, and provides capability information for the STA.  The latter is a state variable which determines whether the feature is currently being used in the STA.  For example, an information element related to Interworking Service should be included in a Beacon frame based on the state of dot11InterworkingServiceEnabled and not on dot11InterworkingServiceImplemented.

This proposal addresses CID #148, 176, 181and 188.

This proposal also adds two venue types plus a network type.

This document is based on 802.11u-d2.0 as modified by 11-08-0543r1.

  - Management frames

    - Fields that are not information elements

- 7.3.1.33a	Network metadata

Modify the rows in Table 7-43s as shown below:

Table 7-43s—Venue Type assignments

| Venue Group | Venue Type Code | Venue Description |
| --- | --- | --- |
| 1 | 11 | Bar |
| 1 | 12 | Coffee Shop |
| 1 | 13 | Theater |
| 1 | 14 | Zoo or Aquarium |
| 1 | 15 – 255 | Reserved |

    - Information elements

      - Extended capabilities element

Insert the following additional rows at the end of Table 7-35a.

Table 7-35a—Capabilities field

| Bit(s) | Information | Notes |
| --- | --- | --- |
| <ANA> | Interworking | When dot11InterworkingServiceEnabled is set to TRUE, the Interworking field is set to 1 to indicate the STA supports Interworking service as described in 11.18.1 through 11.18.5 inclusive. When dot11InterworkingServiceEnabled is set to FALSE, the Interworking field is set to 0 to indicate the STA does not support support this capability. |
| <ANA> | QoS Map | When dot11QosmapEnabled is set to TRUE, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.18.6. When dot11QosmapEnabled is set to FALSE, the QoS Map field is set to 0 to indicate the STA does not support support this capability. |
| <ANA> | EBR | When dot11EbrEnabled is set to TRUE, the EBR field is set to 1 to indicate the STA supports EBR as described in 7.3.2.62. When dot11EbrEnabled is set to FALSE, the EBR field is set to 0 to indicate the STA does not support support this capability. |

EDITORIAL NOTE—The values marked <ANA> will be replaced with a value assigned by the 802.11 ANA once that assignment has been made.

      - Interworking element

Modify Table 7-43s as shown below:

Table 7-43t—Network Type codes

| Network Type Code Bits (B0-B3) | Meaning | Description |
| --- | --- | --- |
| 0x5 to 0xD | Reserved | Not used. |
| 0xE | Test or Experimental | The network is used for test or experimental purposes only. |

- 11.	MLME

- 11.18	WLAN Interworking with External Networks Procedures

- 11.18.1	Interworking capabilities and Information

- 11.18.2	Interworking Procedures: Generic Advertisement Services

- 11.18.3	multiple SSID Set Operation

- 11.18.4	Interworking Procedures: MIH Support

- 11.18.5	Interworking Procedures: Interactions with SSPN

Move the entire following clause to 11.19, rename clause title to “Support for QoS Mapping from External Networks”and renumber as appropriate.

    - Interworking Procedures: Support for QoS Mapping from External Networks

Maintaining proper end-to-end QoS is an important factor when providing interworking services. This is because the interworking networks may employ different network-layer (layer 3) QoS practices. For example, the use of a particular differentiated services code point (DSCP) for a given service may be different between different networks. To ensure the proper QoS over-the-air in the IEEE 802.11 Access Network (AN), the mapping from DSCP to UP for the corresponding network needs to be identified and made known to the non-AP STA. If an inconsistent mapping is used then:

- Admission control at the AP may incorrectly reject a service request, because the non-AP STA used the incorrect UP.

- The user may be given a different QoS over the IEEE 802.11 AN than expected, e.g. a lower QoS may be provided by the IEEE 802.11 AN than the STA expected.

Therefore, a non-AP STA with dot11QosmapEnabled set to TRUE which is associated to an AP having the QoS Map bit set to 1 in the Extended capabilities element, shall request the QoS Map from an AP prior to transmitting frames at other than best-effort user priority (UP = 0). The non-AP STA requests the QoS Map by transmitting a QoS Map Request Action frame to the AP to which it’s associated. In response, the AP transmits the QoS Map Set element in a QoS Map Configure Action frame. Upon receiving the QoS Map Set element, the non-AP STA is then capable of deciding the proper user priority and in turn the AC to use over-the-air for a particular service. The AP shall ensure the correctness of the QoS mapping information at the non-AP STA associated to itself. Therefore, when the AP detects a change in the QoS mapping information, it shall update the non-AP STA using a new QoS Map Set element.

- 11.18.6.1	QoS Map Distribution Operation

The QoS Map Distribution operation described below provides a method for synchronizing the QoS mapping at the non-AP STA and AP. A STA may have dot11QosmapEnabled set to TRUE only if dot11QosOptionImplemented is set to TRUE.

After the non-AP STA having its dot11QosmapEnabled MIB varible set to TRUE finishes the RSNA security procedures, its SME shall generate a MLME-QoSMap.request primitive to the MLME. The MLME of the non-AP STA shall send a QoS Map Request Action frame to the AP.

When the MAC entity at the AP having its dot11QosmapEnabled MIB variable set to TRUE receives a QoS Map Request Action frame from a non-AP STA, the MLME shall generate a MLME-QoSMap.indication primitive to its SME. An AP which has its dot11QosmapEnabled MIB variable set to FALSE shall drop the QoS Map Request Action frame and shall not respond to that request.

When SME of the AP receives a MLME-QoSMap.indication, it shall generate an MLME-QoSMap.response primitive. The SME of the AP can also decide to generate an unsolicited MLME-QoSMap.response primitive to the MLME based on its configuration status as described in 11.18.6.2.

When the MLME of the AP receives a MLME-QoSMap.response, it shall send a QoS Map Configure Action frame to the corresponding non-AP STA MAC entity.

When the MAC entity at the non-AP STA receives a QoS Map Configure action frame from the AP, the MLME shall generate a MLME-QoSMap.confirm primitive to its SME. When the QoS Map Configure action frame is unsolicited, the non-AP STA SME can choose to generate another MLME-QoSMap.request primitive.

When the non-AP STA's SME receives the QoS Map response, it shall make the QoS Map available to higher layers so that in turn, they can invoke the MA-UNITDATA.request with the correct priority.

- 11.18.6.2	QoS Map Distribution Operation

This clause applies to an AP having its dot11QosmapEnabled MIB variable set to TRUE.

Typically, the QoS Map Set is configured on an AP for each external network to which the BSS is interworked. Most of the time, the QoS Map Set remains static. In this case, the QoS Map Set shall be transmitted to the requesting non-AP STA using an individually addressed QoS Map Configure action frame.

However, it is possible that the QoS Map Set may need to be changed during operation of the BSS. If the QoS map set is changed, the AP shall unicast the new QoS map set using an unsolicited QoS Map Configure Action frame to each non-AP STA which has associated to the AP with its QoS Map bit set to 1 in the Extended Capabilities element included in the (Re)-Association request frame.

Move the entire following clause after 11.18.5 and renumber as appropriate.

- 11.18.7	Interworking Procedures: Emergency Alert System (EAS) Support

Insert the following text as shown below:

- Annex D

Other amendments are also making changes in Dot11StationConfigEntry. It will be aligned.

Dot11StationConfigEntry::=
	SEQUENCE {	dot11StationID 			MacAddress,
		dot11MediumOccupancyLimit 			INTEGER,
		dot11CFPollable 			TruthValue,
		dot11CFPPeriod 			INTEGER,
		dot11CFPMaxDuration 			INTEGER,
		dot11AuthenticationResponseTimeOut 			Unsigned32,
		dot11PrivacyOptionImplemented 			TruthValue,
		dot11PowerManagementMode 			INTEGER,
		dot11DesiredSSID 			OCTET STRING,
		dot11DesiredBSSType 			INTEGER,
		dot11OperationalRateSet 			OCTET STRING,
		dot11BeaconPeriod 			INTEGER,
		dot11DTIMPeriod 			INTEGER,
		dot11AssociationResponseTimeOut 			Unsigned32,
		dot11DisassociateReason 			INTEGER,
		dot11DisassociateStation 			MacAddress,
		dot11DeauthenticateReason 			INTEGER,
		dot11DeauthenticateStation 			MacAddress,
		dot11AuthenticateFailStatus 			INTEGER,
		dot11AuthenticateFailStation 			MacAddress,
		dot11MultiDomainCapabilityImplemented 			TruthValue,
		dot11MultiDomainCapabilityEnabled 			TruthValue,
		dot11CountryString 			OCTET STRING,
		dot11SpectrumManagementImplemented 			TruthValue,
		dot11SpectrumManagementRequired 			TruthValue,
		dot11RSNAOptionImplemented 			TruthValue,
		dot11RSNAPreauthenticationImplemented 			TruthValue,
		dot11RegulatoryClassesImplemented 			TruthValue,
		dot11RegulatoryClassesRequired 			TruthValue,
		dot11QosOptionImplemented 			TruthValue,
		dot11ImmediateBlockAckOptionImplemented 			TruthValue,
		dot11DelayedBlockAckOptionImplemented 			TruthValue,
		dot11DirectOptionImplemented 			TruthValue,
		dot11APSDOptionImplemented 			TruthValue,
		dot11QAckOptionImplemented 			TruthValue,
		dot11QBSSLoadOptionImplemented 			TruthValue,
		dot11QueueRequestOptionImplemented 			TruthValue,
		dot11TXOPRequestOptionImplemented 			TruthValue,
		dot11MoreDataAckOptionImplemented 			TruthValue,
		dot11AssociateinNQBSS 			TruthValue,
		dot11DLSAllowedInQBSS 			TruthValue,
		dot11DLSAllowed 			TruthValue,
		dot11InterworkingServiceImplemented 			TruthValue,
		dot11InterworkingServiceEnabled 			TruthValue,
		dot11QosmapImplemented 			TruthValue,
		dot11QosmapEnabled 			TruthValue,
		dot11EbrImplemented 			TruthValue,
		dot11EbrEnabled 			TruthValue,
		dot11GasTimPeriod 			INTEGER,
		dot11GasTimTimeToSuspend 			INTEGER,
		dot11ESNetwork 			TruthValue
		}

dot11InterworkingServiceImplemented OBJECT-TYPE
		SYNTAX TruthValue
		MAX-ACCESS read-write
		STATUS current
		DESCRIPTION

“This attribute when TRUE, indicates the STA is capable of

interworking with external networks. A STA setting this to TRUE

implements Interworking Service. When this is set to FALSE,

the STA does not implement Interworking Service. The default

value of this attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA>}

dot11InterworkingServiceEnabled OBJECT-TYPE
		SYNTAX TruthValue
		MAX-ACCESS read-write
		STATUS current
		DESCRIPTION

“This attribute, when TRUE, indicates the capability of the

station to interwork with external networks is enabled.  The

capability is disabled, otherwise.  The default value of this

attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA+1>}

dot11QosmapImplemented OBJECT-TYPE

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute available at STAs, when TRUE, indicates the STA

is capable of supporting the QoS Map procedures. When this is

set to FALSE, the STA does not implement QoS Map procedures.

The default value of this attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA+2>}

dot11QosmapEnabled OBJECT-TYPE

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates the capability of the STA

to support QoS Map procedures is enabled. The capability is.

disabled otherwise. The default value of this attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA+3>}

dot11EbrImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute available at STAs, when TRUE, indicates the STA

is capable of supporting Expedited Bandwidth Request

procedures. When this is set to FALSE, the STA does not

implement Expedited Bandwidth Request procedures. The default

value of this attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA+4>}

dot11EbrEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute, when TRUE, indicates the capability of the STA

to support Expedited Bandwidth Request procedures is enabled.

the capability is disabled otherwise. The default value of this

attribute is FALSE.”

DEFVAL (FALSE)

::= {dot11StationConfigEntry <ANA+5>}

dot11GasTimPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object identifies the number of Beacon intervals between 
			successive GASTIMs. If all beacons are GASTIMs, the GASTIM Period 
			field has the value 1. The GASTIM Period value 0 is reserved.”

::= {dot11StationConfigEntry <ANA+6>}

dot11GasTimTimeToSuspend OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object identifies the number of TUs after TBTT (GASTIM) after 
			which the AP will not schedule for transmission any further multicast 
			Advertisement frames.“

::= { dot11StationConfigEntry <ANA+7> }

dot11ESNetwork OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“The Emergency Services Network Type set to TRUE for this

HESSID set Indicates that higher layer emergency call services

are reachable via this SSID.“

::= { dot11StationConfigEntry <ANA+8> }